# 1. The networking foundation

## 1.1 The Open Systems Interconnection seven-layer model

The Open Systems Interconnection (OSI) model is best treated as a reference framework for reasoning about communication functions, not as a claim that every real product cleanly implements seven visible layers in hardware or software.

A universally used method for understanding network functionality, with the physical layer closest to the media technology and the application layer closest to the user.

The value of the model is that it brings about separation of functional concerns that otherwise get mixed together.

At Layer 1, the physical layer, the concern is signalling over the medium: electrical, mechanical, and procedural rules for activating and maintaining the physical link.

At Layer 2, the data link layer, the concern is frame transport across that physical link, including physical addressing, media access, topology handling, frame ordering, and flow control. The data link layer is split into the Media Access Control (MAC) and Logical Link Control (LLC) sublayers.

At Layer 3, the network layer, the concern becomes connectivity and path selection between end systems; this is the layer where routing occurs.

At Layer 4, the transport layer, the concern becomes reliable end-to-end communication, fault detection and recovery, and flow control between communicating endpoints. The layer responsible for reliable network communication between end nodes.

The upper layers are more about the meaning and structure of the conversation than about forwarding bits and packets .

Layer 5, the presentation layer ensures that information sent by the application layer of one system is readable by the application layer of another system, including transfer syntax and data structure issues.

Layer 6, the session layer manages the data exchange between presentation-layer entities.

Layer 7, the application layer is the highest layer and is closest to the user or application function.

So, in short: the lower layers move signals, frames, and packets; the middle layers move end-to-end conversations; the upper layers make those conversations useful to applications.

## 1.2 Why the OSI model matters specifically for understanding Ethernet

Ethernet discussions become much easier once you stop treating “Ethernet” as one undivided thing. IEEE 802.3 Ethernet sits mainly in the physical layer and the MAC sublayer of the data link layer. IEEE 802.1 concerns itself with architecture, bridging, network management, security, and protocol layers above MAC and LLC. The IEEE 802.1 Working Group mission states exactly that: it covers 802 LAN/MAN architecture, internetworking among 802 networks, 802 security, overall network management, and protocol layers above the MAC and LLC layers.

The OSI model helps to explain why a PHY issue, a switch-queueing issue, an IP routing issue, and a SOME/IP service issue are different classes of problems.

# 2. The IEEE 802 family structure

## 2.1 What IEEE 802 actually is

The IEEE 802 LAN/MAN Standards Committee (LMSC) is the standards body that develops networking standards for local, metropolitan, and related area networks. The official IEEE 802 page says it develops and maintains standards and recommended practices for local, metropolitan, and other area networks, and that individual working groups provide the focus for each area.

“IEEE 802” is a family of working groups and standards. The active working-group list shows examples such as 802.1 Higher Layer LAN Protocols, 802.3 Ethernet, 802.11 Wireless LAN, and 802.15 Wireless Personal Area Network.

## 2.2 Where 802.1 and 802.3 sit in that family

For an automotive Ethernet scenario, the most important split is between 802.3 and 802.1.

IEEE 802.3 is the Ethernet working group. IEEE 802.3 can be seen as the IEEE Local Area Network protocol that specifies an implementation of the physical layer and the MAC sublayer of the data link layer. That is the “wire and MAC behavior” side of Ethernet. In short deals with the Media access side.

IEEE 802.1 is where bridging, VLANs, Time-Sensitive Networking, security, and higher-layer LAN architecture live. The official 802.1 mission statement says it covers architecture, internetworking, security, management, and protocol layers above MAC and LLC. That is the “system behavior of bridged Ethernet networks” side. In short deals with the network infrastructure side.

Historically, IEEE 802.2 defined the Logical Link Control (LLC) sublayer.

802.2 specifies an implementation of the LLC sublayer and provides the Layer 3 service interface used with 802.3 and 802.5. The old IEEE working-group list also shows 802.2 as a disbanded working group, which is a useful reminder that not every historical 802 number remains an active modern working group.

So the clean model is:

- 802.3: Ethernet physical layer and MAC behavior

- 802.1: bridge, architecture, VLAN, TSN, security, and management behavior above that

- 802.11 / 802.15 / others: different media families or adjacent networking domains.

## 2.3 Why this family structure matters in automotive

Automotive engineers often need both sides at once. If we are looking at 100BASE-T1, 1000BASE-T1, or 10BASE-T1S, we are in 802.3 territory.

If we are looking at bridging, Virtual Local Area Networks (VLANs), Time-Sensitive Networking (TSN), MACsec, or generalized Precision Time Protocol (gPTP), we are in 802.1 territory.

# 3. Domain versus zonal vehicle architecture

## 3.1 Why vehicle architecture changed at all

Older vehicle networks were built around many function-specific Electronic Control Units (ECUs) tied together by one or more gateways. The IEEE automotive use-case document calls this the traditional or present-day model and says many ECUs are connected using non-Ethernet technologies such as Controller Area Network (CAN) and Local Interconnect Network (LIN), with Ethernet limited to the higher-bandwidth links.

As bandwidth, software reuse, and update requirements increased, manufacturers moved first toward domain architecture and then toward zonal architecture. The IEEE automotive profile work treats these as distinct architectural phases, not as mere marketing labels.

## 3.2 Domain architecture from fundamentals

In a domain architecture, functions are grouped by functional responsibility.

So, for example, you may have a domain controller for Advanced Driver Assistance Systems (ADAS), another for infotainment, another for body functions, and another for chassis or powertrain.

The IEEE automotive use-case document says domain networks are the current focus of many original equipment manufacturers, that Ethernet is a clear enabler because of its speed and OSI-layer support, and that many OEMs want ECU applications to communicate using IP so the physical connection details are abstracted away from the application.

That last point is very important. In a domain architecture, Ethernet is not just “more bandwidth.” It is also an abstraction mechanism.

If applications speak over IP and a common Ethernet backbone, then an ECU application can be reused across car programs even if the exact underlying topology or link speed changes.

The IEEE document explicitly says this reuse across different car models is one of the reasons OEMs want IP-based communication.

Domain architectures are also modular. The same IEEE document says they can scale from low-end to high-end variants of a car model and gives examples such as removing an ADAS ECU or scaling infotainment performance.

So domain architecture is still a function-first architecture, but it is much more Ethernet- and software-friendly than the older central-gateway style.

## 3.3 Zonal architecture from fundamentals

In a zonal architecture, the grouping principle changes. Instead of grouping mainly by function, the vehicle is divided into physical topological zones, and functions that used to be physically isolated in separate functional domains now share the same physical backbone and zone-oriented wiring.

The IEEE automotive use-case text says zonal networks are seen by many as the flexible networking solution, that the car is separated into topological zones, and that functions previously isolated in the domain model now share the same physical wire.

It also says Ethernet’s scalable bandwidth and TSN capabilities become requirements here, on top of the OSI layering and IP requirements already used in domain networks.

That sentence captures the essence of zonal architecture. Zonal is not just “domain plus more Ethernet.” It is a topology-first architecture. Zone controllers sit near the physical parts of the car they serve, and a high-speed Ethernet backbone interconnects those zones. The result is less function-specific dedicated cabling and more shared transport infrastructure.

The IEEE document lists the main drivers very clearly:

Reduction in wiring-harness size, weight, cost, and complexity; the ability for any data to go anywhere without unnecessary replication;

Better support for over-the-air (OTA) updates;

Reuse of the same architecture and end nodes across low-, mid-, and high-end variants; and easier use of TSN-based redundancy.

## 3.4 What zonal architecture buys you, and what it costs

Zonal architecture buys packaging efficiency and network flexibility, but it also increases the importance of good Ethernet engineering.

The same IEEE source says zonal networks require the implementer to be familiar with IEEE 802 networking, IEEE 802.1Q, and TSN standards, and that they must solve functional-safety and security concerns.

In other words, zonal architecture gives you a more elegant physical and software architecture, but it shifts more responsibility onto the shared network fabric.

That is why Ethernet becomes so central in zonal vehicles. In the older model, you could often protect a function by giving it its own wire or its own bus segment.

In a zonal model, multiple functions may share the same physical Ethernet infrastructure, so timing, prioritization, isolation, redundancy, and security all become network-design problems instead of wiring-design problems.

## 3.5 A clean comparison

A concise way to understand the transition is this:

- Traditional / central gateway: many function-specific ECUs, lots of non-Ethernet links, Ethernet only where bandwidth forces it.

- Domain: functions grouped by domain, domain controllers connected by Ethernet, strong IP/software modularity.

- Zonal: physical zones become the primary organization, many functions share one Ethernet backbone, TSN and Ethernet switch behavior become architectural requirements.

The IEEE automotive use-case document even quantifies the trend: it shows increasing Ethernet link counts, more congestion points, tighter end-to-end latency needs, and tighter synchronization requirements as you move from traditional to domain to zonal architectures.

# 4. Where the OPEN Alliance SIG fits

## 4.1 What the OPEN Alliance is doing

The OPEN Alliance SIG is one of the main places where the automotive industry turns generic IEEE Ethernet technology into automotive-usable profiles, interfaces, tests, and implementation guidance.

The OPEN Alliance specifications page describes its library as covering multiple automotive Ethernet speed grades, and the committee/news pages show that its work spans channels, components, test specifications, sleep/wake behavior, switch requirements, MACsec, Media Access Control Security (MACsec) Key Agreement (MKA), and software management.

So the OPEN Alliance role is not “replace IEEE 802.” IEEE defines the base Ethernet and TSN standards. The OPEN Alliance then fills the automotive gap with profiles, electrical interfaces, interoperability tests, implementation specs, and ecosystem agreements needed to make those standards work consistently in vehicles.

In an Automotive Ethernet context, OPEN Alliance is a profiling and industrialization layer on top of IEEE Ethernet, not a competing standards body.

## 4.2 Why OPEN Alliance matters in a zonal world

As soon as vehicles move toward domain and zonal Ethernet backbones, the “last mile” details become decisive: wake and sleep behavior, channel requirements, PHY interoperability, topology discovery, EMC behavior, switch requirements, and management abstractions. Those are exactly the kinds of topics the OPEN Alliance specification library covers. The public specifications page lists, among others, sleep/wake-up specifications, 10BASE-T1S system implementation, channel and component requirements, advanced diagnostic PHY features, switch requirements, switch test specifications, and automotive MACsec / MKA specifications.

In other words, IEEE gives you the broad networking language; OPEN Alliance helps make it vehicle-ready.

# 5. TC10 overview

## 5.1 The role of TC10

OPEN Alliance Technical Committee 10 (TC10) focuses on Ethernet sleep and wake-up behavior for automotive networks. The OPEN Alliance organization overview says TC10 specifies the functionality and needs of Ethernet sleep and wake-up for different speed grades.

That sounds narrow, but it is actually very important in vehicles. Unlike an office switch, an automotive Ethernet network must integrate with ignition states, low-power modes, partial networking, wake-up propagation, and fast resume behavior. So TC10 is really about making Ethernet behave like a vehicle network, not just like an always-on office LAN.

## 5.2 What TC10 specifications are trying to achieve

The public Sleep/Wake-up Specification for Automotive Ethernet states objectives such as compatibility with the xMII family of media-independent interfaces, support for global network wake-up, and wake-up/link-startup times under 250 ms. It also says the partial-networking concept based on selective link shutdown and fast global wake-up is especially suited for automotive Ethernet networks.

That tells you the real purpose of TC10 work:

- let links sleep when the vehicle wants low-power behavior

- still wake the network quickly and predictably

- support partial networking rather than only all-on or all-off behavior

- make this work across multiple automotive Ethernet speed grades and PHY types.

## 5.3 TC10 is still active and evolving

A 2025 public OPEN Alliance Sleep/Wake-Up Electrical Interface Specification identifies the TC10 chair and vice chair and gives guidance for PHYs that support dedicated wakeup and sleep signaling pins, explicitly stating that sleep/wakeup commands are not part of the xMII interface. That is a good reminder that TC10 is not just a one-time legacy document; it is an active area where the electrical and signaling details continue to be refined.

# 6. TC14 overview

## 6.1 The role of TC14

OPEN Alliance Technical Committee 14 (TC14) is focused on 10BASE-T1S in automotive applications. The official committee-goal snippet says the goal of TC14 is to enable 10BASE-T1S in an automotive context, including development of conformance, interoperability, and electromagnetic compatibility (EMC) test specifications.

That is an important role because 10BASE-T1S is not just another faster point-to-point PHY. It is a multidrop, short-reach, low-speed Ethernet option aimed at edge connectivity where low cost and simpler wiring are attractive. In automotive systems, that immediately raises questions about interoperability under stress, EMC, wake/sleep integration, and system-level implementation rules. TC14 is the group that turns the raw IEEE 802.3 technology into an automotive-ready ecosystem around those questions.

## 6.2 What TC14 work looks like in practice

The public OPEN Alliance documents around TC14 show exactly what that means in practice. There are public 10BASE-T1S system implementation, Physical Medium Dependent (PMD) transceiver interface, sleep/wake-up, and Physical Medium Attachment (PMA) compliance test documents.

The compliance test specification references the TC14 system implementation spec and verifies physical-layer characteristics such as transmitter behavior, receiver behavior, impedance, and timing.

So TC14 is not only about writing abstract requirements. It is about giving the industry a repeatable way to answer, “Does this 10BASE-T1S device actually behave correctly and interoperably in an automotive system?” That is exactly the sort of committee work a zonal architecture needs at the edge of the network.

## 6.3 TC14 and TC10 interact

The public 10BASE-T1S Sleep/Wake-up Specification is a TC14/TC10 Joint Working Group document, and it explicitly says a 10BASE-T1S solution shall include TC10 standard wake-forwarding mechanisms and segment wake-up behavior. That shows the relationship clearly: TC14 handles automotive 10BASE-T1S system enablement, but it cannot do that in isolation from TC10 sleep/wake behavior.

Real automotive Ethernet standardization is not a stack of isolated documents. It is a coordinated ecosystem where PHY behavior, system implementation, wake/sleep, EMC, and interoperability all have to line up.

# 7. The most useful way to tie it all together

The best way to explain all four requested topics together is this:

The OSI model gives you the conceptual language to separate signaling, framing, routing, transport, and application functions.

The IEEE 802 family tells you where Ethernet and bridged-network behavior actually live: 802.3 for the physical layer plus MAC, 802.1 for architecture, bridging, management, security, VLANs, and TSN.

The move from traditional to domain to zonal vehicle architecture is the story of why Ethernet stopped being just a few high-bandwidth links and became the backbone of the in-vehicle network. Domain architecture is function-centered and software/IP friendly; zonal architecture is topology-centered and depends heavily on Ethernet bandwidth, TSN, and shared switched infrastructure.

The OPEN Alliance SIG then takes those IEEE technologies and makes them automotive-usable through implementation specifications, wake/sleep definitions, channel/component requirements, switch requirements, and interoperability / compliance tests. In that ecosystem, TC10 handles Ethernet sleep/wake behavior across speed grades, and TC14 focuses on making 10BASE-T1S work reliably in automotive systems through system specs and conformance/interoperability work.