# Ethernet frame formats, EtherType, and frame filtering

## 1. Starting with the actual Ethernet frame on the wire

At the most basic level, an Ethernet frame is a Layer 2 container that carries payload from one local node to another across an Ethernet link or switched Ethernet network.

The classic frame components are Destination Address, Source Address, a Length/Type field, MAC client data, padding if needed, and the Frame Check Sequence (FCS). The normal MAC client data length is 46 to 1500 bytes, which is why standard Ethernet has both minimum and maximum frame-size rules.

Immediately after the preamble and start-frame delimiter, both Ethernet and IEEE 802.3 frames carry a 6-byte destination address and a 6-byte source address, and the destination may be unicast, multicast, or broadcast.

So the most useful first mental model is:

An Ethernet frame is a very specific Layer 2 envelope with addressing, protocol-identification or length information, payload, and an end-of-frame integrity check.

## 2. The “Type I / Type II” wording is historically messy

A Microchip Ethernet device datasheet uses the phrase “Ethernet type I” for frames where the two-byte field after the source address is being interpreted as an actual length value, and “Ethernet type II” for the format where that same field contains an EtherType value. The same datasheet says that what it calls “type I” framing may then appear as LLC or LLC/SNAP forms, while “type II Ethernet” is the common basic EtherType-based format. Cisco’s troubleshooting guide matches the practical distinction even though it does not use the “type I” label: in Ethernet frames, the field after the source address is a type field; in IEEE 802.3 frames, that field is a length field.

The clean and modern way to understand this is:

- Ethernet II = the two-byte field after source MAC is an EtherType

- IEEE 802.3 + LLC / SNAP = the two-byte field after source MAC is a length

- some vendor documents loosely call that length-style case “Type I”, but that is not the most useful mainstream term today.

## 3. Ethernet II: the frame format you will see most often

In modern IP networking, Ethernet II is the dominant frame format. Basically all modern applications use Ethernet II, and in Ethernet frames the field following the source address is a type field that specifies which upper-layer protocol should receive the payload after Ethernet processing is complete.

That means Ethernet II works like this:

- destination MAC address

- source MAC address

- EtherType

- payload of the next protocol

- Frame Check Sequence.

The beauty of Ethernet II is simplicity. The receiver does not need to inspect an extra LLC header to understand what comes next. The EtherType directly tells it whether the payload is, for example, IPv4, ARP, IPv6, LLDP, MACsec, or Precision Time Protocol.

## 4. IEEE 802.3 with LLC and SNAP

The other major historical family is IEEE 802.3 framing with Logical Link Control (LLC), optionally with Subnetwork Access Protocol (SNAP).

In IEEE 802.3 frames the two-byte field after the source address is a length field, indicating the number of bytes of data before the Frame Check Sequence (FCS). It also says that in the 802.3 case, the upper-layer protocol must be defined within the data portion of the frame, if at all, instead of by a Layer 2 EtherType field. Length-style Ethernet may appear as plain LLC or LLC plus SNAP, and SNAP follows a 3-byte LLC header.

So the practical difference is:

- Ethernet II identifies the next protocol directly at Layer 2 with EtherType

- 802.3/LLC/SNAP identifies the next protocol inside the payload area after the length field.

This is why Ethernet II became dominant for modern networking stacks. It is simpler for carrying today’s common protocol families.

## 5. VLAN-tagged Ethernet frames

Virtual Local Area Network (VLAN) tagging.

A VLAN-tagged frame inserts an extra 4-byte tag between the source MAC address and the original payload protocol identification. The key point is not only “there is an extra tag,” but also that the link now carries VLAN context and priority context in the frame itself.

The public IEEE ieee802-ethertype.yang module identifies 0x8100 as the Customer VLAN Tag (C-TAG) and the IANA IEEE-802 registry identifies 0x88A8 as the Service VLAN Tag (S-Tag). The IEEE 802.1Q YANG definitions also describe the Priority Code Point (PCP) as a 3-bit field associated with an 802.1Q VLAN-tagged frame, carrying a priority value from 0 to 7.

So a single-tagged VLAN frame is, conceptually:

- destination MAC

- source MAC

- TPID for VLAN tagging, often 0x8100

- tag control information including PCP and VLAN identifier

- original payload protocol indication and payload

- FCS.

The major reason this matters is that the frame no longer says only “who should receive this.” It also says “which VLAN does this traffic belong to?” and “what relative priority should the bridge apply?”

## 6. Single-tagged versus double-tagged frames

A frame can also be double-tagged, often called Q-in-Q in general networking language.

The OPEN Alliance automotive Layer 2 test specification explicitly tests both single-tagged and double-tagged frames and checks that outer and inner Tag Protocol Identifier (TPID) and VLAN Identifiers (VIDs) are correct. It specifically includes cases where the inner TPID is 0x8100 and the outer TPID is 0x88A8.

The operational meaning of double-tagging:

- one tag can express one VLAN domain or service boundary

- a second tag can preserve or transport another VLAN context across the network.

In enterprise/provider networking, this often separates customer and provider VLAN semantics. In automotive test practice, it shows that silicon and ECUs may need to understand not only untagged and single-tagged frames, but also double-tagged traffic where required by the system design.

## 7. Jumbo frames

A jumbo frame is simply an Ethernet frame whose payload size is larger than the standard 1500-byte Maximum Transmission Unit (MTU) used by ordinary Ethernet payloads.

Jumbo frames are Ethernet frames with more than 1500 bytes of payload and they can carry up to 9000 bytes of payload in practice (implementation dependent).

Jumbo frames are common implementation practice, not the default classical IEEE 1500-byte payload case.

That matters because many assume “Ethernet always means 1500-byte MTU.” In practice, it often does not.

## 8. Broadcast, multicast, and unicast at Layer 2

At Layer 2, the destination MAC address tells the switch which delivery pattern is intended.

Layer 2 behaviour:

- unicast sends frames from one node to a single other node

- multicast sends traffic from one node to multiple nodes

- broadcast sends frames to all nodes in the Layer 2 broadcast domain. It also states that the Layer 2 broadcast MAC address is FF:FF:FF:FF:FF:FF. Cisco’s troubleshooting guide adds that the source address is always unicast, while the destination may be unicast, multicast, or broadcast.

This is fundamental, because switches do not treat all destination addresses the same way.

- A known unicast can be forwarded to one specific egress port.

- A broadcast must be replicated across the relevant VLAN or broadcast domain.

- A multicast may be selectively forwarded if multicast membership is known, or more widely flooded if it is not.

## 9. EtherType: what it really does

The EtherType field tells the receiver what the next protocol is, when the frame is in Ethernet II or equivalent EtherType-identified form.

The IANA IEEE-802 registry and the public IEEE EtherType YANG module give authoritative examples, including:

- 0x0800   = IPv4

- 0x0806   = Address Resolution Protocol (ARP)

- 0x8035   = Reverse Address Resolution Protocol (RARP)

- 0x8100   = Customer VLAN tag

- 0x86DD = IPv6

- 0x88A8  = Service VLAN tag

- 0x88CC = Link Layer Discovery Protocol (LLDP)

- 0x88E5  = Media Access Control Security (MACsec)

- 0x88F7  = Precision Time Protocol (PTP).

This is why EtherType matters so much in modern Ethernet training. It is the Layer 2 protocol selector for the payload that follows.

## 10. Frame filtering fundamentals: what a bridge actually does

Frame filtering begins with a simple question:

Should this frame be forwarded at all, and if so, to which ports?

A bridge connects network segments at Layer 2 and filters and forwards frames based on the destination MAC address. When VLAN filtering is enabled, the bridge forwards frames based on both destination MAC address and VLAN tag, and the valid VLAN protocols for filtering include 0x8100 and 0x88A8.

Thus switch forwarding is not only a MAC lookup. In a VLAN-aware bridge, the lookup is really:

destination MAC + VLAN context → forwarding decision.

## 11. Learning, flooding, and multicast filtering

A switch also has to decide what to do when it does not know the destination yet.

A port can learn source MAC addresses from received traffic and that dynamic forwarding-database entries are refreshed by incoming traffic. Unicast traffic for which there is no forwarding-database entry can be flooded, and multicast traffic for which there is no multicast database entry can also be flooded. Unknown unicast flooding can create storms and is therefore something administrators may want to constrain.

So the switch’s basic behavior is:

- learn from source addresses

- forward using destination addresses

- flood unknown unicast if necessary

- flood or selectively forward multicast depending on state

- replicate broadcast within the VLAN or broadcast domain.

That is the real foundation of frame filtering.

## 12. Why VLAN filtering matters so much

Without bridging and VLANs, all devices on the Ethernet LAN are in a single broadcast domain, while bridging creates separate broadcast domains and VLANs group related devices into separate logical networks. Once VLAN filtering is enabled, forwarding depends on both destination MAC and VLAN tag.

VLAN filtering is not “just tagging.” It is how the switch decides that a frame should remain inside one logical Layer 2 domain and not leak into another. In practice, VLAN filtering is one of the most important mechanisms for keeping local traffic local, containing broadcast and multicast scope, and separating traffic classes that should not share the same forwarding universe.

# Automotive perspective and context

## 13. The Ethernet frame is the same, but the design intent changes

In a vehicle, the underlying Ethernet frame formats are still the same Ethernet II, 802.3/LLC, VLAN-tagged, and jumbo-capable structures you see in ordinary data communications. What changes is why they are used and how tightly their forwarding behavior is engineered.

The IEEE automotive Ethernet tutorial shows that automotive switched Ethernet is expected to support deterministic low latency, reduced latency variation, per-stream filtering and policing, scheduled traffic, frame preemption, and security support. It explicitly states end-to-end latency targets around 100 microseconds with the idea of “the right packet at the right time.”

So in automotive systems, frame format is not just an interoperability topic. It is part of a larger timing, filtering, and safety architecture.

## 14. Why Ethernet II dominates in automotive Ethernet backbones

Automotive Ethernet increasingly carries protocols such as IP, Address Resolution Protocol, Precision Time Protocol, Link Layer Discovery Protocol, MACsec, and VLAN-tagged Time-Sensitive Networking traffic. Those protocols all fit naturally into the EtherType-based Ethernet II style of framing. The EtherType registries explicitly show values for IPv4, IPv6, ARP, LLDP, MACsec, PTP, and VLAN tagging. Cisco’s training material also says modern applications basically use Ethernet II.

So, in practical automotive backbone networks, the “frame-format story” is usually not about revisiting every historical 802.3/LLC variant equally. It is mainly about Ethernet II plus VLAN tags, because that is where modern in-vehicle IP, TSN, discovery, and security traffic lives.

## 15. VLAN-tagged frames are especially important in automotive

Automotive TSN and Audio Video Bridging use VLAN tagging not as an optional enterprise convenience, but as a traffic-separation and priority mechanism.

AVB uses Priority Code Point (PCP) and VLAN identifier values to segregate time-sensitive traffic from lower-priority traffic, and that specialized filtering of traffic based on these tag values can significantly relieve software burden. The OPEN Alliance automotive Layer 2 test specification then makes that operationally concrete: it requires that frames be forwarded or dropped exactly according to customer VLAN requirements, and that single- and double-tagged frames carry the correct TPID, inner and outer tags, and VIDs.

So in automotive Ethernet, VLAN tags are not just “there if you want them.” They are often part of the designed forwarding contract for the network.

## 16. Broadcast and multicast are more dangerous in vehicles

In a generic office network, broadcast and multicast are normal parts of daily operation. In a vehicle, uncontrolled flooding is much more problematic because shared switched backbones often carry traffic with tight timing budgets.

The IEEE automotive Ethernet tutorial ties the network to deterministic low latency and tight latency variation, while the enterprise data network  switching documentation show that broadcast, multicast-without-state, and unknown unicast all involve wider replication than ordinary known unicast forwarding. As an engineering consequence, automotive switch behavior has to be more disciplined about how these frame classes are contained and filtered.

That is one reason automotive validation documents, such as the OPEN Alliance Layer 2 test specification, care so much about proving that only the expected ports receive a frame under a given VLAN configuration and that unexpected forwarding does not occur.

## 17. Jumbo frames in automotive: possible, but not always desirable

Jumbo frames are certainly possible on Ethernet technology used in vehicles, because jumbo framing is fundamentally a matter of MTU support, not a completely separate protocol. But they are not automatically a good fit for every automotive traffic class.

Jumbo frames mean more than 1500 bytes of payload and are often around 9000 bytes. Larger frames improve efficiency for bulk data, but they also take longer to serialize onto a link. The IEEE automotive Ethernet tutorial emphasizes tight latency and low variation requirements. From those two facts, the engineering inference is straightforward: jumbo frames may be useful for bulk or service traffic, but they are usually less attractive for the most timing-sensitive control paths because larger frames can increase blocking and serialization delay unless the network is engineered carefully around them.

## 18. The most useful automotive takeaway

In ordinary data networking, frame formats are mainly about compatibility and protocol identification.

In automotive Ethernet, the same frame formats become part of a system contract:

- EtherType-based framing carries modern vehicle protocols cleanly

- VLAN tags carry traffic class and forwarding-scope meaning

- broadcast, multicast, and unknown-unicast behavior must be tightly controlled

- filtering correctness is something OEMs and suppliers actively test, not assume.