Below is the clean way to think about these interfaces.

An Ethernet controller is usually split into a Media Access Control (MAC) block and a Physical Layer (PHY) block. The MAC handles frame formatting, Medium Access Control logic, and the interface upward into the host or switch core. The PHY handles line-side signaling, clock recovery, coding, link establishment, and the actual electrical connection to copper, fiber, or single-pair automotive media. The whole point of a Media-Independent Interface (MII) family interface is to let you keep the MAC largely unchanged while swapping PHYs for different physical media. NXP states this directly: the media-independent interface was originally defined to connect the MAC block to the PHY chip, and “media-independent” means different PHY types can be used without redesigning the MAC hardware.

A second important point is that these names do not all have the same standards status. In the IEEE ecosystem, MII is tied to Clause 22, Gigabit Media-Independent Interface (GMII) to Clause 35, and 10-gigabit Media-Independent Interface (XGMII) to Clause 46. By contrast, Reduced Media-Independent Interface (RMII) and Reduced Gigabit Media-Independent Interface (RGMII) are industry specifications, and Serial Gigabit Media-Independent Interface (SGMII) is identified in the IEEE contribution as a Cisco specification. That matters because implementation details and interoperability expectations are often tighter and more uniform for the IEEE-defined interfaces than for the reduced-pin and serial derivatives.

## 1. Where this interface sits in the MAC–PHY stack

Before jumping into acronyms, it helps to place the boundary correctly.

On the line side of the PHY you have the actual cable-facing functions: Physical Medium Dependent (PMD) behavior, line drivers and receivers, equalization, echo cancellation, and so on. Microchip’s PHY overview describes the PMD as the part dependent on the supported standard, including the line drivers and line receivers. Above that, inside or around the PHY, are Physical Medium Attachment (PMA) and Physical Coding Sublayer (PCS) type functions. The MAC-facing interface is above those line-oriented functions and is the digital handoff between MAC-side logic and PHY-side logic.

So when you see MII, RMII, GMII, RGMII, SGMII, or XGMII, do not think “cable interface.” Think “chip-to-chip or block-to-block digital interface between MAC and PHY.”

## 2. The evolutionary pattern

All six interfaces come from the same engineering pressures:

- Higher Ethernet speed needs either more data wires, higher clock rate, double-data-rate transfer, or serialization.

- Board designers want fewer pins and fewer traces.

- Fewer pins usually mean more multiplexing or tighter timing.

- Serial interfaces reduce pins dramatically, but then require Serializer/Deserializer (SerDes) circuitry and more careful signal integrity.

That is the whole story in one sentence. The interface family evolved from a relatively simple parallel bus, to reduced-pin parallel buses, and then to serial links.

## 3. Media-Independent Interface (MII)

Media-Independent Interface (MII) is the classic 10/100 Mb/s MAC-to-PHY interface.

IEEE’s scorecard for MII shows: - maximum data rate: 100 Mbit/s, - signal count: 16 data/control signals plus 2 management signals, - maximum bus clock: 25 MHz, - clocking style: PHY-synchronous.

NXP’s signal summary shows the basic transmit-side signals as TX_CLK, TXD[3:0], TX_EN, and optionally TX_ER, and the receive-side signals as RX_CLK, RXD[3:0], RX_DV, RX_ER, plus CRS and COL. In plain language, transmit and receive each use a 4-bit path, and the PHY supplies the clocks for data movement.

### What MII fundamentally means

MII is a 4-bit parallel nibble interface. At 100 Mb/s, 4 bits per clock at 25 MHz gives 100 Mb/s. At 10 Mb/s, the clock scales down accordingly. The transmit and receive directions are separate, so MAC and PHY can operate full-duplex without sharing the same data wires.

### Why MII looks the way it does

MII comes from the era when pin count was still acceptable and timing closure at 25 MHz was easy. The interface carries explicit information for: - transmit enable, - optional error signaling, - receive data valid, - receive error, - carrier sense, - collision detect.

Those last two, carrier sense and collision detect, are artifacts of older Ethernet media-access behavior and half-duplex operation. They are much less central in modern full-duplex Gigabit and faster systems, which is one reason later interfaces start dropping or multiplexing them.

### Strengths of MII

MII is simple, debuggable, and forgiving. On a scope or logic analyzer, it is easy to understand. If you want a teaching interface for MAC–PHY fundamentals, MII is excellent.

### Weaknesses of MII

It costs pins and traces. Once port counts rise or board area shrinks, MII becomes unattractive.

## 4. Reduced Media-Independent Interface (RMII)

Reduced Media-Independent Interface (RMII) is the 10/100 Mb/s reduced-pin version of MII.

IEEE’s scorecard for RMII shows: - maximum data rate: 100 Mbit/s, - signal count: 7 data/control signals plus 1 clock plus 2 management signals, - maximum bus clock: 50 MHz, - clocking style: system-synchronous.

NXP’s signal list shows the key RMII signals: - a continuous 50 MHz REF_CLK, - TXD0, TXD1, TX_EN, - RXD0, RXD1, - CRS_DV, which multiplexes carrier sense and receive data valid, - and optionally RX_ER.

### What RMII fundamentally changes

RMII cuts the data path from 4 bits to 2 bits and raises the clock to 50 MHz for 100 Mb/s operation. That preserves bandwidth while reducing pins.

But the deeper conceptual change is the clocking model. MII is PHY-synchronous, while RMII is system-synchronous. In RMII, both sides work from a common 50 MHz reference clock rather than separate transmit and receive clocks from the PHY.

### Why RMII exists

RMII exists for pin-count reduction. Microchip explicitly says RMII is the reduced pin-count version of MII.

### What you pay for the lower pin count

You pay in signal multiplexing and tighter interpretation rules. The most obvious example is CRS_DV, which combines carrier sense and receive data valid on one signal. NXP notes that CRS and RX_DV are multiplexed on alternate clock cycles, with a special behavior in 10 Mb/s mode.

That means RMII is no longer as transparent as MII. The logic is still manageable, but the interface is less “literal” and more encoded.

### Practical view

RMII is common on low-cost microcontrollers, basic 10/100 embedded designs, and systems where one wants Ethernet without spending many pins. In automotive and industrial single-port 100 Mb/s designs, RMII is often chosen when the MAC block is modest and pin budget is tight.

## 5. Gigabit Media-Independent Interface (GMII)

Gigabit Media-Independent Interface (GMII) is the natural extension of MII to Gigabit Ethernet.

IEEE’s scorecard for GMII shows: - maximum data rate: 1000 Mbit/s, - signal count: 25 data/control signals plus 2 management signals, - maximum bus clock: 125 MHz, - clocking style: source-synchronous.

### What GMII fundamentally means

GMII is an 8-bit parallel interface at 125 MHz for 1 Gb/s operation. That is the Gigabit analogue of MII’s 4-bit-at-25-MHz style. The interface is wider because simply scaling the original MII clock much higher would have been unattractive, so designers widened the data path instead.

### Why GMII matters conceptually

GMII is the “clean” Gigabit MAC–PHY handoff. It preserves the explicit parallel nature of the interface and avoids the extra multiplexing tricks of reduced-pin variants. It is therefore conceptually very easy to explain and architecturally tidy.

### Why GMII is not universally loved in boards

The problem is pin count. Microchip notes that GMII has roughly twice the number of pins as MII, and in high-port-count Gigabit designs that pin count becomes limiting. That pin pressure is exactly why reduced-pin alternatives such as RGMII and serial alternatives such as SGMII became popular.

### Practical view

You still see GMII inside intellectual property blocks, older designs, FPGA systems, or places where clarity is more important than pin minimization. But on real boards, RGMII and SGMII usually win.

## 6. Reduced Gigabit Media-Independent Interface (RGMII)

Reduced Gigabit Media-Independent Interface (RGMII) is the reduced-pin Gigabit version.

IEEE’s scorecard for RGMII shows: - maximum data rate: 1000 Mbit/s, - signal count: 12 data/control signals plus 2 management signals, - maximum bus clock: 125 MHz, - clocking style: source-synchronous double-data-rate.

The RGMII Version 2.0 specification states its purpose very clearly: it is intended as an alternative to MII and GMII, with the objective of reducing the MAC–PHY interconnect pin count. It does this by reducing the data paths, multiplexing control, and using both edges of the clock. The specification also states that for Gigabit operation the clock is 125 MHz, and for 10/100 operation the clocks are 2.5 MHz or 25 MHz respectively.

NXP’s signal summary shows the core idea: - TXC, TXD[3:0], TX_CTL on transmit, - RXC, RXD[3:0], RX_CTL on receive, where TX_CTL multiplexes transmit enable and transmit error, and RX_CTL multiplexes receive data valid and receive error.

### What RGMII fundamentally means

RGMII keeps only 4 data bits per direction, but it transfers on both rising and falling clock edges. That is why a 4-bit bus at 125 MHz can still carry 1 Gb/s. This is the most important mental model for RGMII:

- GMII gets Gigabit bandwidth by being wide.

- RGMII gets Gigabit bandwidth by being narrower but using double-data-rate (DDR) timing.

### The really important engineering issue: timing skew

RGMII is famous for clock-to-data skew headaches. The Version 2.0 specification explicitly changed timing so the transmitter may integrate the delay that older versions expected the printed circuit board to provide, and it defines devices implementing this internal delay as RGMII-ID. The timing table shows receiver-side skew and setup/hold expectations, and the note states that devices with internal delay may still be compliant.

This is the design issue students and engineers most often underestimate.

In plain language: - The receiver must sample DDR data at the right instant. - That requires clock edges to be shifted appropriately relative to the data. - In older RGMII practice, board trace delay often created that skew. - In newer RGMII-ID devices, the transmitter or receiver may insert that delay internally.

If the MAC assumes internal delay and the PHY also adds it, or if neither adds it, setup and hold margins collapse. Many “mysterious” RGMII bring-up bugs are really delay-configuration mismatches.

### Why RGMII became dominant

RGMII is the sweet spot when: - you need 10/100/1000 support, - you want low pin count, - you do not want to pay for a full serial SerDes interface.

That is why RGMII appears constantly in processors, switch CPUs, and Gigabit PHY datasheets. TI’s DP83867, for example, exposes RGMII or SGMII toward the MAC.

## 7. Serial Gigabit Media-Independent Interface (SGMII)

Serial Gigabit Media-Independent Interface (SGMII) is the serial version of the MAC–PHY interconnect for 10/100/1000 operation.

The IEEE contribution identifies it as a Cisco specification and summarizes it as: - maximum data rate: 1000 Mbit/s, - signal count: 4 per port plus 2 management, - SerDes maximum rate: 1.25 Gb/s, - clocking style: SerDes or source-sync, - command space carried via ordered sets.

The original Serial-GMII specification says SGMII was designed to convey network data and port speed between a 10/100/1000 PHY and a MAC with significantly fewer pins than GMII. It uses two data signals and two clock signals, with data at 1.25 Gbaud and clocks at 625 MHz as a double-data-rate interface. It further states that the traditional GMII data, valid, and error signals are encoded and serialized, leveraging the 1000BASE-X Physical Coding Sublayer (PCS) from IEEE 802.3z Clause 36, while control information is transferred using Clause 37 auto-negotiation mechanisms.

Microchip summarizes SGMII more simply: it sits between MAC and PHY, takes GMII signals and serializes them, then deserializes them on the receive side.

### What SGMII fundamentally means

SGMII replaces the parallel bus with a SerDes link. Instead of many single-ended data wires and separate parallel clocks, you now have high-speed serial signaling, usually as differential pairs.

This gives three major advantages:

- very low pin count,

- cleaner routing for high port density,

- easier extension across somewhat longer on-board distances than a wide parallel bus.

### What SGMII borrows from Ethernet PCS logic

This is an important conceptual distinction. SGMII is not “just faster wires.” It uses serialization and coding ideas associated with Gigabit Ethernet PCS behavior. That is why the spec explicitly refers to Clause 36 PCS and Clause 37 auto-negotiation style control transfer.

### How SGMII handles 10 and 100 Mb/s even though the serial lane is 1.25 Gbaud

The lane rate stays high while the effective MAC-side port speed is represented through control and serialization behavior. The spec explains that the MAC receives control information from the PHY via a Clause 37-style configuration mechanism, so the MAC knows whether the port is running at 10, 100, or 1000.

### Embedded-clock SGMII versus explicit-clock descriptions

You will see two descriptions in industry documents: - older or descriptive texts talk about two data signals and two clock signals, - many modern PHY datasheets talk about embedded clock SGMII, meaning the serial stream itself carries the timing and the receiver recovers clock from it.

TI’s DP83867 explicitly describes its MAC-side option as embedded clock SGMII.

That is why modern board designers usually think of SGMII as a pair-based serial interface with clock recovery, not as a wide clocked bus.

### Where SGMII is especially useful

SGMII is excellent for: - Gigabit switch uplinks to a processor, - PHY-to-switch links, - SFP-style or fiber-oriented modules, - high-port-density boards.

In automotive and modern industrial designs, SGMII is common when a System on Chip or switch core already has SerDes resources.

## 8. 10-gigabit Media-Independent Interface (XGMII)

10-gigabit Media-Independent Interface (XGMII) is the 10 Gb/s MAC-to-PHY interface.

The IEEE draft text for Clause 46 says the purpose of XGMII is to provide a simple, inexpensive, easy-to-implement interconnection between the MAC sublayer and the PHY, and notes that 10 Gigabit Attachment Unit Interface (XAUI) may optionally be used to extend the distance with reduced pin count.

The same IEEE material states that XGMII is composed of independent transmit and receive paths, each using: - 32 data signals, - 4 control signals, - and a transmit or receive clock.

### What XGMII fundamentally means

XGMII is the 10 Gigabit parallel MAC–PHY handoff. Conceptually, it does for 10 Gb/s what GMII did for 1 Gb/s: it is the direct, explicit, parallel interface at the MAC/PHY boundary.

The reason the math works is: - 32 bits are transferred on each clock edge, - both edges are used, - so effectively 64 bits move per full clock cycle, - and 64 × 156.25 MHz = 10 Gb/s.

### Why XGMII is not usually routed very far on a board

Because it is wide and fast. Even though it is conceptually clean, a 32-bit-per-direction plus control interface is physically heavy. That is why Clause 46 material itself mentions XAUI as the reduced-pin extension option.

So the same evolution pattern repeats: - direct parallel interface for architectural clarity, - reduced-pin serialized alternative for practical packaging.

### Important practical note

Many FPGA or intellectual-property vendors expose internal XGMII-like interfaces that are 64-bit single-data-rate rather than the external standards-style 32-bit double-data-rate version. Intel’s older user guide explicitly notes that the standard XGMII implementation is 32-bit wide at both clock edges of 156.25 MHz, while some internal implementations use a single-data-rate form for convenience.

That means you should always ask: “Is this the external standards-style XGMII, or an internal vendor-adapted XGMII-like bus?”

## 9. The common design logic behind the six interfaces

Now you can see the family relationship clearly.

### MII

Oldest simple parallel bus for 10/100.

### RMII

Same speed range, fewer pins, more multiplexing, common 50 MHz reference clock.

### GMII

Clean Gigabit parallel bus, but pin-heavy.

### RGMII

Gigabit with fewer pins by using double-data-rate transfer and multiplexed control.

### SGMII

Gigabit with dramatically fewer pins by serializing the GMII-style information over SerDes.

### XGMII

10 Gigabit parallel MAC–PHY boundary.

The progression is really: parallel wide → parallel reduced-pin → serial low-pin.

## 10. Clocking styles matter more than many newcomers realize

The clocking model is one of the deepest differences among these interfaces.

MII is PHY-synchronous. The PHY provides timing on the interface.

RMII is system-synchronous. Both sides share a common 50 MHz reference clock.

GMII is source-synchronous. The transmitting side sends data with the associated clock.

RGMII is source-synchronous and double-data-rate, which is why skew management becomes critical.

SGMII is fundamentally SerDes-style clocking or clock recovery, even though the original spec also discusses DDR clocks in the interface description.

XGMII is high-speed parallel MAC/PHY handoff at 10G, with reduced-pin serial alternatives commonly used when real packaging constraints dominate.

If you understand that one paragraph, you understand most bring-up problems.

## 11. Management signals and why they stay around

Most of these interfaces are about data transfer, but Ethernet PHY management is usually separate. The scorecards for MII, RMII, GMII, RGMII, and SGMII all explicitly count 2 management signals, which are the familiar Management Data Clock (MDC) and Management Data Input/Output (MDIO) style interface.

This matters because beginners sometimes think SGMII “replaces everything with a serial pair.” It does not. It greatly reduces the data-path pins, but management access usually still remains as a separate control interface.

## 12. The most common practical mistakes

### With MII

People forget it is fundamentally a 10/100 interface and try to stretch it into places where Gigabit is needed.

### With RMII

People underestimate that CRS_DV is multiplexed and that the common 50 MHz reference clock architecture must be correct.

### With GMII

The usual issue is simply board cost and pin budget.

### With RGMII

The classic mistake is delay misconfiguration: - board-added delay when internal delay is already enabled, - no delay when both MAC and PHY expect the other side to provide it, - misunderstanding whether the device is in RGMII or RGMII-ID mode.

### With SGMII

The common mistake is to think it is just “a faster RGMII.” It is not. It is a serial PCS/SerDes style interface with encoded control behavior, line-rate considerations, and signal-integrity requirements very different from RGMII. TI’s automotive Gigabit PHY documentation explicitly flags SGMII as having stringent signal-integrity requirements for the MAC interface.

### With XGMII

The mistake is mixing up standards-style external XGMII with vendor-internal XGMII-like buses.

## 13. A good mental rule for choosing among them

If you are teaching or debugging fundamentals, think like this:

- MII: simplest conceptual model for 10/100.

- RMII: low-pin 10/100 microcontroller-friendly choice.

- GMII: clean textbook Gigabit interface.

- RGMII: practical mainstream board-level Gigabit choice.

- SGMII: low-pin SerDes Gigabit choice for denser or more modern systems.

- XGMII: parallel 10G MAC/PHY boundary, usually internal or short-reach on board; serialized alternatives often used in practice.

## 14. Automotive-oriented note

Since your context is automotive Ethernet switch training, one practical point is worth stating explicitly.

Automotive 100BASE-T1 and 1000BASE-T1 PHYs often present familiar MAC-side interfaces even though the cable side is single-pair automotive Ethernet. TI’s automotive PHY documentation, for example, shows MAC-side support for MII, RMII, RGMII, and SGMII depending on the device. That is exactly why these interfaces still matter in automotive work: the line side may be 100BASE-T1 or 1000BASE-T1, but the MAC-side digital handoff inside the electronic control unit or switch still uses these classic MAC–PHY interface families.

## 15. Final takeaway

Do not memorize these as six unrelated acronyms.

See them as answers to one repeated engineering question:

How do I carry Ethernet MAC-side transmit, receive, status, and speed information to a PHY with the right tradeoff among simplicity, pin count, clocking complexity, and achievable speed?

- MII answers with a simple parallel nibble bus.

- RMII answers with fewer pins and a shared reference clock.

- GMII answers with a wider bus for Gigabit.

- RGMII answers with fewer pins by using double-data-rate signaling.

- SGMII answers with serialization and SerDes.

- XGMII answers for 10 Gigabit with a much wider/faster parallel boundary.

Absolutely. Here is the signal-by-signal, timing-waveform-style walkthrough.

The most useful way to read all of these interfaces is to ask the same four questions every time:

- Who drives the clock?

- Which side drives the data/control signals?

- On which clock edge is the receiver supposed to sample?

- Is the control signal a dedicated wire, or is it multiplexed/encoded?

## 1. MII — Media-Independent Interface

For Media-Independent Interface (MII) at 10/100 Mb/s, the PHY provides TX_CLK and RX_CLK; the MAC drives TX_EN, TXD[3:0], and optional TX_ER; the PHY drives RX_DV, RXD[3:0], RX_ER, CRS, and COL. On a typical PHY, TX_EN and TXD[3:0] are synchronous to TX_CLK, while RX_DV and RXD[3:0] are synchronous to RX_CLK. TX_CLK is 25 MHz at 100 Mb/s and 2.5 MHz at 10 Mb/s, and CRS/COL do not have to transition synchronously to either clock.

### MII transmit side

Clock direction: PHY → MAC
Data/control direction: MAC → PHY

Signals: - TX_CLK — transmit reference clock from PHY - TX_EN — transmit enable from MAC - TXD[3:0] — 4-bit transmit data nibble from MAC - TX_ER — optional transmit error from MAC

Waveform-style view:

TX_CLK   __/‾\__/‾\__/‾\__/‾\__
TX_EN    ____‾‾‾‾‾‾‾‾‾‾‾‾________
TXD[3:0] ==== D0 ==== D1 ==== D2 ====
             ^      ^      ^
             PHY samples on rising edges

How to read that:

- When TX_EN=1, the MAC is telling the PHY that the values on TXD[3:0] are valid frame data.

- One nibble moves per clock.

- The PHY uses the rising edge of TX_CLK as the sampling instant. TX_EN is also interpreted with that same timing.

So in MII, the simplest mental model is:

- TX_CLK is the beat,

- TX_EN says “this beat carries frame data,”

- TXD[3:0] is the nibble sampled on that beat.

### MII receive side

Clock direction: PHY → MAC
Data/control direction: PHY → MAC

Signals: - RX_CLK — receive reference clock from PHY - RX_DV — receive data valid - RXD[3:0] — 4-bit receive data nibble - RX_ER — receive error - CRS — carrier sense - COL — collision detect

Waveform-style view:

RX_CLK   __/‾\__/‾\__/‾\__/‾\__
RX_DV    ____‾‾‾‾‾‾‾‾‾‾‾‾________
RXD[3:0] ==== R0 ==== R1 ==== R2 ====
             ^      ^      ^
             MAC samples on rising edges
RX_ER    ____________________‾________   (if PHY sees receive error)

How to read that:

- RX_DV=1 means the nibble on RXD[3:0] is valid receive data.

- The MAC samples RX_DV and RXD[3:0] using the rising edge of RX_CLK.

- CRS and COL are additional sideband status signals, mainly relevant to older/half-duplex behavior, and they are not required to be synchronous to RX_CLK or TX_CLK.

## 2. RMII — Reduced Media-Independent Interface

In Reduced Media-Independent Interface (RMII), the wide split-clock MII bus is collapsed into a narrower interface around one continuous 50 MHz reference clock. REF_CLK is the timing reference for TX_EN, TXD[1:0], CRS_DV, RXD[1:0], and RX_ER. TX_EN transitions synchronously to REF_CLK; RXD[1:0] also transitions synchronously to REF_CLK; and CRS_DV is the combined carrier-sense/data-valid indication. A subtle point is that CRS_DV assertion itself may be asynchronous to REF_CLK, so RXD[1:0] can remain 00 until decoding locks and proper receive data begins.

### RMII transmit side

Clock direction: shared/common reference
Data/control direction: MAC → PHY

Signals: - REF_CLK — continuous 50 MHz reference - TX_EN — transmit enable - TXD[1:0] — 2-bit transmit data

Waveform-style view:

REF_CLK  __/‾\__/‾\__/‾\__/‾\__/‾\__
TX_EN    ____‾‾‾‾‾‾‾‾‾‾________________
TXD[1:0] ==d0==d1==d2==d3==d4==d5=====
            ^   ^   ^   ^   ^
            PHY latches on REF_CLK edges

How to read that:

- RMII moves 2 bits per clock, not 4.

- TX_EN=1 means the di-bits on TXD[1:0] belong to the outgoing frame.

- TX_EN is asserted synchronously with the first preamble nibble and remains asserted while all di-bits are presented.

### RMII receive side

Clock direction: same REF_CLK domain
Data/control direction: PHY → MAC

Signals: - CRS_DV — combined Carrier Sense / Receive Data Valid - RXD[1:0] — 2-bit receive data - optional RX_ER

Waveform-style view:

REF_CLK  __/‾\__/‾\__/‾\__/‾\__/‾\__
CRS_DV   ____/‾‾‾‾‾‾‾‾‾‾\____________
RXD[1:0] ==00==00==01==01==01==11==...
             |   |   preamble   SFD
             MAC samples on REF_CLK edges

How to read that:

- CRS_DV is not the same as plain RX_DV; it is a merged signal.

- Once CRS_DV is asserted, RXD[1:0] becomes meaningful, but the PHY may hold RXD[1:0]=00 until receive decoding is ready.

- In 100 Mb/s mode, the PHY drives preamble then start-of-frame delimiter on RXD[1:0], and the MAC should begin real data capture after the start-of-frame delimiter.

- In 10 Mb/s mode, the reference clock stays at 50 MHz while the data is 10 times slower, so the MAC effectively samples useful new values every 10th cycle.

So the practical interpretation of CRS_DV is:

- it tells you there is carrier and/or valid receive activity,

- but you still need to interpret RXD[1:0] carefully at the start of reception.

## 3. GMII — Gigabit Media-Independent Interface

For Gigabit Media-Independent Interface (GMII) at 1000 Mb/s, the MAC sources GTX_CLK at 125 MHz on transmit, the PHY recovers and sources RX_CLK at 125 MHz on receive, and the data path is a full byte: TXD[7:0] and RXD[7:0]. TX_EN, TXD[7:0], and TX_ER are sampled by the PHY on the rising edge of GTX_CLK, while RX_DV, RXD[7:0], and RX_ER are sampled by the MAC on the rising edge of RX_CLK. CRS and COL remain PHY-driven sideband signals and need not be synchronous to those clocks.

### GMII transmit side

Clock direction: MAC → PHY
Data/control direction: MAC → PHY

Signals: - GTX_CLK — 125 MHz transmit clock from MAC - TX_EN - TXD[7:0] - TX_ER

Waveform-style view:

GTX_CLK  __/‾\__/‾\__/‾\__/‾\__
TX_EN    ____‾‾‾‾‾‾‾‾‾‾________
TXD[7:0] === B0 ===== B1 ===== B2 ===
             ^        ^        ^
             PHY samples on rising edges

Interpretation:

- One byte moves per clock.

- TX_EN marks the valid byte window.

- This is the clean byte-wide Gigabit version of MII.

### GMII receive side

Clock direction: PHY → MAC
Data/control direction: PHY → MAC

Signals: - RX_CLK - RX_DV - RXD[7:0] - RX_ER

Waveform-style view:

RX_CLK   __/‾\__/‾\__/‾\__/‾\__
RX_DV    ____‾‾‾‾‾‾‾‾‾‾________
RXD[7:0] === B0 ===== B1 ===== B2 ===
             ^        ^        ^
             MAC samples on rising edges

Interpretation:

- RX_DV is the byte-valid strobe.

- The MAC samples the received byte on each rising edge of RX_CLK.

## 4. RGMII — Reduced Gigabit Media-Independent Interface

Reduced Gigabit Media-Independent Interface (RGMII) cuts the data bus back to 4 bits but uses both clock edges. At 1000 Mb/s, TXC is sourced by the MAC and RXC by the PHY; both run at 125 MHz, and TXD[3:0]/RXD[3:0] transfer data on both rising and falling edges. TX_CTL and RX_CTL are multiplexed control signals: the rising edge carries TX_EN or RX_DV, and the falling edge carries the encoded error information. The RGMII v2.0 spec states the coding as TXERR = GMII_TX_ER XOR GMII_TX_EN and RXERR = GMII_RX_ER XOR GMII_RX_DV, with the enable/valid state taken on the rising edge and the error-encoded value on the falling edge. The same spec and vendor timing guides also emphasize internal-delay mode, commonly called RGMII-ID, because correct clock-to-data skew is critical.

### RGMII transmit side

Clock direction: MAC → PHY
Data/control direction: MAC → PHY

Signals: - TXC - TXD[3:0] - TX_CTL

At 1 Gb/s:

TXC      __/‾\__/‾\__/‾\__/‾\__
TXD[3:0] ==L0==H0==L1==H1==L2==H2==
          ^   ^   ^   ^   ^   ^
          |   |   |   |   |   |
          rise fall rise fall rise fall

TX_CTL   ==EN==ER'==EN==ER'==EN==ER'==

Where: - on the rising edge, TX_CTL = TX_EN - on the falling edge, TX_CTL = TX_EN XOR TX_ER

Interpretation:

- L0 is the lower nibble of one byte.

- H0 is the upper nibble of that byte.

- So one full byte is transferred across one full TXC period using both edges.

- If there is no transmit error, TX_CTL stays high on both edges during valid frame transfer.

- If TX_ER=1 while TX_EN=1, the falling-edge value flips low because of the XOR rule.

### RGMII receive side

Clock direction: PHY → MAC
Data/control direction: PHY → MAC

Signals: - RXC - RXD[3:0] - RX_CTL

Waveform-style view:

RXC      __/‾\__/‾\__/‾\__/‾\__
RXD[3:0] ==L0==H0==L1==H1==L2==H2==
          ^   ^   ^   ^   ^   ^
          MAC samples on both edges

RX_CTL   ==DV==ER'==DV==ER'==DV==ER'==

Where: - on the rising edge, RX_CTL = RX_DV - on the falling edge, RX_CTL = RX_DV XOR RX_ER

Interpretation:

- If a frame is valid and error-free, RX_CTL is high on both edges during the active data window.

- If there is a receive error during a valid frame, the falling-edge control state changes according to the XOR encoding.

- This is why you cannot read RX_CTL as a simple level signal in RGMII. You must interpret it edge-by-edge.

### The skew warning

RGMII only works reliably if clock and data arrive with the intended relative delay. The v2.0 spec explicitly allows internal delay in the transmitter or receiver and refers to such implementations as RGMII-ID. In practice, many bring-up failures are not protocol problems at all; they are wrong delay assumptions between MAC, PHY, and board layout.

## 5. SGMII — Serial Gigabit Media-Independent Interface

With Serial Gigabit Media-Independent Interface (SGMII), the big conceptual shift is this: you no longer see separate parallel pins like TX_EN, RX_DV, TX_ER, or RX_ER on the MAC–PHY boundary. The SGMII specification says the traditional GMII data signals TXD/RXD, valid signals TX_EN/RX_DV, and error signals TX_ER/RX_ER are encoded, serialized, and output over the serial interface. The original SGMII spec describes two data signals and two clock signals, with data at 1.25 Gbaud and clocks at 625 MHz in DDR form, and it says the receiver samples data on both edges of the clock. It also states that CRS is inferred from RX_DV, and COL is logically derived in the MAC when RX_DV and TX_EN are simultaneously asserted.

### What this means for your requested signals

For SGMII:

- there is no external TX_EN pin

- there is no external RX_DV pin

- there is no external CRS_DV pin

- there is no TX_CTL or RX_CTL pin

Those semantics still exist, but they exist inside the encoded serial stream, not as separate interface wires.

Waveform-style mental picture:

TX differential pair   ==== serialized encoded stream ====
TXCLK                  _/‾\_/‾\_/‾\_/‾\_/‾\_   (DDR style in original spec)

Inside the stream:
[TXD bits][TX_EN info][TX_ER info][control words]

So the correct way to think of SGMII is:

- parallel GMII-style semantics go in,

- the interface encodes and serializes them,

- the other side deserializes and reconstructs the GMII-style meaning.

## 6. XGMII — 10-Gigabit Media-Independent Interface

With 10-gigabit Media-Independent Interface (XGMII), the parallel side returns, but the signal model changes. XGMII does not use TX_EN, RX_DV, CRS_DV, TX_CTL, or RX_CTL. Instead, each direction has TXD[31:0] or RXD[31:0], plus TXC[3:0] or RXC[3:0], plus TX_CLK or RX_CLK. IEEE draft material says each direction uses 32 data signals, 4 control signals, and a clock, and the PHY samples TXC<3:0> and TXD<31:0> on both the rising and falling edges of TX_CLK; the same source describes TX_CLK as 156.25 MHz.

### What replaces TX_EN and RX_DV in XGMII

Instead of one enable wire for the whole bus, XGMII uses a per-lane control indication. The 32-bit bus is split into 4 lanes of 8 bits. A corresponding control bit in TXC[3:0] or RXC[3:0] tells the PHY or MAC whether the associated lane carries a data byte or a control character. IEEE material describes Sequence control characters and their lane placement, which shows how control characters are mixed into the 4-lane interface rather than being indicated by a single TX_EN/RX_DV pin.

Waveform-style view:

TX_CLK      __/‾\__/‾\__/‾\__/‾\__
TXD[31:0]   [4 bytes @ rise] [4 bytes @ fall] [4 bytes @ rise] ...
TXC[3:0]    [lane ctrl bits] [lane ctrl bits] [lane ctrl bits] ...
             ^              ^
             PHY samples on both edges

Interpretation:

- On each edge, four byte lanes are presented.

- TXC[0] belongs to byte lane 0, TXC[1] to lane 1, and so on.

- So XGMII is not frame-valid signaling with one wire. It is lane-by-lane data/control tagging.

## 7. The quick intuition you should retain

If you strip everything down:

- MII: dedicated TX_EN and RX_DV, sampled on rising edges of PHY-provided clocks.

- RMII: dedicated TX_EN, but receive validity merged into CRS_DV; everything timed to a shared REF_CLK.

- GMII: same idea as MII, just byte-wide at Gigabit speed.

- RGMII: no separate TX_ER/RX_ER wires; TX_CTL and RX_CTL carry enable/valid on rising edge and error encoding on falling edge.

- SGMII: TX_EN, RX_DV, and friends are no longer pins; they are encoded inside the serial stream.

- XGMII: no single frame-valid pin; instead, each byte lane has its own control indication through TXC/RXC.

Yes. The cleanest way to do this is to start from the same 8-byte sequence and then watch how each interface slices it differently.

The Ethernet packet lead-in seen at the MAC-to-PHY boundary is 7 bytes of preamble followed by 1 byte of Start Frame Delimiter (SFD). In byte form that is:

55 55 55 55 55 55 55 D5

The preamble is seven octets, the SFD is one octet, and the SFD octet is 0xD5. IEEE material also shows the preamble bytes as repeated 0x55.

The other key rule is ordering. Ethernet transmits each octet least-significant bit first, and the IEEE interpretation note explicitly points to Clause 22 Figure 22-11 for the bit and nibble transmission order across Media-Independent Interface (MII). For Reduced Gigabit Media-Independent Interface (RGMII) at 1000 Mb/s, Microchip states directly that the low nibble bits [3:0] appear first and the high nibble bits [7:4] appear second. For Reduced Media-Independent Interface (RMII), the receive and transmit order is lower-order di-bit first, then higher-order di-bit.

So the one byte that really matters is the SFD byte:

- 0x55 = low nibble 0x5, high nibble 0x5

- 0xD5 = low nibble 0x5, high nibble 0xD

- nibble 0x5 = binary 0101 → di-bits 01, then 01

- nibble 0xD = binary 1101 → di-bits 01, then 11

## 1. Ground truth before we map to interfaces

Here is the exact frame lead-in in progressively narrower chunks:

### Byte view

Preamble: 55 55 55 55 55 55 55
SFD:      D5

### Nibble view

Preamble: 5 5  5 5  5 5  5 5  5 5  5 5  5 5
SFD:      5 D

### Di-bit view

Each 0x55 byte -> 01 01 01 01
0xD5 byte      -> 01 01 01 11

That is the reconstruction target. The only thing the interface changes is how many bits move per sampling event.

## 2. MII — Media-Independent Interface byte-to-nibble reconstruction

In Media-Independent Interface (MII), transmit data is 4 bits wide and the PHY samples TXD[3:0] and TX_EN on the rising edge of TX_CLK. On receive, the MAC samples RXD[3:0] and RX_DV on the rising edge of RX_CLK. IEEE’s interpretation note says the bit and nibble order is defined in Clause 22 Figure 22-11.

So on MII, each byte is split into two nibble times:

- first cycle = low nibble

- second cycle = high nibble

### One preamble byte 0x55 on MII

TX_CLK ↑ #1 : TX_EN=1, TXD[3:0]=0x5
TX_CLK ↑ #2 : TX_EN=1, TXD[3:0]=0x5

### The SFD byte 0xD5 on MII

TX_CLK ↑ #1 : TX_EN=1, TXD[3:0]=0x5   ← low nibble first
TX_CLK ↑ #2 : TX_EN=1, TXD[3:0]=0xD   ← high nibble second

So if you expand the full 7-byte preamble plus SFD, the nibble stream is:

Cycle:   1  2  3  4  5  6  7  8  9 10 11 12 13 14 15 16 17 18
TXD:     5  5  5  5  5  5  5  5  5  5  5  5  5  5  5  5  5  D
TX_EN:   1  1  1  1  1  1  1  1  1  1  1  1  1  1  1  1  1  1

The receive side is the same idea, just with RX_DV and RXD[3:0] instead of TX_EN and TXD[3:0].

### MII intuition

MII is the easiest one to visualize:

- each byte takes 2 clock cycles

- the preamble becomes 16 nibble cycles

- the SFD is visibly the point where the second nibble changes from 5 to D

That last point is why D5 is such a useful teaching byte. With 55, you cannot tell low nibble from high nibble because both are 5. With D5, you can.

## 3. RMII — Reduced Media-Independent Interface byte-to-di-bit reconstruction

In Reduced Media-Independent Interface (RMII), the data path is only 2 bits wide. The controller samples RXD[1:0] on the rising edge of REF_CLK, and RMII sends or receives each octet a di-bit at a time with lower-order bits first. TI’s application note and Microchip’s datasheet describe the shared REF_CLK timing, while the RMII bit-order explanation shows lower-order di-bit first, then higher-order di-bit.

A nibble must therefore be broken into two pieces:

- first di-bit = bits [1:0]

- second di-bit = bits [3:2]

So:

- nibble 0x5 = 0101 → 01, then 01

- nibble 0xD = 1101 → 01, then 11

### One preamble byte 0x55 on RMII

0x55 = nibble 5, nibble 5

Each 5 becomes 01, 01

So one whole preamble byte becomes:

REF_CLK ↑ #1 : TX_EN=1, TXD[1:0]=01
REF_CLK ↑ #2 : TX_EN=1, TXD[1:0]=01
REF_CLK ↑ #3 : TX_EN=1, TXD[1:0]=01
REF_CLK ↑ #4 : TX_EN=1, TXD[1:0]=01

### The SFD byte 0xD5 on RMII

0xD5 = low nibble 5, high nibble D

So the sequence is:

REF_CLK ↑ #1 : TX_EN=1, TXD[1:0]=01   ← low nibble, low di-bit
REF_CLK ↑ #2 : TX_EN=1, TXD[1:0]=01   ← low nibble, high di-bit
REF_CLK ↑ #3 : TX_EN=1, TXD[1:0]=01   ← high nibble, low di-bit
REF_CLK ↑ #4 : TX_EN=1, TXD[1:0]=11   ← high nibble, high di-bit

So the full lead-in becomes:

Each 0x55 byte -> 01 01 01 01
Each 0x55 byte -> 01 01 01 01
Each 0x55 byte -> 01 01 01 01
Each 0x55 byte -> 01 01 01 01
Each 0x55 byte -> 01 01 01 01
Each 0x55 byte -> 01 01 01 01
Each 0x55 byte -> 01 01 01 01
SFD 0xD5      -> 01 01 01 11

If you flatten that into cycle numbers:

Cycles  1-28 : 01 01 01 01 repeated seven times
Cycles 29-32 : 01 01 01 11

On receive, the same reconstruction applies, except the PHY drives CRS_DV with RXD[1:0] rather than a separate RX_DV wire. TI’s RMII note explains that CRS_DV is the combined receive indicator in RMII.

### RMII intuition

RMII is just MII made narrower:

- MII: 4 bits per cycle

- RMII: 2 bits per cycle

So whatever took 2 cycles per byte in MII takes 4 cycles per byte in RMII. That is why the same D5 boundary shows up as 01 01 01 11 instead of 5 D.

## 4. GMII — Gigabit Media-Independent Interface byte reconstruction

In Gigabit Media-Independent Interface (GMII), the data path is a full byte wide. The PHY or MAC side samples TXD[7:0], TX_EN, and TX_ER on the rising edge of GTX_CLK, and similarly RXD[7:0] and RX_DV on the rising edge of RX_CLK. Microchip timing data shows the GMII signals are byte-wide and edge-sampled.

That means GMII does not need nibble reconstruction at all. It carries the byte directly.

### One preamble byte 0x55 on GMII

GTX_CLK ↑ #1 : TX_EN=1, TXD[7:0]=0x55

### The SFD byte 0xD5 on GMII

GTX_CLK ↑ #1 : TX_EN=1, TXD[7:0]=0xD5

So the full lead-in is simply:

Cycle 1 : 55
Cycle 2 : 55
Cycle 3 : 55
Cycle 4 : 55
Cycle 5 : 55
Cycle 6 : 55
Cycle 7 : 55
Cycle 8 : D5
TX_EN   : high throughout cycles 1-8

Receive is identical in structure with RX_DV and RXD[7:0].

### GMII intuition

GMII is the most literal representation:

- one byte on the wire-like MAC side

- one cycle per byte

- SFD is simply the first cycle where the byte is D5 instead of 55

It is the easiest interface for software or FPGA debugging because the bus values are already in octet form.

## 5. RGMII — Reduced Gigabit Media-Independent Interface byte-to-edge reconstruction

For Reduced Gigabit Media-Independent Interface (RGMII) at 1000 Mb/s, Microchip states that transmit and receive use both edges of the clock, with the low nibble bits [3:0] on the rising edge and the high nibble bits [7:4] on the falling edge. The same family of documents also describes TX_CTL and RX_CTL as carrying the logical equivalents of transmit-enable/receive-valid and error information across the two edges.

So RGMII packs exactly the same nibble content as MII, but into one clock period per byte instead of two.

### One preamble byte 0x55 on RGMII

TXC rising edge  : TX_CTL=valid, TXD[3:0]=0x5
TXC falling edge : TX_CTL=valid, TXD[3:0]=0x5

### The SFD byte 0xD5 on RGMII

TXC rising edge  : TX_CTL=valid, TXD[3:0]=0x5   ← low nibble
TXC falling edge : TX_CTL=valid, TXD[3:0]=0xD   ← high nibble

Cycle-by-cycle, counting one full clock as one byte time:

Clock 1 : rise=5, fall=5
Clock 2 : rise=5, fall=5
Clock 3 : rise=5, fall=5
Clock 4 : rise=5, fall=5
Clock 5 : rise=5, fall=5
Clock 6 : rise=5, fall=5
Clock 7 : rise=5, fall=5
Clock 8 : rise=5, fall=D

So the SFD boundary is visible as:

- rising edge still looks like preamble nibble 5

- falling edge changes to D

That is the exact RGMII equivalent of MII’s 5, D.

### RGMII intuition

RGMII is best understood as:

- same nibble order as MII

- same byte meaning as GMII

- but the two nibbles of each byte are split across rising and falling clock edges

So:

- MII D5 = two clocks: 5, then D

- RGMII D5 = one clock: rise 5, fall D

## 6. Side-by-side summary for one preamble byte and the SFD byte

Here is the same information compressed.

### One preamble byte 0x55

MII    :  cycle1=5, cycle2=5
RMII   :  cycle1=01, cycle2=01, cycle3=01, cycle4=01
GMII   :  cycle1=55
RGMII  :  rise=5, fall=5

### One SFD byte 0xD5

MII    :  cycle1=5, cycle2=D
RMII   :  cycle1=01, cycle2=01, cycle3=01, cycle4=11
GMII   :  cycle1=D5
RGMII  :  rise=5, fall=D

This is the single most useful reconstruction table to remember. It follows directly from the Ethernet preamble/SFD bytes, the least-significant-bit-first ordering, MII nibble order, RMII lower-di-bit-first rule, and RGMII low-nibble-on-rising/high-nibble-on-falling rule.

## 7. Where the control signal sits during these bytes

During an error-free transmit of preamble plus SFD:

- MII: TX_EN=1 for all 18 nibble cycles

- RMII: TX_EN=1 for all 32 di-bit cycles

- GMII: TX_EN=1 for all 8 byte cycles

- RGMII: TX_CTL indicates valid transfer on each edge across the whole 8-byte lead-in

On receive:

- MII: RX_DV=1

- RMII: CRS_DV=1

- GMII: RX_DV=1

- RGMII: RX_CTL carries the receive-valid meaning on the rising edge, with the falling edge used for the error-related encoding rule.

## 8. The deepest teaching point

All four interfaces are representing the same Ethernet lead-in. They differ only in how they slice it:

- GMII shows the byte directly.

- MII splits the byte into two nibbles.

- RMII splits each nibble into two di-bits.

- RGMII keeps nibble order like MII but moves the two nibbles on opposite clock edges.

So the SFD byte 0xD5 is the best sanity-check pattern:

- if you see D5, you are in byte view,

- if you see 5 D, you are in nibble view,

- if you see 01 01 01 11, you are in di-bit view,

- if you see rise=5, fall=D, you are in RGMII edge view.