# CSMA/CD, duplexing, one-pair Ethernet, and MAC–PHY interfaces

## 1. Data-communications context

### 1.1 Why medium access had to be invented at all

If several stations share the same physical medium, a basic problem appears immediately: two stations may start transmitting at the same time. When that happens on a shared Ethernet medium, their signals interfere, and the receiver cannot decode a valid frame. Early Ethernet therefore needed a rule for who may transmit, when, and what to do if two stations still choose the same moment. That rule became Carrier Sense Multiple Access with Collision Detection (CSMA/CD). The IEEE 802.3 abstract says the CSMA/CD MAC protocol specifies shared-medium (half-duplex) operation as well as full-duplex operation, and CSMA/CD stations detect collisions and then retransmit later according to a backoff algorithm.

How do multiple Ethernet stations share one contention medium without a central scheduler?   Listen before you talk and listen while you talk.

### 1.2 Expand the acronym properly

Carrier Sense means a station first listens to the medium to see whether someone else is already transmitting.
Multiple Access means multiple stations are allowed to contend for the same shared medium.
Collision Detection means that while transmitting, the station monitors for a collision and, if one occurs, stops and retries later.

CSMA/CD is used by traditional Ethernet and IEEE 802.3 in half-duplex mode, that only one station can transmit at a time over the shared Ethernet, and that colliding stations use backoff algorithms to determine when to retransmit.

That gives the real operational sequence:

- listen to the medium

- if it is idle, start transmitting

- if a collision is detected, stop

- wait for a backoff interval

- try again.

### 1.3 Why collisions happen in the first place

A collision can happen even if every node “listens first,” because of propagation delay. One station may begin transmitting at one end of the shared medium while another station, far enough away, still thinks the line is idle and also starts transmitting. By the time each signal reaches the other end, the transmissions overlap. That is the whole reason collision detection and backoff are needed in shared-medium Ethernet.

In the retransmit-and-backoff part, in half-duplex mode, transmit media access management avoids contention by monitoring carrier sense and detects collisions via the physical layer’s collision-detect signal.

CSMA/CD is a carefully engineered compromise for shared media where there is no central arbiter.

### 1.4 Half duplex from first principles

Half duplex means a station can either transmit or receive at a given moment on that shared medium, but not both simultaneously as part of independent send/receive streams. Traditional 10/100 Mb/s Ethernet typically operates in half-duplex mode and that in that mode stations can either receive or transmit. It also says collisions are normal for half-duplex interfaces and must not appear on full-duplex interfaces.

Half duplex therefore implies two things:

- the link is treated as a contention domain

- CSMA/CD matters, because collisions are possible.

### 1.5 Full duplex from first principles

Full duplex means both ends can transmit and receive simultaneously. Full-duplex mode allows stations to transmit and receive data simultaneously, expects point-to-point links, and has no collisions, so CSMA/CD is not needed. Full duplex is the effective solution to collision-related congestion and the effective Ethernet bandwidth doubles when packets flow in both directions simultaneously.

Full duplex removes the whole collision problem by removing shared contention from the link model.

Once Ethernet became predominantly switched and point-to-point, the historical importance of CSMA/CD fell sharply. Modern switched Ethernet is usually full duplex, which means collision handling is no longer the central performance story. Queueing, buffering, scheduling, and priority are.

### 1.6 Why CSMA/CD is mostly historical in modern switched Ethernet

The IEEE 802.3 abstract still mentions CSMA/CD because it is part of the Ethernet heritage and standard model, but modern switched point-to-point links do not rely on it in the way classic shared-medium Ethernet did. In full-duplex mode there are no collisions and CSMA/CD is not needed. Full duplex expects point-to-point links.

- if the medium is shared and half duplex, CSMA/CD matters

- if the link is point to point and full duplex, CSMA/CD is effectively out of the picture.

### 1.7 One-pair Ethernet:

“One-Pair Ethernet” refers to Ethernet carried over a single twisted pair rather than the multiple pairs used in older 100BASE-TX or 1000BASE-T office-style cabling. In current engineering language this is often grouped under single-pair Ethernet. The two specific standards that matter in this case are:

- 100BASE-T1, specified in IEEE 802.3 Clause 96

- 1000BASE-T1, specified in IEEE 802.3 Clause 97.

The IEEE automotive tutorial lists 100BASE-T1 (100 Mb/s) and 1000BASE-T1 (1 Gb/s) as automotive Ethernet PHYs, and the 802.3 maintenance summary confirms their clause numbers.

### 1.8 Why one-pair Ethernet exists

The main reason is not protocol purity. It is system efficiency. A single twisted pair reduces harness bulk, weight, connector size, and cost compared with multi-pair Ethernet cabling, while still preserving Ethernet’s layered architecture. The IEEE automotive Ethernet tutorial explicitly links automotive Ethernet adoption to wiring-harness cost and weight pressure and presents 100BASE-T1 and 1000BASE-T1 as key standard links in that space.

So one-pair Ethernet is best understood as Ethernet adapted to environments where cable mass, reach, electromagnetic behavior, and connector complexity matter more than they do in office LANs.

### 1.9 100BASE-T1 overview

100BASE-T1 is the 100 Mb/s automotive single-pair PHY. The 802.3 maintenance summary identifies it as a single balanced twisted-pair copper cabling PHY in Clause 96.

Devices provide transmit and receive capability over unshielded twisted-pair cables and are compliant with IEEE 802.3bw / 100BASE-T1.

100BASE-T1 allows full-duplex bidirectional communication, the classic MII collision (COL) and carrier sense (CRS) signals are not needed.

Even though Ethernet historically grew up with CSMA/CD, 100BASE-T1 is a full-duplex point-to-point PHY, so it does not rely on collision-based medium access the way classic shared Ethernet did.

### 1.10 1000BASE-T1 overview

1000BASE-T1 is the 1 Gb/s automotive single-pair PHY. The 802.3 maintenance summary identifies it as the single twisted-pair copper cable PHY in Clause 97. IEEE 802.3bp working material says the 1000BASE-T1 PHY operates using full-duplex communications, using echo cancellation, over a single pair of balanced copper conductors. TI’s 1000BASE-T1 automotive PHY datasheet likewise says the device provides all physical-layer functions needed to transmit and receive data over a single twisted-pair cable.

That means 1000BASE-T1 is not “gigabit Ethernet with fewer wires” in a trivial sense. It depends on significantly more sophisticated PHY behavior to support simultaneous bidirectional data flow over one pair. The mention of echo cancellation in IEEE working material is the key clue that the transmitter and receiver are sharing the same pair at the same time rather than time-sharing it as a half-duplex medium.

### 1.11 Why 100BASE-T1 and 1000BASE-T1 change the access story

This is the conceptual bridge between the first half of your topic and the second half.

Classic CSMA/CD grew out of a shared-medium, half-duplex Ethernet world.
100BASE-T1 and 1000BASE-T1 are point-to-point, full-duplex PHYs.

So in practice, the interesting problems on T1 links are no longer “who wins the collision?” but rather:

- PHY signaling quality

- link startup and training

- latency and determinism

- MAC–PHY interfacing

- switching, queuing, and time synchronization.

That is why a modern automotive Ethernet course should teach CSMA/CD as essential history and conceptual grounding, but not as the center of modern T1 system behavior.

## 2. MAC–transceiver interfaces: why they exist

### 2.1 The basic idea

A Media Access Control (MAC) block and a Physical Layer Device (PHY) do different jobs. The MAC handles Ethernet framing and MAC-side logic. The PHY handles line coding, signaling, and electrical/physical link behavior. The interface between them is commonly called an xMII family interface, where “MII” means Media Independent Interface and the “x” marks different speed or pin-count variants. The IEEE 802.3 abstract says speed-specific MIIs allow selected PHY interfaces to be used over different media. The IEEE 802.3 definitions say MII is a transparent signal interface at the bottom of the reconciliation sublayer.

A MAC–PHY interface is a chip-to-chip or block-to-block boundary that lets the same MAC be reused with different PHYs.

That is exactly why the name says media independent. The MAC does not need to know whether the PHY is talking copper, fiber, multi-pair twisted pair, or single-pair automotive cabling.

### 2.2 MII

MII is the classic Media Independent Interface for 10/100 Mb/s Ethernet. IEEE 802.3 describes it as a transparent interface at the bottom of the reconciliation sublayer and says it is designed to allow flexibility in intermixing PHYs and data-terminal equipment. The same source points to Clause 22 for MII. 100 Mb/s versions of Automotive Ethernet typically include a 15-pin MII and a 9-pin Reduced MII (RMII).

MII is therefore the baseline MAC–PHY attachment concept for Fast Ethernet class devices. It is parallel, relatively pin-hungry by modern standards, and conceptually straightforward.

### 2.3 RMII

RMII means Reduced Media Independent Interface. It is essentially the nine-pin RMII, contrasted with the 15-pin MII. RMII is the lower-pin-count way to connect a 10/100 Mb/s MAC and PHY when board cost, package pins, and routing simplicity matter.

So RMII is best understood not as a different Ethernet protocol, but as a leaner electrical interface between MAC and PHY for the same 10/100 Mb/s MAC-side job.

### 2.4 GMII

GMII stands for Gigabit Media Independent Interface. IEEE 802.3 says GMII is designed to connect a 1 Gb/s capable MAC or repeater unit to a 1 Gb/s PHY, and that it is intended as a chip-to-chip interface. The 802.3 definitions also place the Reconciliation Sublayer and GMII in Clause 35.

So GMII is the natural parallel interface concept once you move from Fast Ethernet into Gigabit Ethernet. It keeps the same overall idea as MII but at the 1 Gb/s class.

### 2.5 RGMII

RGMII stands for Reduced Gigabit Media Independent Interface. TI’s automotive Ethernet article says the 1 Gb/s xMII family includes RGMII and SGMII, and that RGMII is a 12-pin interface. It also says RGMII still uses single-ended signaling and offers a 10x increase in data bandwidth for only a few additional signal lines compared with RMII.

So RGMII is the common reduced-pin-count parallel option for gigabit-class MAC–PHY attachment. In automotive systems, it is especially attractive when designers want a MAC or switch port that can talk to either a 100BASE-T1 or 1000BASE-T1 PHY with minimal platform redesign. TI explicitly says that connecting either PHY type to the same RGMII or SGMII port saves development time and lowers complexity.

### 2.6 SGMII

SGMII means Serial Gigabit Media Independent Interface. TI’s article says SGMII is part of the 1 Gb/s xMII family, that it can operate as a four- or six-pin interface, and that it uses Low Voltage Differential Signaling (LVDS). TI also says SGMII offers the benefit of 10x the data bandwidth with fewer signal lines, reducing solution size.

So SGMII is the serial, lower-pin-count alternative to more parallel gigabit MAC–PHY interfaces. In automotive systems, that often makes it attractive for backbone ports and for switch-to-PHY attachment where pin budget and printed-circuit-board routing matter. TI and NXP material both show automotive devices using SGMII for exactly that reason.

### 2.7 XGMII

XGMII is the 10 Gigabit Media Independent Interface. IEEE 802.3 says XGMII is designed to connect a 10 Gb/s capable MAC to a 10 Gb/s PHY, and the 802.3 section table places Reconciliation Sublayer and 10 Gigabit Media Independent Interface (XGMII) in Clause 46. IEEE 802.3 evolution material also describes XGMII as a high-complexity 32-bit wide bus.

So XGMII is conceptually the 10 Gb/s extension of the same MAC–PHY separation idea, but at a much higher data rate and complexity class. In mainstream automotive systems today, it is less common than MII/RMII/RGMII/SGMII, but it matters as link rates continue rising.

### 2.8 The clean comparison

The cleanest way to think about the xMII family is by speed class and pin-count philosophy:

- MII / RMII live in the 10/100 Mb/s world

- GMII / RGMII / SGMII live in the 1 Gb/s world

- XGMII lives in the 10 Gb/s world.

Within each speed class, the “reduced” or “serial” forms are usually about lowering pin count, routing difficulty, and implementation cost without changing the higher-level MAC semantics.

## 3. Automotive perspective and context

### 3.1 Why CSMA/CD mostly disappears in automotive Ethernet

Modern automotive Ethernet is overwhelmingly about point-to-point links, not classic shared coax or hub-style shared media. Full duplex expects point-to-point links and has no collisions, so CSMA/CD is not needed. The IEEE automotive tutorial frames automotive Ethernet around switched deterministic communication, not legacy shared-medium collision domains.

That means for automotive engineers, CSMA/CD is mainly important as conceptual history and as a contrast case. It explains why Ethernet once behaved the way it did, but it is not the central operating principle of 100BASE-T1 and 1000BASE-T1 vehicle links.

One useful contrast is 10BASE-T1S, which does bring back a shared-medium, half-duplex model. IEEE 802.3cg tutorial material says 10BASE-T1S requires the half-duplex Clause 4 MAC using CSMA/CD.

### 3.2 Why one-pair Ethernet became so important in vehicles

The automotive Ethernet tutorial ties the move to T1 Ethernet directly to harness cost, weight, and architecture convergence. It explicitly highlights 100BASE-T1 and 1000BASE-T1 as the standard communication links supporting the transition to Ethernet and “ubiquitous IP” in vehicles.

That means one-pair Ethernet is not merely a neat PHY optimization. It is one of the enabling technologies behind:

- lighter and simpler wiring harnesses

- more centralized or zonal vehicle networking

- fewer protocol gateways

- easier reuse of IP- and Ethernet-based software components.

### 3.3 100BASE-T1 and 1000BASE-T1 are full-duplex building blocks

This point is worth making very explicitly in an automotive course.

100BASE-T1 allows full-duplex bidirectional communication, so classic MII collision signals are not needed. IEEE 802.3bp material says 1000BASE-T1 operates using full-duplex communications using echo cancellation over a single pair.

Automotive Ethernet backbone story is fundamentally a full-duplex switched Ethernet story. That is why timing, queueing, VLANs, MACsec, and Time-Sensitive Networking become the important design concerns, not collision arbitration.

### 3.4 Why MAC–PHY interface choice matters so much in automotive hardware

In a vehicle, the MAC–PHY interface is not just a chip-design detail. It affects:

- processor or switch pin count

- printed-circuit-board routing complexity

- platform reuse across 100 Mb/s and 1 Gb/s variants

- signal integrity and diagnostics

- migration path from domain to backbone links.

A 100BASE-T1 PHY that supports RGMII or SGMII provides an easy migration path to a 1000BASE-T1 PHY, letting designers build reusable platforms and reduce system complexity.

That is why automotive hardware architects care deeply about xMII choices. The interface determines not only whether the PHY works, but whether the platform scales.

### 3.5 A practical automotive reading of the interface family

In a realistic automotive design:

- MII / RMII are natural for simpler 10/100 Mb/s endpoints and microcontrollers

- RGMII / SGMII are often the most relevant for T1 backbones and switch uplinks because they support an easier path between 100BASE-T1 and 1000BASE-T1 systems

- XGMII belongs more to higher-end multi-gig or 10-gig designs than to the mainstream current vehicle edge.

That is not a rule of the standards. It is the practical engineering consequence of pin count, bandwidth, and platform-reuse pressure in automotive systems.

## 4. Summary

CSMA/CD is the classic Ethernet access method for shared-medium, half-duplex operation: listen, contend, detect collisions, back off, retry. Full duplex removes the collision problem by using point-to-point simultaneous transmit and receive, so CSMA/CD is not needed. Modern automotive 100BASE-T1 and 1000BASE-T1 are full-duplex point-to-point PHYs on a single twisted pair, with 100BASE-T1 in Clause 96 and 1000BASE-T1 in Clause 97.

The xMII family is the MAC–PHY attachment family:
MII and RMII for 10/100 Mb/s,
GMII, RGMII, and SGMII for 1 Gb/s-class designs,
and XGMII for 10 Gb/s-class designs. In automotive systems, RGMII and SGMII are especially important because they make it easier to reuse one MAC or switch platform across 100BASE-T1 and 1000BASE-T1 designs.