# PAM-3: Pulse Amplitude Modulation with 3 Levels

PAM-3 is a line coding scheme that encodes data by using three distinct voltage levels to represent symbols on a transmission line. Let me walk through the fundamentals clearly.

## What is PAM-3?

In standard binary signaling (NRZ), each symbol carries one of two voltage levels, encoding exactly 1 bit per symbol. PAM-3 extends this by using three voltage levels — typically +V, 0, and −V — allowing each symbol to carry more information. Specifically, each PAM-3 symbol represents one of three states, encoding log₂(3) ≈ 1.585 bits per symbol.

PAM-3 is most famously used in 100BASE-T2 and 1000BASE-T Ethernet, where it helps achieve higher data rates over copper twisted-pair cabling without increasing the baud rate (symbol rate) proportionally.

Let us start with a diagram showing the three voltage levels and how a bit stream maps to them.

The waveform above shows eight consecutive PAM-3 symbols. At each clock edge, the transmitter drives the line to one of three discrete levels. The receiver samples at each symbol period and decides which of the three levels was sent.

## The three levels and their encoding

The mapping from symbols to bits isn't a simple 1:1 correspondence like binary. Since 3 states per symbol gives log₂(3) ≈ 1.585 bits, you can't encode an exact integer number of bits per symbol. There are two common strategies:

Strategy 1 — Ternary digit mapping (used in 100BASE-T2): Groups of bits are mapped onto groups of ternary symbols. For example, 4 bits (16 values) can be encoded into 3 PAM-3 symbols (3³ = 27 possible values, 16 of which are used). The remaining 11 unused codewords can be reserved for control signaling or chosen for favorable spectral properties (DC balance).

Strategy 2 — Direct ternary coding (simpler conceptual model): Each symbol is assigned a meaning: +V → "+1", 0V → "0", −V → "−1". This is useful for understanding the signal on the wire, even if the actual bit-to-symbol mapping uses a lookup table.

Now let us see how PAM-3 compares to binary NRZ in terms of bandwidth efficiency.

The key insight here: because each PAM-3 symbol carries ~1.585 bits instead of 1 bit, you need roughly 37% fewer symbol periods to transmit the same data. This means you can either run the symbol clock slower (saving bandwidth) or push more data at the same clock rate.

## Eye diagram and noise margins

The trade-off with PAM-3 is that the voltage spacing between levels is smaller than in binary. If the total voltage swing is the same (say ±1V), then binary has a distance of 2V between its two levels, while PAM-3 has only 1V between adjacent levels. This makes PAM-3 more susceptible to noise — the receiver has tighter decision thresholds.

The receiver uses two decision thresholds at +V/2 and −V/2. Any received signal above the upper threshold is decoded as +V, between the thresholds as 0, and below the lower threshold as −V. The noise margin — the amount of noise a signal can tolerate before being misinterpreted — is V/2 per level, compared to V for binary NRZ. This is the fundamental PAM-3 trade-off.

## Summary of key properties

Information density: Each PAM-3 symbol carries log₂(3) ≈ 1.585 bits, compared to 1 bit for binary. This means ~58.5% more data per symbol period.

Bandwidth efficiency: For a given data rate, PAM-3 requires a lower symbol rate than binary, which means the signal occupies less bandwidth on the wire. This is critical in Ethernet standards where cable bandwidth is constrained.

Noise sensitivity: The reduced spacing between levels (V instead of 2V for the same swing) means PAM-3 requires better signal-to-noise ratio (SNR) at the receiver. Roughly 3–4 dB more SNR is needed compared to binary for the same bit error rate.

DC balance: The three levels (including zero) make it easier to achieve DC balance on the line, which is important for transformer-coupled interfaces. Encoding schemes can choose codewords that keep the running digital sum near zero.

Practical usage: PAM-3 is used in 100BASE-T2 Ethernet (100 Mbps over two pairs of Cat 3 cable) and as a building block concept in 1000BASE-T (which uses PAM-5 but with related principles). It sits in the family between simple binary NRZ and higher-order schemes like PAM-4 (used in 400G Ethernet) and PAM-5.

Interactive Encoder PAM-3