## 1. Why spanning tree exists in a bridge network

A bridge or Layer 2 switch learns Media Access Control (MAC) addresses and forwards Ethernet frames. Redundant physical links are good for availability, but in a pure Layer 2 network they create loops. A loop is disastrous because the same frame can circulate indefinitely, broadcasts can multiply, and the forwarding database can become unstable as the same source MAC address is learned on different ports. Spanning Tree Protocol (STP) was created to prevent those loops while still keeping redundant links available as backups. Cisco’s overview summarizes this well: STP prevents loops by exchanging Bridge Protocol Data Units (BPDUs) and guarantees a single active path between Layer 2 devices; if a primary segment fails, STP recalculates and activates a standby path.

## 2. The core idea of spanning tree

The protocol takes a physical topology that may have cycles and turns it into a logical tree with no cycles. A tree has exactly one active path between any two points. To do that, spanning tree keeps some ports forwarding and places redundant ones into a non-forwarding state. Cisco’s STP guide describes the result as a topology with a root bridge and a loop-free path from that root to all bridges, with redundant data paths held in a blocked or standby state until needed.

## 3. How the basic STP algorithm works

The basic algorithm works in three big steps. First, all bridges exchange BPDUs and elect a root bridge. Second, every non-root bridge chooses its best path toward that root, based mainly on root path cost. Third, on each shared path segment, one port is chosen to forward toward or away from the segment, while redundant ports are blocked so that the overall active topology has no cycles. Cisco’s documentation describes root election, the root-centric tree, and the use of port priority and port path cost to decide which port forwards and which blocks.

## 4. Bridge ID and root election

A bridge becomes root by advertising the best Bridge ID in the received BPDUs. In practical terms, Bridge ID is built from a bridge priority field and a switch MAC-based identifier; lower wins. Cisco’s guide states that each VLAN on each device has a unique 64-bit Bridge ID and that changing bridge priority changes the probability of the switch being elected root. Once the root is chosen, every other forwarding decision is oriented around the best path to that root.

## 5. Port roles and states

In classic STP, ports pass through blocking, listening, learning, and forwarding states, with disabled as a separate administrative state. Blocking means no user-frame forwarding and no MAC learning; listening means the bridge is participating in STP but still not learning or forwarding; learning means it begins building the forwarding database; forwarding means both learning and data forwarding are active. Cisco’s STP guide describes these state behaviors in detail.

In Rapid Spanning Tree Protocol (RSTP), the operational model is cleaner. Cisco’s RSTP description says that in a stable topology, root ports and designated ports move to forwarding, while alternate and backup ports stay in discarding. The big advantage is speed: Cisco documents that RSTP can reconfigure in under a second in many cases, whereas default classic STP settings can take around 50 seconds.

## 6. Why this matters in VLAN bridge networking

Now bring Virtual Local Area Networks (VLANs) into the picture. VLANs create multiple logical Layer 2 domains on top of the same physical switch fabric and trunks. That means the network now has at least three topological notions:

- the physical topology

- the active forwarding topology

- the per-VLAN topology

The current IEEE 802.1Q bridge standard explicitly says it covers automatic configuration of bridged and VLAN topology, defines RSTP and Multiple Spanning Tree Protocol (MSTP), and includes the consistent allocation of VLAN IDs (VIDs) to spanning trees or other active topologies. In other words, spanning tree is not separate from VLAN-aware bridging; it is one of the mechanisms by which the bridge decides which links are active for which VLANs.

## 7. The simplest VLAN case: one tree for all VLANs

The simplest model is Single Spanning Tree (SST) or Common Spanning Tree (CST): one spanning-tree instance governs the whole bridged network, so all VLANs share the same blocked and forwarding links. IEEE material describes an SST bridge as one that supports only a single spanning tree, the CST. This is simple and interoperable, but it means every VLAN inherits the same active topology, even if different VLANs would benefit from different blocked links.

## 8. Why a single tree becomes limiting in VLAN networks

In a VLAN-rich network, one active tree for all VLANs wastes flexibility. Suppose you have two uplinks. Maybe VLAN 10 would best use uplink A while VLAN 20 would best use uplink B. With one spanning tree for all VLANs, both VLANs are forced to live with the same blocked port. That is the motivation for per-VLAN or multi-instance spanning tree protocols. Cisco and Juniper both explain that per-VLAN protocols let different VLANs use different active topologies, while MSTP lets multiple VLANs be grouped into shared instances for better scale.

# 9. The main spanning-tree protocol families used with VLAN bridging

## 9.1 STP — Spanning Tree Protocol — IEEE 802.1D

This is the original spanning-tree protocol. It is the historical baseline. It prevents loops and provides failover, but convergence is slow because of timer-driven state transitions through blocking, listening, and learning. IEEE 802.1 tutorials now explicitly note that classic STP is historical and has been replaced by RSTP. Cisco still describes STP as the original IEEE 802.1D bridge protocol.

### Where it fits in VLAN bridging

Classic STP can be used with VLAN-aware bridges, but if you use only one STP instance, all VLANs share one common tree. That is the simplest but least flexible design.

## 9.2 RSTP — Rapid Spanning Tree Protocol — IEEE 802.1w

RSTP is the fast-converging successor to STP. It keeps the same basic aim — elect a root and maintain a loop-free tree — but replaces much of the slow timer-based behavior with faster role/state handling and handshake mechanisms on point-to-point links. Cisco’s guide says RSTP takes advantage of point-to-point wiring and can converge in less than a second in many cases. IEEE 802.1Q revision material places RSTP among the spanning-tree protocols currently specified by the bridge standard.

### Where it fits in VLAN bridging

RSTP is the modern baseline for a single active tree. If all VLANs share one common active topology, RSTP is the usual standards-based choice rather than classic STP.

## 9.3 CST / SST — Common or Single Spanning Tree

These terms describe the topology model, not a completely separate algorithm. The idea is one spanning-tree instance for the whole bridged network or VLAN environment. IEEE material describes Single Spanning Tree (SST) bridges as supporting only a single Common Spanning Tree (CST). In practice, that single tree may be run using STP or RSTP.

### Practical meaning

All VLANs see the same blocked links and the same forwarding links.

### Good for

Small, simple, highly interoperable networks.

### Weakness

No per-VLAN load-sharing.

## 9.4 PVST+ — Per-VLAN Spanning Tree Plus — Cisco

Per-VLAN Spanning Tree Plus (PVST+) is a Cisco style in which each VLAN gets its own spanning-tree instance. Cisco documentation lists PVST and PVST+ among spanning-tree sub-protocols, and Cisco device guides describe Rapid PVST+ as the per-VLAN version of RSTP. Juniper’s VSTP documentation also states that its per-VLAN mode interoperates with PVST+ and Rapid-PVST+, which is further evidence of the “one tree per VLAN” model.

### Why people wanted it

Because VLAN 10 and VLAN 20 can now block different uplinks and therefore use different active forwarding trees.

### Cost

More BPDUs, more control-plane state, more CPU and memory.

## 9.5 Rapid PVST+ — Cisco per-VLAN RSTP

Rapid PVST+ takes the per-VLAN idea and uses RSTP behavior per VLAN. Cisco states that Rapid PVST+ creates one spanning-tree topology per VLAN and is an implementation of IEEE 802.1w on a per-VLAN basis. Cisco also says it interoperates with devices running legacy IEEE 802.1D STP.

### Practical meaning

Every VLAN has its own rapid-converging tree.

### Advantage

Fine-grained load-sharing plus fast reconvergence.

### Disadvantage

Scale. In a large VLAN environment, hundreds of per-VLAN trees can become expensive.

## 9.6 VSTP — VLAN Spanning Tree Protocol — Juniper

Juniper’s VSTP is also one spanning-tree instance per VLAN. Juniper says VSTP maintains a separate spanning-tree instance per VLAN, allows fine-grained load balancing, but increases BPDU and CPU processing as the number of VLANs increases. Juniper also notes that VSTP interoperates with PVST+ and Rapid-PVST+.

So from a conceptual point of view, PVST+ / Rapid PVST+ / VSTP all belong to the same broad family:

one spanning-tree instance per VLAN

with vendor-specific details.

## 9.7 MSTP — Multiple Spanning Tree Protocol — IEEE 802.1s, now within IEEE 802.1Q

This is the protocol that scales VLAN-aware spanning tree much better. Instead of “one tree for all VLANs” or “one tree per VLAN,” MSTP lets you map multiple VLANs to one spanning-tree instance. IEEE 802.1Q revision material says the bridge standard defines MSTP and the consistent allocation of VIDs to spanning trees. Cisco explains that MST is an IEEE standard and that VLANs are mapped to instances, with the topology determined by the instance, not by the individual VLAN.

### The key idea

You might decide:

- VLANs 10, 20, 30 → MST instance 1

- VLANs 40, 50 → MST instance 2

- all remaining VLANs → instance 0

Each instance gets one logical tree, and all VLANs mapped to that instance share that tree.

### Why it matters

It gives much of the traffic-engineering flexibility of per-VLAN spanning tree, but with far fewer protocol instances and BPDUs. Cisco and Juniper both position MSTP as the scalable alternative to per-VLAN models.

# 10. MSTP terminology that confuses almost everyone

## 10.1 MST region

An MST region is a group of bridges that all agree on the same MST configuration, especially the same VLAN-to-instance mapping. Cisco explains that the region identity is checked using region name, revision, and a digest derived from the VLAN-to-instance mapping table; if the digest differs, the port is at a region boundary.

That matters because MSTP only works sensibly if all bridges inside a region interpret “instance 1” and “instance 2” the same way.

## 10.2 IST — Internal Spanning Tree

Cisco says every MST bridge must support at least one Internal Spanning Tree (IST) and one or more Multiple Spanning Tree Instances (MSTIs). The IST is instance 0 and is always present across the region.

## 10.3 MSTI — Multiple Spanning Tree Instance

These are the user-facing additional instances that carry mapped VLANs inside the region. Cisco explains that MSTIs are RSTP-based instances that exist only inside the region; they do not send their own separate BPDUs outside the region.

## 10.4 CST and CIST

Outside an MST region, the region must still interoperate with the rest of the bridged world. Cisco explains that the IST extends the Common Spanning Tree (CST) through the region, making the region appear as one virtual bridge to the outside. IEEE 60802 material expresses the same idea by stating that each MST region computes an IST, regions are connected by a CST, and CST + IST together form the Common and Internal Spanning Tree (CIST).

So the clean picture is:

- CST: the outside-world common tree

- IST: the region’s internal common tree

- CIST: the end-to-end combination of the two

# 11. How VLAN mapping changes the active topology

This is the key operational difference between the protocol families.

### With CST / SST

All VLANs share one active topology. One blocked uplink is blocked for all VLANs.

### With PVST+ / Rapid PVST+ / VSTP

Each VLAN can have a different root bridge and different blocked uplinks. That enables per-VLAN load balancing. Juniper explicitly notes that one spanning tree per VLAN allows fine-grained load balancing.

### With MSTP

The active topology belongs to the instance, not the VLAN. Cisco warns that if VLAN 10 and VLAN 20 are mapped to the same MST instance, they share the same logical topology and therefore the same blocked ports; you do not get separate load balancing unless you map them to different instances.

That point is extremely important in switch-driver or bridge configuration work:

In MSTP, do not think “one VLAN, one tree.” Think “one instance, one tree; VLANs inherit the tree of their assigned instance.”

# 12. Why RSTP and MSTP are the standards-based answers today

The current IEEE 802.1Q bridge work explicitly places RSTP and MSTP in the standard bridge model. IEEE tutorials from 2012, 2013, and 2021 all note that classic STP is historical and has been replaced by RSTP. So if you are thinking in terms of modern standards-based VLAN-aware bridging, the real standards-based spanning-tree tools are:

- RSTP for one active tree

- MSTP for multiple active trees mapped to VLAN groups.

# 13. A very practical way to choose among them

### Use a single tree (RSTP/CST style) when:

- the network is small

- you want simplicity and interoperability

- per-VLAN traffic engineering is not important.

### Use per-VLAN spanning tree (PVST+, Rapid PVST+, VSTP) when:

- you want different VLANs to use different uplinks

- you can tolerate the control-plane scale cost

- you are in a vendor ecosystem that supports it well.

### Use MSTP when:

- you need VLAN-aware load sharing

- you have many VLANs

- you want to reduce instance count by grouping VLANs into a manageable number of trees.

# 14. One final caution

People often say “spanning tree for a VLAN” as if the VLAN itself runs a protocol. That is sloppy language. The protocol runs on the bridges. What actually varies is the association between VLAN IDs and active spanning-tree topologies. IEEE 802.1Q says bridges consistently assign frames for a given VLAN to a spanning-tree active topology, and Cisco’s MST explanation says clearly that the topology is determined by the instance, regardless of which VLANs are mapped to it.

## Bottom line

Spanning tree exists to keep a bridged Ethernet network loop-free while preserving redundant links for failover. In VLAN-aware bridging, the central design question is whether all VLANs share one tree, each VLAN gets its own tree, or multiple VLANs are grouped into shared instances. The protocol families line up exactly with those choices:

- STP: original, slow, historical

- RSTP: fast single-tree baseline

- CST / SST: one tree for all VLANs

- PVST+ / Rapid PVST+ / VSTP: one tree per VLAN

- MSTP: multiple trees, each serving a group of VLANs.

Think of a VLAN-aware bridge as combining three different pieces of state for every frame:

- VLAN state: “Is this frame allowed in VLAN 100 on this port?”

- Forwarding database (FDB) state: “For this destination MAC in VLAN 100, which port should I use?”

- Spanning-tree active-topology state: “Even if that port is a VLAN member and the FDB points there, is that port currently forwarding or blocked for the active topology of this VLAN or VLAN-instance?”

IEEE 802.1 explicitly separates physical network topology, active topology, station location, and VLAN topology. Spanning tree is what constrains the active topology out of the physical one.

## The packet walk inside a VLAN-aware bridge

A good switch-pipeline mental model is this:

Ingress port

↓

VLAN classification / admission

↓

Spanning-tree ingress state check

↓

Source MAC learning (if allowed)

↓

FDB lookup / flood decision

↓

Per-egress-port VLAN membership check

↓

Per-egress-port spanning-tree state check

↓

Egress queue / transmit

Now let us walk that carefully.

### 1. Ingress VLAN classification

When a frame arrives, the bridge first decides which VLAN it belongs to.

- If the frame is tagged, the VLAN Identifier (VID) comes from the tag.

- If it is untagged, the bridge can assign the ingress port’s default VLAN.

At this point the bridge asks:

“Is ingress port 5 allowed to receive frames for VLAN 100?”

If not, the frame is dropped before normal bridging continues. This is a pure VLAN admission question, not yet an FDB or spanning-tree question. IEEE 802.1Q bridging treats VLAN topology as a separate concept that constrains forwarding behavior.

### 2. Ingress spanning-tree state check

Next the bridge checks the spanning-tree state associated with that ingress port for the relevant active topology.

In classic STP terms, if the port is in blocking, it does not participate in frame forwarding, discards received data frames, discards switched frames that would be forwarded out it, and does not learn source addresses into the forwarding database. It still receives BPDUs and sends them to the control logic. Cisco states exactly that for a blocking Layer 2 interface.

So at ingress, the hardware logic is effectively:

if VLAN admission fails:

drop

if STP state for this VLAN/topology on ingress port is blocking/discarding:

drop data frame

deliver BPDU to control plane if it is a BPDU

This is very important: spanning-tree state can stop a frame before FDB learning and before forwarding.

### 3. Source MAC learning

If the ingress port is in a state that allows learning, the bridge learns the source MAC address location for that VLAN.

Cisco documents the classic behavior clearly:

- blocking: no address-database update

- learning: learns end-station location information into the address database, but still does not forward user frames

- forwarding: both learning and forwarding are enabled.

So the bridge learns something like:

{SA}, VID} ingress port

For example, if a source MAC 00-11-22-33-44-55 arrives on port 5 in VLAN 100 while the port is in learning or forwarding state, the bridge can install:

{00-11-22-33-44-55, VLAN 100} → port 5

That is why topology changes matter so much: learned MAC locations are tied to the active topology.

### 4. Destination lookup in the FDB

Now the bridge looks up the destination.

If the destination is known, it checks an FDB entry like:

{DA, VLAN 100} → port 7

If the destination is unknown unicast, broadcast, or multicast, the bridge creates a candidate flood set based on VLAN membership and bridge flooding rules.

This is the point where many people think “we are done.” But we are not. The FDB gives only a candidate egress port or port set. Spanning-tree state still has to prune that set.

### 5. Egress VLAN membership filtering

For each candidate egress port, the bridge asks:

“Is this egress port actually a member of VLAN 100?”

If not, remove it from the egress set.

So if the unknown-destination flood set was {2, 7, 8, 9} but only {7, 9} are members of VLAN 100, the bridge reduces the egress set to {7, 9}. This is VLAN topology filtering.

### 6. Egress spanning-tree state filtering

Now the bridge checks spanning-tree state on each remaining candidate egress port for the relevant active topology.

If port 9 is blocked for that VLAN or for the spanning-tree instance that VLAN maps to, port 9 is removed. Only forwarding/designated/root-path-allowed ports remain.

So the final transmit set may become just:

{7}

This is the key idea:

VLAN membership says where a frame may go.

The FDB says where it should go.

Spanning tree says which of those otherwise valid choices are currently active.

That is how a VLAN-aware bridge becomes loop-free without losing the logical segmentation of VLANs.

## A concrete example

Assume:

- ingress port = 5

- VLAN = 100

- source MAC = 00-11-22-33-44-55

- destination MAC = 66-77-88-99-AA-BB

Assume the bridge has:

- VLAN 100 members = {5, 7, 9}

- FDB entry: {66-77-88-99-AA-BB, 100} → 9

- STP state for VLAN/topology:

  - port 5 = forwarding

  - port 7 = forwarding

  - port 9 = blocking

Then the pipeline result is:

- ingress VLAN admission on port 5 for VLAN 100: pass

- ingress STP state on port 5: forwarding, so data is accepted

- learn {00-11-22-33-44-55, 100} → 5

- FDB says destination should go to port 9

- VLAN membership says port 9 is a valid VLAN-100 member

- spanning-tree egress state says port 9 is blocking

- therefore the frame is not transmitted on port 9

This is exactly how spanning tree prevents a loop even though VLAN and FDB state alone would have allowed the transmission.

## What changes with different spanning-tree families

The pipeline above stays the same. What changes is which active-topology state table the bridge consults.

### 1. Common or single spanning tree

With one common tree, all VLANs share the same blocked and forwarding links. So the spanning-tree check is effectively:

stp_state = common_tree_state[port]

That means VLAN 100 and VLAN 200 see the same blocked uplink. IEEE and Cisco material treat this as the simple common-tree model.

### 2. Per-VLAN spanning tree

With PVST+ / Rapid PVST+ / VSTP, the bridge consults spanning-tree state per VLAN:

stp_state = vlan_stp_state[VID][port]

So VLAN 100 may block port 9 while VLAN 200 blocks port 7. Cisco and Juniper describe these protocols as maintaining a separate tree per VLAN.

### 3. Multiple spanning tree

With MSTP, the bridge does one extra mapping step:

mst_instance = VID_to_MSTI[VID]

stp_state    = msti_state[mst_instance][port]

So VLAN 100 does not directly have its own tree. Instead it inherits the active topology of its assigned Multiple Spanning Tree Instance (MSTI). That is why in MSTP the important configuration is not just the VLAN itself, but the VLAN-to-instance map. IEEE 802.1Q material says the standard includes consistent allocation of VLAN IDs to spanning trees or other active topologies, and MSTP is one of the protocols defined there.

## Where topology changes hit the pipeline

When spanning-tree state changes, the bridge’s learned forwarding information may no longer be valid.

Cisco explicitly notes that:

- Rapid STP and MST can trigger a topology change when a port goes to forwarding or changes role

- Rapid STP immediately flushes the Layer 2 forwarding table

- classic 802.1D instead shortens the aging time rather than immediately flushing.

Pipeline-wise, that means:

- spanning-tree control plane changes the active topology

- the bridge invalidates or ages out affected FDB state

- subsequent frames may flood temporarily until new source learning repopulates the FDB on the new active topology

That is why STP changes often correlate with bursts of flooding.

## The clean mental model to keep

For a VLAN-aware bridge, each frame is processed by three filters in sequence:

1. VLAN filter:

Is this VLAN valid on this port?

2. Forwarding filter:

What does the FDB say about the destination in this VLAN?

3. Active-topology filter:

Is that ingress/egress port actually active for this VLAN’s spanning-tree topology?

Or even shorter:

Forwarding decision = VLAN rules ∩ FDB result ∩ spanning-tree active topology

That is the switch-pipeline way to understand how spanning tree, VLAN membership, and the forwarding database work together.