# Detailed Training Material

## IPv4 / IPv6 basics; TCP / UDP; ICMP; SOME/IP service model and service discovery; STP / RSTP / MSTP

### Fundamentals first, then the automotive perspective

## How to use this material

This training pack is written for someone who does not want hidden assumptions. It starts from first principles and builds upward. The idea is to answer four questions for every protocol:

- What problem does it solve?

- Where does it sit in the stack?

- What does the header or control message look like?

- What changes when you see it in an automotive Ethernet network?

This is organized in two halves:

- Part I: Data communications context first

- Part II: Automotive interpretation and engineering context

# Part I - Data communications context first

## 1. The big picture: stop mixing the layers

A lot of confusion in networking comes from treating all protocol names as if they were doing the same job.

They are not.

Think in layers:

- Ethernet moves frames on a local link.

- Internet Protocol (IP) gives logical addressing and packet forwarding across networks.

- Transmission Control Protocol (TCP) and User Datagram Protocol (UDP) provide end-to-end transport behavior.

- Internet Control Message Protocol (ICMP) carries control and error-reporting information for the network layer.

- Scalable service-Oriented MiddlewarE over IP (SOME/IP) gives application-level meaning such as methods, events, and service instances.

- Spanning Tree Protocol (STP), Rapid Spanning Tree Protocol (RSTP), and Multiple Spanning Tree Protocol (MSTP) are not transport protocols at all; they are Layer-2 bridge-control mechanisms used to prevent loops.

### Mental picture

When one ECU sends an application message, the stack is not “one protocol instead of another.” It is more like nested envelopes:

Application data / middleware payload
    inside SOME/IP message
        inside UDP or TCP segment
            inside IPv4 or IPv6 packet
                inside Ethernet frame
                    on a copper or single-pair link

That layering is the first thing to keep straight.

## 2. Internet Protocol: what IP actually does

Internet Protocol (IP) is the network-layer protocol that gives a packet:

- a source address,

- a destination address,

- a way for intermediate devices to forward it,

- and a small amount of control information.

IP is deliberately best effort.

That means IP by itself does not guarantee:

- delivery,

- order,

- duplicate suppression,

- or recovery from loss.

That is not a flaw. That is the design.

IP’s job is: move packets as well as possible through an interconnected network.

### Why this matters

If IP does not promise reliability, then a higher layer must decide whether reliability is needed.

That is why TCP exists.

If the network needs a way to report errors or diagnostics, a companion mechanism is needed.

That is why ICMP exists.

## 3. Internet Protocol version 4 (IPv4)

### 3.1 What problem IPv4 solves

Internet Protocol version 4 (IPv4) gives every participating node a 32-bit address and defines the structure of the packet used to move data between those addresses.

An IPv4 address is written in dotted decimal, for example:

192.168.10.24

That is just a human-friendly way to display 32 bits.

### 3.2 What an IPv4 address means

An IPv4 address is not just a label. It has forwarding meaning.

A host uses:

- the address, and

- the subnet mask or prefix length

…to determine whether the destination is:

- local (same subnet), or

- remote (must be sent to a router / default gateway).

Example:

Host A: 192.168.10.24/24
Destination: 192.168.10.80   -> local
Destination: 192.168.20.10   -> remote, send to gateway

### 3.3 Local delivery versus routed delivery

If the destination is on the same local network, the sender still cannot transmit using the IP address alone. Ethernet needs a local Media Access Control (MAC) address.

So IPv4 on Ethernet commonly uses Address Resolution Protocol (ARP) to learn:

Which MAC address corresponds to this IPv4 address?

If the destination is remote, the sender does not ARP for the remote host. It ARPs for the gateway, then sends the packet to the gateway for onward forwarding.

## 4. IPv4 header format

### 4.1 Why the header matters

The IPv4 header tells you how the packet should be interpreted and forwarded.

It is variable in size because options can exist.

### 4.2 IPv4 header diagram

0                   1                   2                   3
0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|Version|  IHL  |   DSCP/ECN   |         Total Length          |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|         Identification        |Flags|    Fragment Offset     |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|   Time to Live |   Protocol   |       Header Checksum        |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|                       Source Address                          |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|                    Destination Address                        |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|                    Options + Padding (optional)               |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

### 4.3 IPv4 header fields explained clearly

#### Version

This tells the receiver that the packet is IPv4.

#### IHL - Internet Header Length

This tells how long the IPv4 header is.

Why needed? Because IPv4 options make the header length variable.

- Minimum header: 20 bytes

- Maximum header: 60 bytes

#### DSCP / ECN

Originally this area was called Type of Service. Today it is used for:

- Differentiated Services Code Point (DSCP) for traffic treatment,

- Explicit Congestion Notification (ECN) for congestion signaling.

#### Total Length

Total size of the entire IPv4 packet:

- header

- plus payload

#### Identification, Flags, Fragment Offset

These support fragmentation.

If a packet is too large for some link, it may need fragmentation.

Important practical point: Modern network design tries to avoid fragmentation because it complicates performance and troubleshooting.

#### Time to Live (TTL)

Despite the name, in modern networks this behaves as a hop counter.

Every router decrements it. If it reaches zero, the packet is dropped.

This prevents packets from circulating forever in loops.

#### Protocol

This tells IPv4 what sits above it.

Examples:

- ICMP

- TCP

- UDP

This field is how the receiver knows which higher-layer logic should process the payload.

#### Header Checksum

This protects the IPv4 header only, not the payload.

#### Source Address / Destination Address

The sender and intended receiver logical addresses.

#### Options + Padding

Rare in many everyday systems, but they explain why the IPv4 header is not fixed in size.

## 5. Internet Protocol version 6 (IPv6)

### 5.1 Why IPv6 exists

Internet Protocol version 6 (IPv6) solves multiple issues, not just address-space exhaustion.

Yes, the bigger address space matters.

But IPv6 also aims for:

- cleaner header design,

- less per-hop processing,

- better extensibility,

- improved multicast-based control behavior,

- and removal of some older IPv4 complications.

### 5.2 IPv6 addressing

IPv6 addresses are 128 bits long.

They are written in hexadecimal groups, for example:

2001:0db8:0000:0000:0000:0000:0000:0042

This can often be compressed:

2001:db8::42

### 5.3 Important conceptual difference from IPv4

Do not think:

IPv6 = IPv4 with longer addresses.

That is too shallow.

Important operational differences include:

- fixed 40-byte base header,

- no IPv4-style header checksum,

- no broadcast in the IPv4 sense,

- much heavier use of multicast,

- Internet Control Message Protocol version 6 (ICMPv6) as a central control-plane mechanism,

- optional information moved into extension headers.

## 6. IPv6 base header format

0                   1                   2                   3
0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|Version| Traffic Class |           Flow Label                  |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|         Payload Length        |  Next Header  |   Hop Limit   |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|                                                               |
+                                                               +
|                                                               |
+                       Source Address                          +
|                                                               |
+                                                               +
|                                                               |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|                                                               |
+                                                               +
|                                                               |
+                    Destination Address                        +
|                                                               |
+                                                               +
|                                                               |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

### 6.1 IPv6 header fields explained

#### Version

Identifies IPv6.

#### Traffic Class

Comparable in spirit to IPv4’s service-related field.

#### Flow Label

Intended to help identify packets belonging to the same flow for special treatment.

#### Payload Length

Length of everything after the IPv6 base header.

#### Next Header

Tells what comes next:

- a transport protocol such as TCP or UDP,

- or an IPv6 extension header.

#### Hop Limit

This is the IPv6 equivalent of IPv4 TTL.

#### Source Address / Destination Address

Each is 128 bits.

### 6.2 Why IPv6 removed the header checksum

Routers already inspect and modify some header fields such as Hop Limit.

If a header checksum existed, every router would need to recalculate it.

IPv6 removes that cost and assumes other layers already provide integrity checks where needed.

### 6.3 Neighbor Discovery and why ICMPv6 is central

IPv4 uses Address Resolution Protocol (ARP) for local address resolution.

IPv6 instead relies on Neighbor Discovery, which uses ICMPv6 messages.

That means ICMPv6 is not just “diagnostic traffic.” It is part of normal IPv6 operation.

## 7. IPv4 versus IPv6: what an engineer should actually compare

### IPv4

- 32-bit addresses

- variable header length

- header checksum exists

- Address Resolution Protocol (ARP) used for local resolution

- broadcast exists

### IPv6

- 128-bit addresses

- fixed 40-byte base header

- no base-header checksum

- ICMPv6-based Neighbor Discovery

- heavy multicast use

- cleaner extensibility through extension headers

### The real takeaway

IPv6 is usually easier to parse in a standards sense, but operationally it demands that you understand ICMPv6 and multicast behavior properly.

## 8. Transport layer: why TCP and UDP both exist

IP gets a packet to the destination host.

But once the packet reaches that host, another question appears:

Which application or conversation should receive this payload?

That is where the transport layer comes in.

Both TCP and UDP add port numbers, but they offer very different delivery contracts.

## 9. Transmission Control Protocol (TCP)

### 9.1 What TCP solves

Transmission Control Protocol (TCP) is used when an application wants:

- reliable delivery,

- ordered delivery,

- duplicate suppression,

- flow control,

- and a stateful connection.

TCP is connection-oriented and presents a byte stream to the application.

### 9.2 Byte stream is a very important phrase

A byte stream means TCP does not preserve application message boundaries.

If an application sends:

- 100 bytes,

- then 50 bytes,

…the receiver might receive those as:

- one 150-byte read,

- or several smaller reads,

…but the order of bytes is preserved.

That is fundamentally different from UDP.

### 9.3 Core TCP ideas

#### Sequence numbers

Each byte in the stream occupies sequence space.

#### Acknowledgments

The receiver acknowledges what data has arrived.

#### Retransmission

If data appears lost, TCP retransmits it.

#### Flow control

The receiver advertises how much data it can accept.

#### Congestion behavior

TCP also adapts to network conditions rather than blindly blasting data forever.

## 10. TCP header format

0                   1                   2                   3
0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|          Source Port          |       Destination Port        |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|                        Sequence Number                        |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|                    Acknowledgment Number                      |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|Data Off|Res| Flags           |            Window Size         |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|           Checksum            |         Urgent Pointer        |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|                    Options + Padding (optional)               |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

### 10.1 TCP header fields explained

#### Source Port / Destination Port

Identify the communicating application endpoints.

#### Sequence Number

Tells where this segment’s payload belongs in the stream.

#### Acknowledgment Number

Tells the peer what data has been received successfully so far.

#### Data Offset

Tells where the payload begins, because TCP options make the header variable in size.

#### Flags

The control bits coordinate connection behavior.

Common ones to know:

- SYN - synchronize / start connection setup

- ACK - acknowledgment field is meaningful

- FIN - sender finished sending

- RST - reset / abort connection

- PSH - push data promptly upward

- URG - urgent pointer meaningful

Modern TCP also includes additional flags such as ECE and CWR related to congestion notification.

#### Window Size

Used for flow control.

#### Checksum

Protects TCP header and payload.

#### Urgent Pointer

Only meaningful when URG is set.

#### Options + Padding

Very important in practice. Examples include:

- Maximum Segment Size (MSS)

- window scaling

- timestamps

- selective acknowledgment (SACK)

### 10.2 TCP connection setup: the three-way handshake

The famous handshake is not just tradition. It establishes shared state.

Client -> Server : SYN
Server -> Client : SYN + ACK
Client -> Server : ACK

This synchronizes sequence numbers and confirms both directions are alive.

### 10.3 Why TCP can be the wrong tool

TCP is powerful, but that power has cost:

- connection state,

- handshake overhead,

- retransmission behavior,

- head-of-line blocking,

- potentially higher latency under loss.

That is why UDP still exists and is still very important.

## 11. User Datagram Protocol (UDP)

### 11.1 What UDP solves

User Datagram Protocol (UDP) gives a very small transport wrapper for message-based delivery.

UDP provides:

- source port,

- destination port,

- length,

- checksum.

That is basically it.

No handshake. No retransmission. No in-order guarantee. No connection state in the TCP sense.

### 11.2 Why that simplicity is useful

Sometimes reliability at the transport level is not what you want.

Examples:

- discovery protocols,

- multicast announcements,

- periodic sensor/event updates,

- time synchronization,

- applications where the newest sample matters more than old retransmitted data.

UDP preserves message boundaries.

If one UDP datagram is sent, one datagram is received - unless it is lost.

That is very different from a TCP byte stream.

## 12. UDP header format

0                   1                   2                   3
0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|          Source Port          |       Destination Port        |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|            Length             |           Checksum            |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

### 12.1 UDP header fields explained

#### Source Port / Destination Port

Identify sender and receiver application endpoints.

#### Length

Length of the UDP header plus payload.

#### Checksum

Integrity protection.

### 12.2 TCP versus UDP in plain language

#### TCP says:

I will manage the conversation and work hard to deliver a correct ordered stream.

#### UDP says:

I will carry your discrete message with minimal ceremony. If you need more behavior, you implement it above me.

## 13. Internet Control Message Protocol (ICMP)

### 13.1 What ICMP actually is

Internet Control Message Protocol (ICMP) is how the network layer reports:

- errors,

- reachability issues,

- diagnostic results,

- and certain control conditions.

It is not “user data transport.”

It is the network layer talking back.

### 13.2 Why ICMP exists

If IP is best effort, then when something goes wrong there must be some way to say things like:

- destination unreachable,

- time exceeded,

- packet too big / fragmentation problem,

- echo request / echo reply.

That is ICMP’s job.

### 13.3 ICMP is not just ping

Ping uses ICMP echo.

But ICMP also matters for:

- traceroute behavior,

- path MTU discovery,

- general network debugging,

- and in IPv6, core control-plane behavior.

## 14. ICMP general format

0                   1                   2                   3
0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|     Type      |     Code      |           Checksum            |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|                    Message-specific data                      |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|                    Additional payload / data                  |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

### 14.1 Type and Code

These tell the receiver which ICMP message this is.

Examples:

- Echo Request

- Echo Reply

- Destination Unreachable

- Time Exceeded

### 14.2 Message-specific data

This area depends on the message type.

### 14.3 Additional payload

Often includes part of the original packet that triggered the ICMP message.

That is extremely useful for debugging because it helps identify exactly which transmission caused the error.

## 15. ICMPv4 and ICMPv6

### 15.1 ICMPv4

Internet Control Message Protocol version 4 (ICMPv4) supports classic IPv4 control and diagnostic behavior.

### 15.2 ICMPv6

Internet Control Message Protocol version 6 (ICMPv6) is much more central to IPv6 operation.

ICMPv6 supports not just diagnostics, but also:

- Neighbor Discovery,

- Router Solicitation,

- Router Advertisement,

- Neighbor Solicitation,

- Neighbor Advertisement.

That means if ICMPv6 is broken, IPv6 may be broken even when simple traffic seems partly alive.

## 16. SOME/IP: why application middleware appears above IP

### 16.1 The problem SOME/IP solves

Once you have IP connectivity, you still need application-level meaning.

An ECU often does not want to think in terms of:

- raw UDP port 12345,

- raw TCP bytes,

- or fixed signal offsets only.

It wants to think in terms of:

- services,

- methods,

- events,

- fields,

- and service instances.

That is where Scalable service-Oriented MiddlewarE over IP (SOME/IP) comes in.

### 16.2 What SOME/IP gives you conceptually

SOME/IP supports a service-oriented communication model.

Instead of saying:

send these bytes to this hard-coded endpoint forever,

…you say something more like:

this service instance offers these methods and these events.

That is a very different abstraction, and it fits complex software systems much better.

### 16.3 Core SOME/IP concepts

#### Service

The logical capability.

#### Service instance

One concrete provider of that service.

#### Method

A callable operation.

#### Event

Asynchronous published data.

#### Eventgroup

A subscription grouping for events.

#### Field

A value-like interface element that can often be read, set, or notified.

## 17. SOME/IP base header format

0                   1                   2                   3
0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|                           Message ID                           |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|                            Length                              |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
|                           Request ID                           |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+
| Protocol Ver. | Interface Ver.| Message Type  |  Return Code  |
+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

### 17.1 SOME/IP header fields explained

#### Message ID

Identifies the service and method/event.

A common mental split is:

- Service ID

- Method ID or Event ID

#### Length

Length of the SOME/IP message starting from Request ID onward.

#### Request ID

Used to correlate a request and the corresponding response.

Often understood as containing:

- Client ID

- Session ID

#### Protocol Version

Protocol-level version information.

#### Interface Version

Major version of the service interface contract.

This is very important for compatibility.

#### Message Type

Tells what kind of SOME/IP message this is.

Examples include:

- request,

- response,

- notification,

- error.

#### Return Code

Indicates success or failure semantics where appropriate.

### 17.2 Why this header matters so much

TCP and UDP know ports. IP knows addresses.

But they do not know:

- service identity,

- method identity,

- event identity,

- interface contract version.

That semantic meaning exists at the SOME/IP layer.

## 18. SOME/IP Service Discovery: why the service model needs a control plane

### 18.1 Why discovery is needed

A service-oriented architecture becomes messy if every client must be manually configured with:

- exact IP address,

- exact port,

- exact lifetime assumptions,

- exact provider presence state.

That is why SOME/IP Service Discovery exists.

Service discovery answers questions like:

- Which services are currently available?

- Which service instance is being offered?

- Which version is being offered?

- Which endpoint should I use?

- Where should event traffic be sent?

- How long is this offer or subscription valid?

### 18.2 The two big pieces inside SD

#### Entries

These are the main declarations.

There are two broad categories:

- Service entries

- Eventgroup entries

#### Options

These carry operational details such as:

- IP address,

- port number,

- transport protocol,

- multicast information,

- configuration information.

That split is important.

A very useful sentence is:

The entry tells you what is being discussed. The option tells you where and how to reach it.

## 19. SOME/IP-SD packet organization

A high-level view looks like this:

+-------------------------------+
| SOME/IP Header                |
+-------------------------------+
| SD Flags / Reserved           |
+-------------------------------+
| Entries Array Length          |
+-------------------------------+
| Entries Array                 |
|   - Service entries           |
|   - Eventgroup entries        |
+-------------------------------+
| Options Array Length          |
+-------------------------------+
| Options Array                 |
|   - Endpoint options          |
|   - Multicast options         |
|   - Configuration options     |
+-------------------------------+

### 19.1 Service entries

Typical service-entry actions include:

- FindService

- OfferService

- StopOfferService

### 19.2 Eventgroup entries

Typical eventgroup actions include:

- SubscribeEventgroup

- StopSubscribeEventgroup

- SubscribeEventgroupAck

- negative acknowledgment behavior when subscription is rejected

### 19.3 Time to Live (TTL)

TTL is extremely important in SD.

It tells the receiver for how long an offer or subscription remains valid.

That means discovery is not just about finding a service once. It is also about lifetime management.

## 20. SOME/IP-SD message flow explained properly

A simple mental model:

Server ECU -> network : OfferService
Client ECU -> network : FindService (optional if passive listening is enough)
Client ECU -> server  : SubscribeEventgroup (if events are needed)
Server ECU -> client  : SubscribeEventgroupAck
Server ECU -> client  : event notifications / event traffic

### 20.1 OfferService

The server announces that a service instance exists.

### 20.2 FindService

The client actively searches when passive waiting is not enough.

### 20.3 SubscribeEventgroup

The client asks to receive a defined group of events.

### 20.4 SubscribeEventgroupAck

The server accepts the subscription and event delivery can begin.

### 20.5 Why endpoint options are essential

At some point the client must know:

- IP address,

- port,

- transport protocol,

…for actual communication.

That information is carried by options.

Without options, “service exists” is not yet usable.

## 21. Spanning Tree Protocol: why Ethernet loops are catastrophic

### 21.1 Why loops are bad in Layer 2 networks

Suppose you connect switches with redundant links for resilience.

That sounds good.

But if you leave all links forwarding in a naïve bridged topology, Ethernet has a problem:

- broadcasts can circulate forever,

- unknown unicasts can circulate,

- source MAC learning becomes unstable,

- traffic can multiply explosively.

This is often called a broadcast storm or switching loop problem.

### 21.2 Why IP does not fix that

This happens below IP.

TTL / Hop Limit protects routed packets at Layer 3.

But classic Ethernet forwarding of Layer-2 frames can still loop destructively if the bridge topology is wrong.

That is why a bridge-control protocol is needed.

## 22. Spanning Tree Protocol (STP)

### 22.1 What STP does

Spanning Tree Protocol (STP) computes a single loop-free logical topology from a physically redundant bridged topology.

In simple words:

- keep redundancy available,

- but do not let all redundant links forward simultaneously if that would create a loop.

### 22.2 The three central STP ideas

#### Root bridge

One bridge is elected as the reference point.

#### Root port

On each non-root bridge, the best path toward the root.

#### Designated port

The forwarding port chosen for a segment.

Ports that would create a loop are blocked from normal forwarding.

### 22.3 How STP communicates

Switches exchange Bridge Protocol Data Units (BPDUs).

These carry enough information to decide:

- who the root bridge is,

- what the best path cost is,

- and which ports should forward or block.

## 23. STP / RSTP BPDU format (simplified view)

+-------------------------------+
| Protocol ID                   |
+-------------------------------+
| Version                       |
+-------------------------------+
| BPDU Type                     |
+-------------------------------+
| Flags                         |
+-------------------------------+
| Root Bridge ID                |
+-------------------------------+
| Root Path Cost                |
+-------------------------------+
| Bridge ID                     |
+-------------------------------+
| Port ID                       |
+-------------------------------+
| Message Age                   |
+-------------------------------+
| Max Age                       |
+-------------------------------+
| Hello Time                    |
+-------------------------------+
| Forward Delay                 |
+-------------------------------+
| Extensions / version-length   |
+-------------------------------+

### 23.1 BPDU fields explained in practical terms

#### Root Bridge ID

Who is believed to be the root.

#### Root Path Cost

How expensive the path to that root is.

#### Bridge ID

Who sent this BPDU.

#### Port ID

Which sending port originated it.

#### Message Age / Max Age / Hello Time / Forward Delay

These govern convergence timing and stability.

## 24. Classic STP behavior and why it feels slow

Classic STP is careful.

Ports typically move through states such as:

- blocking,

- listening,

- learning,

- forwarding.

This prevents temporary loops during changes, but it can make reconvergence slow.

That is why classic STP is often associated with relatively long recovery times.

## 25. Rapid Spanning Tree Protocol (RSTP)

### 25.1 What RSTP improves

Rapid Spanning Tree Protocol (RSTP) keeps the same overall goal as STP:

- one loop-free active Layer-2 topology.

But it improves how convergence happens.

Instead of depending so heavily on slow timer expiration, RSTP uses faster synchronization behavior and more intelligent role handling.

### 25.2 Practical intuition

Classic STP often feels like:

Wait carefully before trusting the new topology.

RSTP feels more like:

Exchange enough information quickly to move to the correct topology faster.

### 25.3 Important effect

In practice, RSTP usually converges much faster than old-style STP, especially on well-understood point-to-point switched links.

## 26. Multiple Spanning Tree Protocol (MSTP)

### 26.1 What problem MSTP solves

Suppose you have many Virtual Local Area Networks (VLANs).

Using one single spanning tree for everything can waste bandwidth and create poor path usage.

Multiple Spanning Tree Protocol (MSTP) allows:

- multiple VLANs to be mapped to one spanning-tree instance,

- and different VLAN groups to use different loop-free active paths.

### 26.2 The big idea

MSTP is not “more loops.”

It is:

- multiple carefully defined spanning-tree instances,

- each still loop-free for its assigned VLAN set.

### 26.3 Why this matters

It improves scale and can allow load distribution across redundant topologies while still controlling loops.

## 27. STP versus RSTP versus MSTP in one clean comparison

### STP

- single spanning tree

- classic state machine

- slower convergence

### RSTP

- still essentially one active tree model

- faster convergence behavior

- improved role/state handling

### MSTP

- multiple spanning-tree instances

- VLAN-to-instance mapping

- better scaling and traffic engineering for VLAN-rich environments

# Part II - Automotive perspective

## 28. Why Ethernet and IP became important in vehicles

Automotive networks did not start with Ethernet.

They were historically dominated by bus systems such as:

- Controller Area Network (CAN),

- Local Interconnect Network (LIN),

- FlexRay.

Those technologies still matter.

But modern vehicles increasingly need:

- high-bandwidth sensor movement,

- camera traffic,

- central compute integration,

- over-the-air software update capability,

- service-oriented software architectures,

- richer diagnostics,

- easier aggregation through switches and gateways.

Ethernet and IP fit that direction well because they bring:

- standard layering,

- standard tooling,

- switching,

- routability,

- and application-layer extensibility.

### Important caution

Automotive Ethernet is not just enterprise Ethernet dropped into a car unchanged.

The protocols may be familiar, but the engineering constraints are different:

- startup timing,

- determinism,

- fault containment,

- safety,

- validation discipline,

- topology control.

## 29. IPv4 and IPv6 in automotive context

### 29.1 IPv4 in vehicles

IPv4 is often attractive because:

- tools are mature,

- private addressing is straightforward,

- debugging culture is strong,

- many stacks and workflows already support it well.

For bring-up, diagnostics, and internal communication, IPv4 is very practical.

### 29.2 IPv6 in vehicles

IPv6 matters because:

- it scales cleanly,

- it supports modern address planning,

- it avoids some IPv4-era baggage,

- it aligns well with multicast-based control models,

- and standards-based automotive software stacks increasingly support it.

### 29.3 The right question in automotive programs

The useful question is usually not:

Which protocol is universally better?

The useful question is:

What does this platform need, and what does the stack support robustly?

That may lead to:

- IPv4-only,

- IPv6-only,

- or dual-stack operation.

## 30. TCP and UDP in automotive context

### 30.1 Why both matter in a car

The same trade-offs exist in automotive systems as anywhere else, but they become very concrete.

#### TCP is attractive when:

- reliable ordered delivery matters,

- a stateful session is useful,

- application semantics depend on complete correct transfer.

#### UDP is attractive when:

- message freshness matters more than retransmitted history,

- multicast is useful,

- discovery is required,

- lightweight periodic/event communication is preferred.

### 30.2 A very practical engineering test

Ask:

If one message is lost, is it better to retransmit the old one, or is it better to move on and use the newest one?

If the newest sample is what matters, UDP often fits better.

If correctness of ordered delivery matters more, TCP often fits better.

## 31. ICMP and ICMPv6 in automotive debugging

ICMP becomes very useful during:

- ECU bring-up,

- switch and gateway validation,

- path troubleshooting,

- MTU troubleshooting,

- IPv6 neighbor-discovery debugging.

### Practical examples

#### Ping

Useful to establish basic IP reachability before blaming the application.

#### Time Exceeded / path behavior

Useful when packets are taking unexpected routes or loops exist.

#### ICMPv6 Neighbor Discovery

Absolutely central when IPv6 addresses exist but communication is unstable or incomplete.

### Important lesson

If you ignore ICMP entirely, you lose one of the cleanest windows into what the network layer is trying to tell you.

## 32. SOME/IP in automotive software architecture

### 32.1 Why SOME/IP fits automotive systems so well

Modern vehicles are increasingly software-defined systems.

That means software modules want cleaner interaction models than:

- fixed signal tables only,

- static endpoint assumptions forever,

- or manually coordinated byte layouts everywhere.

SOME/IP supports:

- service-oriented methods,

- event publication,

- field-like interactions,

- and version-aware interfaces.

That matches large modular automotive software systems much better.

### 32.2 Why service discovery matters especially in vehicles

A vehicle platform may evolve over time.

A logical service can move between compute nodes or be split differently across ECUs in later platform generations.

If clients bind only to raw static addresses, that evolution is painful.

If clients bind to services and discover providers dynamically, the software architecture becomes much more flexible.

### 32.3 Methods, events, and eventgroups in automotive use

A simple way to think about them:

- Method = call-and-response style operation

- Event = producer pushes information asynchronously

- Eventgroup = subscription bundle used to manage event delivery efficiently

That makes SOME/IP good for both:

- command-like interactions,

- and state/update publication.

## 33. SOME/IP-SD in automotive engineering terms

### 33.1 What the control plane is doing

A packet trace may show the transport is alive but the application still appears dead.

That can happen because the service control plane failed, even though IP connectivity exists.

Examples:

- OfferService is missing

- interface version mismatch exists

- endpoint options point to the wrong place

- subscription lifetime expired

- client subscribed incorrectly

### 33.2 A crucial diagnostic mindset

If you can ping an ECU but cannot consume its SOME/IP service, you have not proven the service path is healthy.

You have only proven that basic IP reachability exists.

That is a huge difference.

## 34. STP / RSTP / MSTP in automotive Ethernet

### 34.1 Why the automotive interpretation is more nuanced

In enterprise networks, spanning tree is a routine answer to broad Layer-2 redundancy.

In a vehicle, the topology is usually:

- smaller,

- more engineered,

- more tightly controlled,

- and more constrained by startup and validation needs.

So production in-vehicle networks are often designed to avoid casual Layer-2 loops from the start.

### 34.2 Then why should an automotive engineer still care?

Because loops are still dangerous anywhere switched bridging exists.

You may see STP-related thinking in:

- switch feature understanding,

- lab and bench setups,

- redundant bridged service networks,

- fault-injection or failure analysis,

- integration environments where links are added or changed.

### 34.3 What remains true

A Layer-2 loop inside a vehicle is still a Layer-2 loop.

It can still cause:

- broadcast storms,

- MAC-table instability,

- confusing intermittent failures,

- total loss of useful switching behavior.

### 34.4 What changes in practice

In automotive systems, spanning-tree knowledge is often more about:

- understanding switch behavior,

- validating topology assumptions,

- and debugging abnormal network states,

…rather than being the universal default architecture for every in-vehicle network.

## 35. Putting the whole automotive stack together

Imagine this sequence:

- Two ECUs are connected through one or more Ethernet switches.

- The physical link comes up.

- Ethernet frames can move locally.

- IP gives each node logical addressing.

- UDP carries SOME/IP Service Discovery traffic.

- The client learns that a service instance exists.

- The client subscribes to an eventgroup.

- The server starts publishing SOME/IP events.

- If the switched topology contains redundant paths, Layer-2 loop-control logic must ensure only a safe forwarding topology is active.

This is the systems view:

- Ethernet provides the local frame transport.

- IP provides logical addressing.

- TCP / UDP / ICMP provide transport and control behavior.

- SOME/IP provides service semantics.

- SOME/IP-SD provides service control-plane discovery.

- STP / RSTP / MSTP protect bridged Layer-2 forwarding from loops.

Each solves a different problem.

## 36. Troubleshooting by symptom

### Symptom: I can ping the ECU, but the service still does not work

Likely focus:

- SOME/IP header semantics

- interface version

- service discovery state

- subscription state

- endpoint options

### Symptom: Discovery works, but event traffic never arrives

Likely focus:

- SubscribeEventgroup flow

- SubscribeEventgroupAck reception

- TTL / subscription lifetime

- endpoint address / port / transport mismatch

### Symptom: Small messages work, big ones fail

Likely focus:

- path MTU,

- fragmentation expectations,

- transport behavior,

- SOME/IP transport choices.

### Symptom: Network collapses when a redundant cable is connected

Likely focus:

- Layer-2 loop behavior,

- STP / RSTP / MSTP,

- switch forwarding state.

### Symptom: IPv6 looks configured but behaves strangely

Likely focus:

- ICMPv6,

- Neighbor Discovery,

- router advertisements,

- multicast handling.

## 37. Misconceptions to remove early

### Wrong idea 1

IPv6 is just IPv4 with longer addresses.

No. The control behavior and packet model differ in important ways.

### Wrong idea 2

UDP is an inferior TCP.

No. UDP is a different transport contract.

### Wrong idea 3

ICMP is only ping.

No. It is core control and diagnostic signaling.

### Wrong idea 4

SOME/IP is a transport protocol.

No. It is middleware above transport protocols.

### Wrong idea 5

Service discovery is the same thing as application payload delivery.

No. Discovery is the control plane that enables the payload path.

### Wrong idea 6

Spanning tree is routing.

No. It is Layer-2 loop prevention.

## 38. Fast memory anchors

If you remember only a few things, remember these:

- IP gives addressing and forwarding, not reliability.

- TCP and UDP are different delivery contracts, not a good/bad pair.

- ICMP is the network layer talking back.

- SOME/IP gives service-level meaning above IP.

- SOME/IP Service Discovery makes service-oriented communication usable in a dynamic system.

- STP / RSTP / MSTP keep bridged Ethernet loop-free.

- In automotive systems, you always need to ask: Which layer is actually failing?

## 39. Suggested instructor discussion prompts

- Why is a successful ping not proof that a SOME/IP service works?

- When is UDP actually the better design choice in a vehicle?

- Why is ICMPv6 more operationally important than many beginners expect?

- Why can a physically redundant switch network become less reliable if loop control is wrong?

- What is the difference between a service ID and a TCP/UDP port number?

## 40. Closing summary

This whole topic becomes much easier once you stop memorizing acronyms in isolation.

The clean mental model is:

- IPv4 / IPv6 tell packets where to go.

- TCP / UDP decide what kind of end-to-end delivery contract exists.

- ICMP / ICMPv6 report network-layer control and error conditions.

- SOME/IP expresses application services, methods, and events.

- SOME/IP Service Discovery tells nodes which services exist and how to reach them.

- STP / RSTP / MSTP ensure redundant bridged Ethernet topologies do not self-destruct through loops.

That is the data communications understanding.

Then the automotive interpretation is simply this:

A vehicle network is still a network, but the engineering discipline around timing, topology, diagnostics, and software integration is much tighter. Once you understand the protocol fundamentals clearly, the automotive behavior becomes much easier to reason about.