# What is EUI and How Is It Derived from a MAC Address?

A concise engineering note with worked examples

# 1. What is EUI?

EUI stands for Extended Unique Identifier. It is an identifier format defined by IEEE for globally unique hardware identifiers.

In Ethernet discussions, the two important forms are EUI-48 and EUI-64.

| Form | Length | Typical use |
| --- | --- | --- |
| EUI-48 | 48 bits | Traditional Ethernet MAC address form |
| EUI-64 | 64 bits | Expanded identifier form, used in some link-layer and IPv6-related contexts |

A normal Ethernet MAC address such as 00:1A:2B:3C:4D:5E is already an EUI-48.

# 2. How EUI-48 relates to a MAC address

When people say “derive the EUI from the MAC address,” the phrase can mean two different things.

If they mean EUI-48, then the MAC address itself is already the EUI-48.

If they mean EUI-64, then the 48-bit MAC address is expanded into a 64-bit form.

# 3. How to derive EUI-64 from a 48-bit MAC address

Start with a 48-bit MAC address in six octets:

00:1A:2B:3C:4D:5E

Split it into two halves:

First 3 bytes: 00:1A:2B

Last 3 bytes: 3C:4D:5E

Insert FF:FE in the middle. This gives the 64-bit EUI form:

00:1A:2B:FF:FE:3C:4D:5E

# 4. Modified EUI-64 and the U/L bit

In many networking contexts, especially older IPv6 interface identifier derivation, one more step is applied: the Universal/Local bit, also called the U/L bit, in the first octet is inverted.

The U/L bit is the second least significant bit of the first byte.

For the MAC address 00:1A:2B:3C:4D:5E, the first byte is 00 hex, which is 00000000 in binary.

If you flip the U/L bit, the first byte becomes 02 hex, which is 00000010 in binary.

So the modified EUI-64 becomes:

02:1A:2B:FF:FE:3C:4D:5E

# 5. Worked examples

## Example 1

Original MAC address: 00:1A:2B:3C:4D:5E

Insert FF:FE in the middle → 00:1A:2B:FF:FE:3C:4D:5E

Invert the U/L bit in the first byte 00 → 02

Modified EUI-64 → 02:1A:2B:FF:FE:3C:4D:5E

## Example 2

Original MAC address: AC:DE:48:12:34:56

Insert FF:FE in the middle: AC:DE:48:FF:FE:12:34:56

Now invert the U/L bit in the first byte.

AC hex = 10101100 binary. After flipping the U/L bit it becomes AE hex = 10101110 binary.

So the modified EUI-64 is:

AE:DE:48:FF:FE:12:34:56

# 6. Important distinction

MAC address = EUI-48

Insert FF:FE to expand EUI-48 into EUI-64

Invert the U/L bit only when you need the modified EUI-64 form

Strictly speaking, plain EUI-64 expansion and modified EUI-64 are not identical. The modified form is the one commonly seen in IPv6 interface identifier discussions.

# 7. Compact summary

A 48-bit Ethernet MAC address is already an EUI-48. To derive a 64-bit EUI-style identifier, insert FF:FE in the middle. If you need the modified EUI-64 form, also invert the U/L bit in the first byte.