A GPS clock is a clock that uses signals from the Global Positioning System (GPS) to set or discipline its time.

The basic idea is simple:

- satellites in the Global Positioning System carry very accurate atomic clocks

- each satellite broadcasts timing information

- a GPS receiver on Earth listens to those signals

- from them, it determines an extremely accurate time reference

So a GPS clock is not “just a digital clock with GPS in it.” It is really a time-reference device that locks itself to the timing transmitted by the satellites.

## What it actually means

A normal clock runs from its own local oscillator. That oscillator drifts.

A GPS clock instead says:

“I will compare my local clock against the time coming from GPS satellites and keep correcting myself.”

So internally it usually has two parts:

- a local oscillator or local clock source

- a GPS receiver that disciplines that local clock

That local oscillator may be:

- a quartz oscillator

- a temperature-compensated crystal oscillator

- an oven-controlled crystal oscillator

- in high-end equipment, a rubidium oscillator

The better the local oscillator, the better the clock behaves when GPS reception is weak or temporarily lost.

## Why GPS time is useful

The Global Positioning System was built for positioning, but the timing aspect is just as important.

A GPS clock is useful because it can provide:

- very accurate absolute time

- a common time base over widely separated locations

- synchronization for networks, power systems, telecom systems, industrial control, measurement systems, and laboratories

For example, if two devices in different cities both lock to GPS, they can share nearly the same time even though they are far apart.

## How it works in practice

A GPS receiver gets signals from multiple satellites. Each satellite transmits:

- its own timestamp

- orbital data

- correction information

From the propagation delays and the satellite data, the receiver solves for:

- position

- and time

For pure timing applications, the device may use a known fixed antenna location. That simplifies the calculation and improves timing stability.

Once the receiver has a good solution, it can generate outputs such as:

- a time-of-day value

- a Pulse Per Second (PPS) signal

- Network Time Protocol or Precision Time Protocol reference signals

- frequency references like 10 megahertz

## What is the Pulse Per Second signal

One of the most important outputs of a GPS clock is the Pulse Per Second (PPS) signal.

This is a very sharp electrical pulse that occurs once every second, aligned to the start of the second.

That pulse is extremely useful because a system can use it to align its own local clock very precisely. In many engineering systems, the Pulse Per Second output is more important than the displayed clock time.

## GPS clock versus atomic clock

A GPS clock is often confused with an atomic clock.

They are not the same thing.

- a GPS clock usually does not contain a cesium atomic clock

- it derives its accuracy from GPS satellite timing

- the satellites themselves carry atomic clocks

So the ground receiver is typically a disciplined clock, not necessarily an atomic clock of its own.

Some high-end systems combine:

- a local high-quality oscillator

- GPS disciplining

- holdover logic

That gives performance closer to a precision timing instrument.

## GPS time versus Coordinated Universal Time

Another important point:

GPS time is not exactly the same as Coordinated Universal Time (UTC).

Why?

Because GPS time does not insert leap seconds in the same way that Coordinated Universal Time does.

So a GPS clock usually has to apply an offset to convert GPS system time into Coordinated Universal Time for civil timekeeping.

This is why timing receivers often expose both:

- raw GPS-related timing internally

- corrected Coordinated Universal Time externally

## What happens if GPS is lost

A good GPS clock does not instantly become useless if satellite lock is lost.

Instead, it enters holdover mode.

That means:

- it keeps running from its local oscillator

- it remembers its recent correction history

- it tries to maintain accurate time for some period without GPS updates

The holdover quality depends heavily on the quality of the oscillator.

A cheap crystal-based GPS clock may drift quickly.
A good oven-controlled or rubidium-based unit may hold accurate time much longer.

## Examples of where GPS clocks are used

In telecom, a GPS clock may synchronize base stations.

In power systems, it may synchronize disturbance recorders and phasor measurement units.

In networking, it may act as the grandmaster reference for the Precision Time Protocol (PTP) or the generalized Precision Time Protocol (gPTP) domain.

In labs, it may provide precise Pulse Per Second and frequency outputs.

In automotive development or test setups, it can be used when engineers want an external absolute time reference for logging, correlation, or synchronization across distributed systems.

## A good engineering mental model

Think of a GPS clock like this:

- the local oscillator is the engine

- GPS is the steering correction

- the output is a disciplined, stable notion of time

So it is not that the device blindly trusts every incoming satellite message every instant. Rather, it continuously compares its own clock to GPS and nudges itself into alignment.

## Summary

A GPS clock is a clock or timing source that uses Global Positioning System satellite timing to set and discipline its local oscillator, providing a highly accurate common time reference.