# Cross Timestamping in Precision Hardware Clock and System Counter Architectures

A normal packet timestamp gives one clock reading at one packet event. A cross timestamp gives a coherent pair:

- one value from the device’s Precision Hardware Clock (PHC)

- one value from the system-side counter or clock domain

captured as nearly simultaneously as the hardware allows.

## 1. The two clocks you are trying to relate

In this architecture you usually have:

- a device clock inside the Ethernet controller or Media Access Control (MAC) / Physical Layer (PHY) timing block, used for packet timestamping and Precision Time Protocol operations

- a system counter or CPU-side timebase, such as a Time Stamp Counter (TSC) or platform reference counter

The fundamental problem is:

[  T_{},  T_{}  ]

That is what cross timestamping solves.

## 2. The hardware blocks involved

A useful microarchitecture picture is:

PHC counter ------------------\\
                               \\
                                > snapshot/latch fabric -> latched PHC regs
System counter / reference ----/                         -> latched SYS regs

                    snapshot control register
                    snapshot status / sequence register

The important pieces are:

- a free-running PHC counter

- a free-running system/reference counter

- a common snapshot trigger

- latch registers that hold the captured values

- a status/sequence mechanism so software knows the pair is coherent

## 3. The best implementation: true hardware snapshot

The highest-quality design is:

- software writes a snapshot request

- hardware fans that request into both clock domains

- both counters are latched

- software reads a stable pair from dedicated registers

Conceptually:

write SNAPSHOT_REQ = 1
        ↓
common internal pulse
        ↓
latch(PHC_counter) and latch(SYS_counter)
        ↓
SNAPSHOT_DONE = 1

This is what “simultaneous” means in practice. It does not mean mathematically perfect simultaneity, but that both domains are captured by the same hardware event with bounded and characterized skew.

## 4. What the registers look like

A vendor-neutral register map usually looks something like this:

SNAP_CTRL
  bit0: SNAP_REQ

SNAP_STATUS
  bit0: SNAP_DONE
  bit1: SNAP_OVERRUN
  bits[31:16]: SNAP_SEQ

PHC_SNAP_LO
PHC_SNAP_HI

SYS_SNAP_LO
SYS_SNAP_HI

Optional designs add:

SNAP_ERR_NS        // known fixed skew/error estimate
SNAP_DOMAIN_SEL    // choose which PHC or which system counter
SNAP_VALID_WINDOW  // only accept snapshots within timing bounds

The key rule is that software should read a coherent pair. Good hardware usually provides:

- a done bit

- a sequence number

- or an atomic snapshot bank

so software can detect whether one side updated while the other was being read.

## 5. Why a reference counter often sits between PHC and system time

In serious implementations, the paired hardware snapshot is often between:

- the device clock

- and a reference system counter

not directly between the device clock and wall-clock time.

That matters because wall-clock time can be stepped or slewed, while a raw reference counter is stable and monotonic.

So the practical pipeline is often:

[ (, )  (, ) ]

## 6. The conversion path after the hardware snapshot

Once you have the raw pair, there are usually two conversions.

### 6.1 Device cycles to PHC nanoseconds

If the PHC is implemented as a cyclecounter/timecounter pair:

[ T_{} = f_{}(C_{}) ]

where:

- (C_{}) is the raw PHC cycle snapshot

- (f_{}) converts those cycles into nanoseconds

### 6.2 System counter cycles to system time

Similarly:

[ T_{} = f_{}(C_{}) ]

where:

- (C_{}) is the raw system/reference counter snapshot

- (f_{}) converts that into the system time domain

Only after those two conversions do you have the final cross timestamp pair:

[ (T_{}, T_{}) ]

## 7. The core correction equation

Once you can obtain repeated coherent pairs, the relationship between the PHC and the system domain is modeled as an affine map:

[ T_{} = a T_{} + b ]

where:

- (b) is the offset

- (a) captures any relative frequency error or skew

If the clocks are frequency-locked, then (a ) and you mostly care about the fixed offset (b).

If there is no guaranteed frequency lock, then multiple cross-timestamp pairs are used to estimate both (a) and (b).

With two pairs:

[ a  ]

[ b T_{,1} - a T_{,1} ]

Then a packet timestamp taken in the PHC domain can be translated into the system domain with:

[ {} = a T{} + b ]

## 8. A worked register-level example

Assume the hardware exposes:

PHC_SNAP_HI = 0x00000002
PHC_SNAP_LO = 0x540BE400     // PHC cycles

SYS_SNAP_HI = 0x00000010
SYS_SNAP_LO = 0x2A05F200     // system/reference cycles

The driver does:

- request snapshot

- wait for SNAP_DONE

- read both 64-bit values

- convert the PHC cycles through its PHC conversion path

- convert the system/reference cycles through the system time conversion path

Suppose those conversions produce:

[ T_{} = 12.345678900  ]

[ T_{} = 105.200001230  ]

That pair means:

At one common hardware snapshot event, the PHC read 12.345678900 s while the system domain read 105.200001230 s.

Now suppose a packet timestamp later arrives as:

[ T_{} = 12.445678900  ]

If the two domains are frequency-locked, you can use the fixed offset:

[ b = 105.200001230 - 12.345678900 = 92.854322330  ]

and get:

[ T_{}  + 92.854322330 = 105.300001230  ]

That is how a network packet timestamp becomes comparable with CPU-domain timestamps from sensors, interrupts, or other subsystems.

## 9. Snapshot skew term

No real implementation captures both clocks at an infinitely sharp mathematical instant. There is always some finite skew:

[ t_{} ]

So the real relation is better written as:

[ T_{} = a T_{} + b + _{} ]

where (_{}) is the residual snapshot error.

What makes hardware cross timestamping valuable is that this error is usually:

- much smaller than software-bracketed reads

- bounded

- repeatable enough to calibrate or ignore at the target precision level

## 10. PPS-assisted locking architecture

Not every platform exposes a “snapshot both counters now” mechanism. Another important architecture is:

- the Ethernet MAC or PHC generates a Pulse Per Second (PPS) event at a known boundary

- the system counter domain captures or disciplines itself to that PPS

- the two counters become aligned, often with fixed offset and negligible drift

- later conversions become simple because the two domains have been hardware-locked

In that design, cross timestamping is less about on-demand snapshots and more about maintaining a permanent hardware relation between domains.

## 11. Fallback architecture: software-bracketed reads

For completeness, the older fallback is not true hardware cross timestamping.

It looks like:

system time read (t1)
device time read (d)
system time read (t2)

Then the associated system time is estimated using the bracket around the device read.

That method is useful when hardware support is absent, but it is not what you want for high-precision Time-Sensitive Networking, generalized Precision Time Protocol, or sensor-fusion correlation.

## 12. Driver API view

From the driver writer’s point of view, the hardware cross timestamp path is:

driver callback
   ├─ read synchronized device snapshot registers
   ├─ read synchronized system/reference snapshot registers
   ├─ convert device cycles to PHC ns
   └─ return raw system counter value for timekeeping conversion

A good implementation relies on:

- coherent register pairs

- a snapshot request mechanism

- a status/sequence handshake

- conversion helpers for both time domains

## 13. Clean implementation recipe

If you were designing this in silicon, the best-practice recipe is:

- keep the PHC and system/reference counter free-running

- provide a common snapshot trigger

- latch both counters into dedicated snapshot registers with one internal event

- expose status/sequence bits so software can verify coherence

- convert raw cycles through:

  - a PHC conversion path for the device side

  - a correlated system-counter path for the system side

- use repeated pairs to estimate the affine relation:

[ T_{} = a T_{} + b ]

- if possible, add PPS-assisted hardware locking so (a) becomes effectively 1 and only (b) matters

## 14. Final takeaway

Packet hardware timestamping answers:

What time was this packet in the PHC domain?

Cross timestamping answers:

At the same hardware instant, what were the PHC and system counters?

And the whole implementation reduces to:

- registers that expose coherent snapshots

- latches triggered by one internal event

- conversion equations that map one time domain into the other

That is the essence of cross timestamping in PHC plus system-counter architectures.