Following is  an actual datapath walkthrough, separately for the Media Access Control (MAC) and the Physical Layer (PHY), and separately for transmit and receive.

The key idea is:

A hardware timestamp is only meaningful if you know exactly which pipeline event caused the clock capture.

In Ethernet timestamping, that event is usually tied to the start of frame delimiter (SFD) or to a fixed internal reference point that has a known relationship to the SFD. TI’s PHY documentation explicitly uses the SFD as the common timestamp point and notes that PHY-level SFD timing is more stable and repeatable than measuring at the MAC interface.

# 1. The clock that is being captured

Before packet pipelines, first remember that hardware timestamping needs a dedicated hardware timebase.

A typical IEEE 1588 / Precision Time Protocol-capable device has a local hardware clock with fields for seconds, nanoseconds, and subnanoseconds/fractional nanoseconds. For example, Microchip’s LAN7430 describes a hardware PTP clock with a 32-bit seconds field, a 30-bit nanoseconds field, and a 32-bit subnanosecond field; it is used for PTP-related functions and can be read, written, and adjusted in hardware. TI’s DP83640 similarly describes a hardware IEEE 1588 clock with seconds, nanoseconds, and fractional nanoseconds.

So when I say “capture a timestamp,” what is really happening is:

packet event  ──► latch current hardware clock value

not “ask software what time it is.”

# 2. What the packet pipeline looks like

A simplified Ethernet path is:

## Transmit

Host / DMA / descriptors

↓

MAC transmit logic

↓

MAC-PHY interface (MII / RMII / RGMII / SGMII / xGMII)

↓

PHY transmit pipeline

↓

MDI / cable

## Receive

MDI / cable

↓

PHY receive pipeline

↓

MAC-PHY interface

↓

MAC receive logic

↓

DMA / descriptors / host

The important question is:

At which one of these boundaries is the timestamp captured?

That choice determines both accuracy and repeatability.

# 3. The reference event: why SFD matters

Ethernet frames are preceded by preamble and then the start of frame delimiter (SFD). TI’s PHY documentation states that the SFD is commonly used as the identifier for the start of the packet and is therefore commonly used for timestamping. It also shows that a PHY can output a transmit or receive SFD indication on a GPIO for timestamping.

That makes the SFD the natural “anchor” for precise packet timing:

- on transmit, it means “this frame is starting to leave”

- on receive, it means “this frame has just started to arrive”

In practice, hardware may not expose the raw SFD itself. Instead it may expose:

- a captured timestamp register

- a FIFO entry

- a sideband metadata field

- or automatic insertion into the packet for one-step operation

But architecturally, the timestamp is still tied to a defined event point near SFD.

# 4. MAC-level hardware timestamping — transmit path

Now let us start with MAC transmit timestamping.

## 4.1 What the MAC sees

The MAC does not see the actual analog waveform on the cable. It sees:

- packet bytes

- frame boundaries

- the transmit enable / valid signals

- the MAC-PHY interface clocking

So MAC-level timestamping usually means:

Capture the local PTP clock when the frame reaches a defined MAC transmit event, commonly the beginning of transmission at or near the MAC interface.

This is much better than software, because it avoids scheduler, interrupt, and queueing jitter, but it still occurs before the PHY has completely turned the data into symbols on the medium. TI explicitly says timestamping in the MAC is straightforward but is subject to multiple sources of variation, and that PHY timestamping is more accurate.

## 4.2 MAC TX pipeline step-by-step

Conceptually:

(1) Frame is queued for TX

(2) MAC parser recognizes it as a timestamped frame

(3) MAC begins frame launch at a defined transmit reference point

(4) MAC latches local PTP clock

(5) Timestamp is attached to the frame or placed in a TX timestamp register/FIFO

(6) Frame continues through MAC-PHY interface into PHY

If the MAC supports one-step PTP, it may also modify the frame with the transmit timestamp as it leaves.

## 4.3 What is good about MAC TX timestamping

- avoids software and DMA completion uncertainty

- can be tightly integrated with descriptor processing

- often easier to implement than PHY-level timestamping

## 4.4 What still limits MAC TX accuracy

Between the MAC timestamp point and the actual wire there is still:

- MAC-PHY interface timing

- clock-domain crossing

- PHY serialization / coding latency

- possible elastic buffering in the PHY

That residual path is why MAC timestamping is not the ultimate reference point.

# 5. MAC-level hardware timestamping — receive path

## 5.1 What the MAC sees on RX

On receive, the MAC sees data only after the PHY has recovered the signal from the cable, decoded it, and presented it across the MAC-PHY interface.

So MAC RX timestamping usually means:

Capture the local PTP clock when the MAC detects frame start at its receive-side reference point.

## 5.2 MAC RX pipeline step-by-step

(1) PHY receives symbols from cable

(2) PHY decodes and forwards data over MAC-PHY interface

(3) MAC detects start of frame

(4) MAC latches local PTP clock

(5) Timestamp is stored with RX metadata / descriptor / RX timestamp FIFO

(6) Frame continues to DMA / host

Again, this is much earlier and more precise than a software receive timestamp, but it is still downstream of PHY processing.

## 5.3 Why MAC RX has residual uncertainty

TI’s SFD note explains that the MAC and PHY may have independent clock sources, so a clock-domain crossing at the MAC interface can introduce timestamp variation. TI shows that the receive MAC interface signal can vary relative to the PHY’s receive SFD, and that the PHY SFD provides the more stable timestamp point.

That is the main hardware reason why two MAC-level receive timestamps can still have more jitter than PHY-level receive timestamps.

# 6. PHY-level hardware timestamping — transmit path

This is the “closer to the wire” method.

TI’s article says the most accurate approach is to implement timestamp functionality directly in the hardware of the Ethernet PHY, because the transceiver inserts or captures the timestamp as close to the wire as possible. TI’s DP83640 datasheet also describes transmit packet parsing, start-of-frame detection, and dedicated transmit timestamp units in the PHY itself.

## 6.1 PHY TX pipeline step-by-step

(1) MAC sends frame over MII/RGMII/SGMII/etc.

(2) PHY transmit parser watches the outgoing frame

(3) PHY recognizes a timestamp-relevant frame (for example, a PTP event message)

(4) PHY detects the transmit start event at its internal reference point near SFD

(5) PHY latches its local 1588 clock

(6) Timestamp is stored in a TX timestamp register/FIFO or inserted into the frame

(7) PHY encodes/serializes and drives the signal onto MDI/cable

TI’s DP83640 says the transmit parser monitors outgoing packet data, detects Precision Time Protocol event messages in Layer 2 or User Datagram Protocol over Internet Protocol, and upon detection captures the transmit timestamp and provides it to software; the device can buffer four transmit timestamps.

## 6.2 One-step PHY transmit

This is where PHY-level timestamping gets especially powerful.

TI’s DP83640 says that in one-step mode, for Sync messages, the transmitter can automatically insert timestamp information into the outgoing packet, eliminating the need for software to read the timestamp and send a Follow_Up; it also regenerates the checksum and cyclic redundancy check as needed.

So the sequence becomes:

PHY sees Sync frame

↓

PHY captures exact TX timestamp

↓

PHY writes correction/origin timestamp into the outgoing frame

↓

PHY fixes checksums / CRC

↓

Frame leaves

That is about as “on the fly” as packet timing gets.

## 6.3 Why PHY TX is better than MAC TX

Because it removes or reduces uncertainty from:

- MAC-PHY boundary timing

- clock-domain crossing

- internal PHY launch latency variability

TI’s SFD documentation explicitly says timestamping within the Ethernet PHY provides better accuracy than timestamping in the MAC, and that the PHY SFD pulse provides a stable, repeatable timestamp reference.

# 7. PHY-level hardware timestamping — receive path

This is the receive-side analogue and is usually the cleanest receive reference.

## 7.1 PHY RX pipeline step-by-step

(1) Signal arrives at MDI/cable

(2) PHY receiver detects / equalizes / decodes

(3) PHY identifies frame start / SFD internally

(4) PHY latches the local 1588 clock at that receive reference point

(5) PHY stores timestamp in RX timestamp register/FIFO

or inserts it into the packet metadata/payload

(6) Frame is delivered across MAC interface

(7) MAC / DMA / host receive the frame later

TI’s DP83640 says the receive timestamp unit can be enabled to capture timestamps for receive event messages, can record associated message identification fields, and can deliver the timestamp either through registers or by inserting it into the received packet.

## 7.2 Why PHY RX is better than MAC RX

Again, because it occurs before:

- MAC interface timing uncertainty

- receive-side clock-domain crossing

- MAC packet-start interpretation jitter

TI’s DP83867 application note compares RX interface timing against RX SFD timing and shows that the RX SFD is the more repeatable timing point. It says similar repeatability improvement applies in both transmit and receive directions.

# 8. The exact difference between MAC timestamping and PHY timestamping

This is the cleanest way to think about it.

## MAC timestamping

Timestamp point is tied to a MAC event:

- frame start at MAC transmit side

- frame start detected at MAC receive side

So it is precise relative to the MAC clock domain, but it still includes uncertainty between MAC and medium.

## PHY timestamping

Timestamp point is tied to a PHY event:

- SFD / frame-start event inside the PHY

- or another internal reference point with known relation to the medium

So it is closer to the actual line event and is therefore usually more repeatable and more accurate. TI explicitly says PHY timestamping is more accurate than MAC timestamping, and that SFD pulses provide a stable and repeatable point.

# 9. Where the timestamp is stored or delivered

Once a timestamp is captured, hardware still has to expose it somehow.

Typical hardware mechanisms are:

## TX side

- transmit timestamp register

- timestamp FIFO

- descriptor-side metadata

- one-step insertion into the outgoing frame

TI’s DP83640 has dedicated transmit timestamp registers and can buffer multiple timestamps; it also supports one-step insertion for transmit Sync messages.

## RX side

- receive timestamp register

- receive timestamp FIFO

- receive descriptor metadata

- inserted timestamp inside the received packet

TI’s DP83640 explicitly supports receive timestamp registers and receive timestamp insertion into the received packet.

# 10. Why “TX completion timestamp” is not the same thing

This is worth stating because many engineers confuse it with true hardware packet timestamps.

The Linux kernel timestamping documentation says a TX completion timestamp is generated when the kernel receives a completion report from the hardware, and it reflects the timing of the report rather than the actual transmit instant. It separately refers to reporting raw hardware timestamps generated by hardware TX/RX timestamping.

So:

- real hardware TX timestamp = captured at the defined packet launch event

- completion timestamp = captured when completion is reported upstream

Those are not the same pipeline point.

# 11. A good physical picture of all four cases

## A. MAC TX timestamp

Host → MAC [timestamp here] → MAC/PHY interface → PHY → cable

## B. PHY TX timestamp

Host → MAC → interface → PHY [timestamp here, near SFD / wire-side launch] → cable

## C. PHY RX timestamp

cable → PHY [timestamp here, near SFD / wire-side arrival] → interface → MAC → Host

## D. MAC RX timestamp

cable → PHY → interface → MAC [timestamp here] → Host

The closer the timestamp point is to the actual line event, the less uncertainty remains between the timestamp and the real physical packet timing.

# 12. What “inside the pipeline” really means in practice

A hardware timestamping block inside a MAC or PHY usually has these internal subfunctions:

packet parser / recognizer

↓

timestamp trigger logic

↓

local 1588/PTP clock capture

↓

timestamp association logic

↓

delivery path (register/FIFO/descriptor/packet insertion)

The parser decides whether the frame is relevant for timestamping.
The trigger logic decides the exact event point.
The capture logic latches the hardware clock.
The association logic makes sure the correct timestamp is matched to the correct frame.

TI’s DP83640 explicitly describes transmit and receive packet parsing, start-of-frame detection, and dedicated transmit and receive timestamp units.

# 13. One-step vs two-step in the pipelines

This becomes very important in transmit.

## Two-step

The hardware captures the transmit timestamp, but the timestamp is delivered separately. Then a Follow_Up message carries it later.

Pipeline view:

TX frame starts

↓

timestamp captured

↓

timestamp reported separately

↓

Follow_Up sent later

## One-step

The hardware captures the timestamp while the frame is being transmitted and writes it into the frame in flight.

Pipeline view:

TX Sync starts

↓

timestamp captured in hardware

↓

frame field updated in flight

↓

checksums/CRC fixed

↓

frame exits already carrying precise time

TI’s DP83640 explicitly documents this one-step transmit capability for Sync messages.

# 14. What actually limits timestamp precision after you move into hardware

Even with hardware timestamping, precision is not infinite. Main limits are:

- clock quality of the local PTP clock

- timestamp counter resolution

- exactness of the chosen reference point

- fixed latency calibration between the timestamp point and the true medium event

- MAC-PHY clock-domain crossing if the timestamp point is at the MAC side

- PHY internal latency repeatability

TI’s PHY note emphasizes both latency and repeatability, and specifically says that variation in PHY latency and MAC-interface timing affects synchronization precision.

# 15. The engineering summary you should keep

Here is the shortest correct mental model:

## Hardware timestamping in the MAC

The MAC captures the PTP clock when a frame crosses a defined MAC transmit or receive boundary. This is much better than software but still includes uncertainty between the MAC and the actual wire.

## Hardware timestamping in the PHY

The PHY captures the PTP clock when the frame crosses a PHY reference point tied closely to the start of frame delimiter or another near-medium event. This minimizes uncertainty from the MAC-PHY boundary and is usually the most accurate packet timestamp available.

# 16. One-line conclusion

MAC timestamping measures packet time at the MAC boundary; PHY timestamping measures packet time closer to the actual line event, usually around SFD detection, so PHY timestamping is generally the more accurate and repeatable hardware reference.

If you want, I can next draw this as four detailed timing diagrams:

- MAC TX

- PHY TX

- PHY RX

- MAC RX

with the exact event markers, buffers, and clock domains shown side by side.