A bridge does not usually “calculate” the stream handle from the packet fields by a formula.

In standards terms, the stream identification function examines configured fields, and when a rule matches, it associates the received frame with a configured stream_handle.

The fields used for identification depend on the configured identification type.

IEEE material shows examples such as destination address + VLAN identifier, source address + VLAN identifier, and richer mask-and-match styles.

So for your example, if the bridge is configured to identify a stream using:

{ingress port, destination MAC, VLAN}

then the bridge does this at ingress:

- read the incoming metadata

  - ingress port = 5

  - VLAN = 100

  - destination MAC = 01-00-5E-...

  - priority = 5

- look up that tuple in its stream-identification rule table

- if a rule matches, assign the stream_handle value stored in that rule. That value is an administrative/configured identifier, not something mathematically derived from the packet. IEEE Qci/Qcr text then uses the received frame’s stream_handle and priority to select the applicable stream filter.

A good mental model is:

{ingress port, DA, VID}  ──match rule──►  stream_handle = configured value

For example, the hardware may contain a rule like:

Rule 17:

match ingress port = 5

match VLAN = 100

match destination MAC = 01-00-5E-12-34-56

→ assign stream_handle = 17

If your incoming frame matches that rule, the bridge assigns stream_handle = 17. If the table had stream_handle = 44 for that same tuple, then the assigned value would be 44 instead. The packet fields decide which rule matches; the rule decides which stream_handle is output.

In your specific example, priority 5 does not affect the stream_handle assignment if the identification rule is only {ingress port, destination MAC, VLAN}. Priority can matter later, because IEEE stream filtering uses the frame’s stream_handle together with its priority to select the stream filter instance. In other words, priority may affect later policing/gating, but not the initial stream-handle assignment unless the identification rule itself was configured to include priority.

There is also one subtle point about your destination MAC 01-00-5E-...: that is only a prefix, not a full address. So the bridge can behave in one of two ways depending on configuration:

- exact-match rule: it matches only one full destination MAC, for example 01-00-5E-12-34-56

- masked/prefix rule: it matches a family of addresses sharing that prefix, if the implementation supports mask-and-match style identification. IEEE extended stream identification explicitly allows masked matching over destination MAC and VLAN-related fields.

So the answer to your question is:

The bridge assigns a stream handle based on which configured stream-identification rule matches the frame. It does not inherently derive the value from the fields themselves. With {ingress port, destination MAC, VLAN} as the identification key, your frame on port 5, VLAN 100, destination MAC 01-00-5E-... gets whatever stream_handle that matching rule was configured to output. Priority 5 is ignored for that assignment unless priority is explicitly part of the identification rule.

If you want, I can draw this as a small switch lookup table example with 4 or 5 rules and show exactly how your sample frame walks through the match logic.