## 1-step vs 2-step timestamping; hardware timestamp pipeline; cross-timestamping (AUTOSAR StbM); Stream ID (802.1CB); Stream Reservation (802.1Qat/Qcc SRP)

## 1. Why these topics belong together

These topics all sit at the boundary between “ordinary Ethernet packets” and “deterministic communication.”

One-step versus two-step timestamping is about how precise time information is inserted into synchronization messages.

The hardware timestamp pipeline is about where, physically, the timestamp is captured so software latency does not ruin accuracy.

Cross-timestamping is about correlating one clock domain with another.

IEEE 802.1CB Frame Replication and Elimination for Reliability (FRER) stream identification is about recognizing which packets belong to the same deterministic stream.

IEEE 802.1Qat Stream Reservation Protocol (SRP) and IEEE 802.1Qcc SRP Enhancements and Performance Improvements are about reserving network resources for those streams.

In other words, time, stream identity, and reserved resources are three pieces of the same deterministic-network puzzle.

## 2. 1-step versus 2-step timestamping from first principles

In one-step timestamping, the precise egress timestamp is carried directly inside the outgoing Sync message itself, so no separate Follow_Up message is needed.

In two-step timestamping, the Sync message goes out first, and the precise timestamp is then sent in a later Follow_Up message.

IEEE tutorial material states this very plainly: one-step clocks carry the Sync egress timestamp in the Sync message and do not send Follow_Up, whereas two-step clocks carry the Sync egress timestamp in the Follow_Up message.

That difference matters because the true transmission timestamp is only known at the instant the message actually leaves the timestamping point.

If the hardware can modify the outgoing Sync frame “on the fly,” one-step is possible.

If it cannot do that cleanly, the device can still achieve precise timing by transmitting the Sync first and then reporting the exact timestamp afterward in a Follow_Up.

This is why two-step is not “less precise by definition.” It is often simply a different implementation strategy for delivering the same underlying timing information.

## 3. Why 2-step was historically favored in generalized Precision Time Protocol

IEEE 802.1AS generalized Precision Time Protocol (gPTP) originally chose the two-step style.

IEEE 802.1 tutorial material says IEEE 802.1AS-2011 was limited to two-step, while later revision work added one-step ports.

Another IEEE presentation on whether to adopt one-step says that two-step Precision Time Protocol (PTP) was chosen for IEEE 802.1AS and that the standards committee later considered adding one-step capability as networks became more engineered.

That historical choice makes sense.

Two-step is often easier to implement because the hardware does not need to patch the precise egress timestamp into the Sync frame during transmission.

Instead, it can capture the exact send time and hand it to the Follow_Up generation path.

One-step can reduce message count, but it pushes more work into the critical transmit datapath. IEEE deterministic Ethernet tutorial material later noted that explicit one-step support was being added while remaining backward compatible to two-step.

## 4. The hardware timestamp pipeline from first principles

A timestamp is only useful if it is taken at a well-defined physical event boundary.

If software timestamps a packet long before transmission or long after reception, queueing delay, interrupt latency, and operating-system scheduling noise will dominate the measurement.

Hardware timestamping happens at the network interface hardware boundary.

Hardware timestamps are typically exposed separately from ordinary software timestamps and the PTP Hardware Clock (PHC) is exposed directly as a hardware time source rather than hiding everything behind converted system time.

On transmission, the cleanest timestamp point is as close as possible to the moment the frame is actually put on the wire.

On reception, the cleanest point is as close as possible to the moment the frame is accepted from the wire. Incoming packets are timestamped before they are distributed further into the system, and outgoing packets are timestamped very close to placing them on the wire.

PHY timestamping support, when present, is mediated by the Media Access Control (MAC) driver, because PHYs do not appear as ordinary standalone network interfaces.

So the hardware timestamp pipeline is best understood as a chain:

Application → Operating System Stack → Driver → MAC / PHY timestamp point → physical medium.

The closer the timestamp is taken to the MAC/PHY event boundary, the less software noise contaminates the result.

That is the whole reason hardware timestamping exists in PTP systems.

## 5. Why the timestamp location matters for one-step and two-step

One-step and two-step are really two different ways of handling that same physical fact: the true egress time is only known at the transmission boundary.

In one-step, the hardware must capture the time and insert or correct the timestamp in the outgoing event frame itself.

In two-step, the hardware captures the timestamp, but the precise value is communicated afterward in a Follow_Up.

This is why one-step is often associated with more demanding hardware pipelines.

The transmit datapath must be able to determine the precise launch time and update the outgoing event message at the right moment.

Two-step relaxes that requirement because the event frame and the precise timestamp report are separate protocol objects.

The trade-off is not “simple versus advanced” in the abstract; it is “do I want timestamp insertion in-band during transmission, or reported afterward out of band?”

## 6. Cross-timestamping from first principles

Cross-timestamping means taking an atomic or near-atomic snapshot of two different clock domains so their readings can be correlated.

In general networking systems, that often means correlating the NIC’s or PHY’s timestamp clock with the host’s PTP Hardware Clock or another system clock.

Hardware timestamps typically are explicitly separated from generic system timestamps and it is recommended that the hardware PTP clock is exposed directly so time conversion can be done properly.

The deep reason cross-timestamping matters is that “this packet was timestamped at 12:00:00.123456789” is meaningless unless you know which clock that number came from.

A synchronization stack usually needs to correlate timestamps captured in the network hardware with the system’s disciplined time base.

Cross-timestamping is therefore a bridge between clock domains, not just another timestamp.

## 7. Stream ID in IEEE 802.1CB from first principles

IEEE 802.1CB is the FRER standard, and its key operational concept is the stream_handle, often described informally as the identifier for a stream within the 802.1CB machinery.

IEEE stream-identification material says 802.1CB specifies how an end station or relay system can use the stream_handle to perform frame replication and elimination, and that IEEE 802.1Q can use the same stream_handle for Quality of Service (QoS) functions including Time-Sensitive Networking (TSN).

That means a “Stream ID” in this context is not just a label for human readability. It is the switch or end station’s way of saying, “these packets belong to the same deterministic stream and therefore should be treated together for replication, elimination, policing, or traffic management.”

The same IEEE material also says there is no provision in the standard for using the stream_handle to choose the output port directly.

That is important: stream identification is for traffic-treatment logic, not for replacing ordinary forwarding decisions.

## 8. What fields can determine a stream

IEEE stream-identification material gives a very useful list of candidate fields that can determine the stream_handle: input port number, source MAC address, destination MAC address, VLAN ID, Layer 2 priority, IP source address, IP destination address, next protocol such as UDP or TCP, source port number, destination port number, and Differentiated Services Code Point (DSCP).

The IEEE 802.1CB stream-identification YANG model also defines concrete identification types such as null stream identification, Source MAC and VLAN stream identification, Active Destination MAC and VLAN stream identification, and IP stream identification.

This is one of the most important conceptual points in deterministic networking: a stream is not always identified by one single field.

It is typically identified by a set of packet-header properties plus context, such as ingress port.

That is why stream identification is a classification problem, not a simple one-field lookup.

## 9. Stream Reservation Protocol from first principles

The official IEEE 802.1Qat page says Stream Reservation Protocol (SRP) provides a signaling protocol for end-to-end management of resource reservation for Quality of Service (QoS) guaranteed streams.

It also says the protocol supports registration, deregistration, and retention of resource reservation information and is an essential component for automatic configuration in bridged local-area-network applications requiring latency and bandwidth guarantees.

So the core purpose of SRP is simple: if a sender wants to establish a deterministic stream with bandwidth and latency expectations, the network should not just “hope for the best.”

It should explicitly reserve the needed resources along the path.

That is what SRP is for.

It turns stream establishment into an admission-control problem instead of leaving it entirely to best-effort forwarding.

## 10. Talkers, listeners, and how SRP works

A Talker announces the stream it can send, and a Listener requests the stream it wants to receive.

IEEE tutorial slides say Multiple Stream Reservation Protocol (MSRP or SRP) is used by talkers that want to send or listeners that want to receive data flows with bandwidth, latency, and congestion-loss requirements.

SRP automatically creates a path with guaranteed bandwidth from a talker to one or more listeners, and if the bridge can reserve sufficient bandwidth it adds the reservation to its tables; if not, failure is propagated back.

The clean mental model is therefore:

- the talker advertises the stream and its needs

- listeners declare interest

- each bridge on the path checks whether it has the required resources

- if all bridges can honor the request, the stream becomes reserved end to end

- if one bridge cannot, the reservation fails and that failure is propagated.

## 11. What IEEE 802.1Qcc changed

The official IEEE 802.1Qcc page says the amendment adds support for more streams, configurable SR classes and streams, better description of stream characteristics, support for Layer 3 streaming, deterministic stream reservation convergence, and a User/Network Interface (UNI) for routing and reservations.

An IEEE configuration-enhancements presentation also says 802.1Qcc supports three configuration models — decentralized, hybrid, and centralized — and provides YANG structures for the centralized model.

That is a major conceptual expansion.

Original SRP is strongly associated with distributed talker/listener signaling.

Qcc keeps compatibility with that world but also acknowledges that some deterministic networks want centralized control or hybrid approaches.

So Qcc is not “a different reservation protocol.” It is the modernization of SRP toward richer stream descriptions, more scale, and more configuration models.

# Automotive perspective and context

## 12. One-step versus two-step in vehicles

Automotive timing discussions have been unusually explicit on this topic. IEEE 802.1AS-2020 introduced one-step ports, whereas older 802.1AS revisions were limited to two-step.

At the same time, automotive profile discussions have often recommended the two-step approach to avoid complexity in vehicle implementations.

A 2019 IEEE automotive time-sync profile presentation explicitly says “recommend to use Two-Step approach,” and a 2025 JASPAR automotive time-sync proposal says the in-vehicle profile uses only two steps to avoid complexity in automotive systems.

That does not mean one-step is “wrong” for automotive.

It means the trade-off is different. Vehicles often prioritize implementation robustness, predictable validation, and multi-vendor interoperability over squeezing out the last bit of protocol efficiency.

Two-step fits that mindset because it avoids on-the-fly frame modification in the timing-critical egress datapath.

One-step remains attractive where the hardware pipeline already supports it cleanly and the architecture benefits from fewer timing messages.

## 13. Hardware timestamp pipeline in vehicles

Automotive networks care deeply about where timestamps are taken because time synchronization supports sensor fusion, time-aware scheduling, coordinated actuation, and event correlation.

AUTOSAR’s Ethernet stack has clearly moved toward explicit support for timestamp clock handling and hardware clock correlation.

The AUTOSAR Ethernet Driver and Ethernet Interface specifications both define Eth_GetCurrentTimeTuple as an API that reads the current time of the timestamp clock and the current time of the PTP Hardware Clock (PHC) in an atomic operation.

AUTOSAR’s Synchronized Time-Base Manager, or StbM, then builds higher-level synchronized time bases on top of such hardware-related notions.

That is extremely important in a car because application software often should not care whether the timestamp originated in a MAC, PHY, controller clock, or disciplined system time source.

What matters is that the lower software stack can correlate those clock domains precisely enough that higher layers get a stable synchronized time base.

AUTOSAR’s hardware-facing timestamp APIs and StbM’s time-tuple model are the software architecture answer to that requirement.

## 14. Cross-timestamping in AUTOSAR StbM

AUTOSAR does not always use the exact phrase “cross-timestamping” in the way Linux or some timing papers do, but the concept is clearly present.

AUTOSAR StbM manages time as tuples that correlate a time-base value with a local hardware-related reference.

In the StbM specification, the Main Time Tuple is described as [TGSync; TVSync], where TGSync is the synchronized time-base value and TVSync is the corresponding Virtual Local Time (VLT) reference.

The current time tuple is similarly [TL; TV], where TL is the local instance of global time and TV is the current virtual local time.

The Time Synchronization over Ethernet specification also defines variables such as T3VLT and T4VLT specifically for correlating StbM local time with the Ethernet hardware clock.

That is, in substance, cross-timestamping: one tuple ties one clock domain to another so time can be translated or disciplined consistently.

AUTOSAR’s newer Ethernet APIs strengthen this interpretation by explicitly offering an atomic read of the timestamp clock together with the PHC.

So if you say “AUTOSAR cross-timestamping,” the most standards-grounded way to explain it is: AUTOSAR correlates synchronized time, virtual local time, timestamp clock time, and PHC time through time tuples and atomic dual-clock reads, even if the documents do not always brand that mechanism with the single term cross-timestamping.

## 15. Stream ID in vehicles

In automotive Ethernet, stream identification is not academic.

A vehicle network often needs to know which packets belong to the same deterministic perception stream, control stream, media stream, or reliability-protected stream.

IEEE 802.1CB’s stream_handle is useful here because IEEE material says it can be used for FRER and by IEEE 802.1Q for QoS functions, including TSN. It can be derived from fields ranging from MAC and VLAN information to IP addresses, ports, and DSCP.

That makes Stream ID valuable in automotive for at least four reasons: deterministic queuing and shaping, per-stream filtering and policing, FRER replication/elimination, and diagnostics/observability.

The key caution remains the same as in the data-com case: stream_handle is not a replacement for forwarding.

It is a classification key for traffic treatment.

In a vehicle, that distinction matters because you often want one stream to receive special handling across multiple hops without redefining ordinary bridge forwarding rules.

## 16. Stream reservation in vehicles

Stream reservation is one of those areas where automotive networks are more selective than generic enterprise Ethernet.

The reservation concept itself is extremely relevant: deterministic streams with bandwidth and latency requirements should not simply compete in best-effort fashion.

SRP’s talker/listener model therefore maps naturally to in-vehicle media and control streams.

AVnu’s SRP material even has an automotive-focused presentation saying SRP automatically creates a path with guaranteed bandwidth from a talker to one or more listeners and keeps non-compliant devices from corrupting those streams.

But automotive systems are also often highly engineered and centrally integrated.

That is exactly why IEEE 802.1Qcc matters so much in vehicles: it adds centralized, hybrid, and decentralized configuration models plus a UNI and YANG-based structures for centralized control.

In practice, that means a vehicle program can keep the reservation concept while choosing a configuration style that matches an engineered in-vehicle network rather than relying purely on dynamic runtime signaling everywhere.

## 17. A realistic automotive design reading

Translating all of  this into automotive design language, the picture becomes:

- two-step timestamping is often favored because it is simpler to implement and validate in automotive systems, though one-step is supported by newer timing standards and can be valuable where hardware is ready for it

- the hardware timestamp pipeline matters because automotive timing accuracy depends on capturing time at the MAC/PHY boundary rather than in delayed software paths

- AUTOSAR StbM provides the software architecture for correlating synchronized time with local hardware time references

- Stream ID gives the network a durable way to recognize deterministic streams for reliability and QoS treatment

- SRP / Qat / Qcc give the architecture a way to reserve and configure resources for those streams, with Qcc especially relevant for centrally engineered vehicle networks.

## 18. Summary

In pure data communications, one-step means the precise Sync timestamp is carried inside Sync, while two-step means the precise timestamp is carried later in Follow_Up.

The hardware timestamp pipeline is the path from the application through the driver to the MAC/PHY timestamp point, and accuracy depends on taking timestamps as close to the wire event as possible.

Cross-timestamping means correlating two clock domains. IEEE 802.1CB uses a stream_handle to identify deterministic streams for FRER and QoS functions.

IEEE 802.1Qat SRP reserves resources for streams through talker/listener signaling, and IEEE 802.1Qcc extends that with richer stream descriptions and centralized, hybrid, or decentralized configuration models.

In the automotive context, two-step is often preferred for implementation simplicity, the hardware timestamp pipeline is essential for gPTP accuracy, AUTOSAR StbM provides tuple-based clock correlation that is effectively the cross-timestamping mechanism software uses, Stream ID underpins deterministic stream treatment, and Qcc-style reservation/configuration is often more natural than purely distributed reservation in a tightly engineered in-vehicle network.