VLAN-aware bridge-centric treatment of Forwarding Database (FDB) learning and aging along with the IEEE standards view.

## 1. The model

A VLAN-aware bridge is doing two big jobs at the same time.

First, it must answer where should this frame go?
That is the job of the Forwarding Database (FDB).

Second, it must answer what should I do when too many frames want the same output resources at the same time?
That is the job of congestion management.

So the bridge has a knowledge plane and a pressure plane:

- knowledge plane: learn source locations, look up destinations, forget stale information

- pressure plane: classify traffic, place it into queues, decide which queue gets service next, and possibly shape, police, drop, or signal congestion

The current IEEE bridge standard for this family of behavior is IEEE Std 802.1Q-2022, Bridges and Bridged Networks, with later amendments such as IEEE Std 802.1Qcz-2023 adding newer congestion features. The public IEEE 802.1 bridge YANG modules explicitly state that they model IEEE Std 802.1Q-2022 and amendments such as 802.1Qcz-2023 and 802.1Qcw-2023.

## 2. FDB learning from first principles

A bridge cannot forward intelligently unless it has learned where end stations live. In practical switch implementations, an FDB entry is naturally viewed as a {port, MAC address, VLAN} tuple. Linux switchdev documentation states exactly that, and the IEEE 802.1Q bridge model describes the filtering database as the information used by the forwarding process to decide through which ports frames should be sent.

Why does the VLAN matter? Because in a VLAN-aware bridge, the same destination Media Access Control (MAC) address is not interpreted globally first. It is interpreted in a VLAN context first. Public IEEE material also shows that each VLAN is associated with a Filtering Database Identifier (FID), meaning the standard allows a VLAN to map to a particular filtering database.

So the real lookup question is not merely:

Where is MAC B?

It is:

Where is MAC B for this VLAN or FID context?

That is the basic reason VLANs isolate Layer 2 forwarding domains.

## 3. What exactly gets learned

The bridge learns from the source address of a frame that has successfully come through ingress processing. In plain language, if a frame for VLAN 10 arrives on port P2 with source MAC A, the bridge learns that MAC A in VLAN 10 is reachable via P2.

Conceptually, the learning flow is:

- classify the frame into a VLAN

- apply ingress rules

- if accepted, use the source MAC address plus VLAN context to update dynamic forwarding information

- use destination lookup for forwarding

That is why a bridge becomes smarter over time. Unknown traffic is expensive because it tends to flood. Learned traffic is efficient because it can be forwarded to one specific port. This is also how Linux bridge offload models learned and forgotten addresses.

## 4. Independent VLAN Learning and Shared VLAN Learning

IEEE 802.1Q does not force only one learning model. The public bridge model exposes capability flags for:

- Independent VLAN Learning (IVL)

- Shared VLAN Learning (SVL)

- hybrid-capable

IVL means learning is effectively per VLAN. SVL means multiple VLANs can share a filtering database. Hybrid means both modes can exist in the implementation. The same public model also exposes the mapping of a VLAN identifier to a Filtering Database Identifier.

From a first-principles perspective:

- IVL is what most people mentally mean by “VLAN-aware learning.”
MAC A learned on VLAN 10 is not automatically valid for VLAN 20.

- SVL is useful when multiple VLANs intentionally share Layer 2 reachability knowledge.

- FID mapping is the standards mechanism behind that distinction.

So when engineers casually say “the switch learns per VLAN,” that is usually true in practice, but the standard is more general and more precise.

## 5. Dynamic entries versus static entries

IEEE 802.1Q distinguishes dynamic and static filtering entries. The public bridge model exposes counts for both, and the entry type itself is explicitly modeled as either static or dynamic. It also states that static entries can be created, deleted, and retrieved, while dynamic entries are not generally intended to be arbitrarily rewritten by management except for deletion or retrieval.

From a first-principles point of view:

- a dynamic entry is learned from observed traffic

- a static entry is configured by an administrator or control plane

- a static filtering rule can say “forward” or “filter” regardless of dynamic learning

The public IEEE 802.1Q type definitions explicitly show per-port control elements such as forward and filter for static filtering entries.

So dynamic learning answers “what do I think I have observed?” while static entries answer “what am I told to enforce?”

## 6. What aging means

Learning without forgetting is dangerous. Hosts move. Links change. Topology changes. So the bridge needs a timeout for dynamically learned information. IEEE’s public bridge model gives the key managed object directly: aging-time, defined as the timeout period in seconds for aging out dynamically learned forwarding information. Its default is 300 seconds. The Linux bridge documentation exposes the same idea operationally: the FDB entry aging time is how long a MAC address is kept after a packet has been received from that address, and the default is also 300 seconds.

So aging means:

- if the bridge keeps seeing frames from MAC A in VLAN 10, the dynamic entry is refreshed

- if it stops seeing that source for longer than the aging timer, the entry is removed

- future traffic to that destination may again become unknown unicast and therefore flood within the VLAN until relearned

This is the bridge’s way of preventing stale reachability from living forever.

## 7. What does not age

Static entries do not age out the way dynamic learned entries do.
That is exactly why static entries exist. If the network designer knows that a certain MAC or multicast group must always be treated a certain way, it should not disappear just because the timer expired. The IEEE bridge model separates static and dynamic entry accounting for precisely that reason.

## 8. A simple learning and aging walk-through

Imagine this bridge:

- Port 1: access VLAN 10

- Port 2: access VLAN 10

- Port 3: trunk VLANs 10 and 20

At time t0, Host A sends a frame on Port 1 in VLAN 10.
The bridge learns:

(MAC A, VLAN 10) → Port 1

At time t1, Host B sends to MAC A in VLAN 10.
The bridge looks up MAC A in VLAN 10 and forwards only to Port 1.

Now suppose Host A is unplugged and moved elsewhere, and no more frames are seen from MAC A on Port 1.
If the dynamic entry is not refreshed, after the aging timer expires the bridge deletes that learned entry. The next unicast frame to MAC A in VLAN 10 becomes unknown unicast again and may be flooded inside VLAN 10 until the bridge relearns the correct location. That is exactly why aging is necessary: it converts old certainty back into temporary uncertainty instead of letting the switch keep making a wrong decision forever.

## 9. Why FDB size matters

The public IEEE bridge model exposes the FDB size, meaning the maximum number of entries that can be held. This matters because large access networks, virtualization-heavy systems, or multicast-heavy environments can exhaust table resources. When the table is under pressure, the bridge cannot keep perfect knowledge of everyone, and the practical symptom is usually more unknown traffic and more flooding.

So FDB learning and aging are not only correctness mechanisms. They are also memory-management mechanisms.