Below is a first-principles explanation of priority regeneration in a VLAN-aware bridge, with the IEEE 802.1Q view made explicit.

## 1. What problem priority regeneration solves

A bridge receives frames from ports that may not mean the same thing by “priority.” Even when two devices both use a 3-bit Priority Code Point (PCP), the bridge may need to reinterpret that incoming priority before it decides which internal queue or traffic class should handle the frame. IEEE 802.1Q models priorities as values 0 through 7 and supports 1 to 8 traffic classes in a bridge.

So priority regeneration is the bridge’s way of answering:

“I have received a frame with some priority indication. What priority should this bridge use internally for forwarding and queueing decisions on this ingress port?”

That is why the standard defines a Priority Regeneration Table on a per-port basis. The public IEEE 802.1Q bridge YANG says it is “a list of Regenerated User Priorities for each received priority on each port of a Bridge,” and that the regenerated priority “may be used to index the Traffic Class Table for each input port.” It also says the default regenerated values are the same as the received user priorities.

## 2. The key distinction: regeneration is not the same as remarking

This is the most important conceptual distinction.

Priority regeneration is an ingress-side internal decision.
It tells the bridge what internal priority to use after reception.

Priority Code Point (PCP) encoding/remarking is an egress-side tagging decision.
It tells the bridge what PCP and possibly Drop Eligible Indicator (DEI) to place in the outgoing VLAN tag. IEEE’s public type model says the PCP decoding table decodes priority and drop eligibility from the received PCP, while the PCP encoding table encodes the priority and drop-eligible parameters into the outgoing VLAN tag.

So these are not the same pipeline step:

- regeneration decides internal bridge priority

- encoding/remarking decides transmitted tag values

A lot of confusion comes from mixing those two up.

## 3. Where priority regeneration sits in the bridge pipeline

A clean bridge-centric picture is:

- receive frame

- determine the received priority

- regenerate that priority if needed on this ingress port

- use the regenerated priority to index the Traffic Class Table

- place the frame into an internal traffic class / queue

- later, possibly encode a transmitted PCP/DEI on egress

The IEEE 802.1Q public models make that sequence visible. The bridge-port model exposes default-priority, priority-regeneration, pcp-selection, pcp-decoding-table, pcp-encoding-table, and use-dei, while the type model separately exposes the Traffic Class Table used to map a priority to a traffic class.

## 4. What is the “received priority”?

The bridge first has to decide what priority value the incoming frame is presenting.

For VLAN-tagged traffic, IEEE 802.1Q uses the 3-bit Priority Code Point (PCP) in the VLAN tag as the basis for quality-of-service priority signaling. The public type model describes PCP as a 3-bit field associated with 802.1Q VLAN-tagged frames, with values 0 through 7.

But the bridge does not always use the raw PCP directly. IEEE 802.1Q also provides:

- a default priority per bridge port

- a PCP decoding table

- a PCP selection mode

- optional use of the DEI bit for drop eligibility handling.

So the received priority is already a slightly richer concept than “just read the 3 PCP bits.”

## 5. The Priority Regeneration Table itself

IEEE 802.1Q’s public type model says the Priority Regeneration Table “provides the ability to map incoming priority values on a per-Port basis, under management control.” It contains eight entries, one for each incoming priority 0 through 7, and the defaults are identity mappings:

- incoming 0 → regenerated 0

- incoming 1 → regenerated 1

- …

- incoming 7 → regenerated 7.

So by default, the bridge is saying:

“I trust the incoming priority values as they are.”

But management can change that per ingress port. That means a bridge port can deliberately reinterpret traffic from a given source domain.

## 6. Why would a bridge regenerate priority?

Because the bridge may sit between different trust domains or policy domains.

Examples:

- an access port may receive PCP-marked traffic from an endpoint that is not trusted

- a boundary port may need to compress or remap priorities into the bridge’s local policy

- traffic entering from a customer side may need a different internal interpretation than traffic entering from a backbone or scheduled network side

The standard mechanism is per-port because the trust decision is usually where the frame came from, not just what tag it carried. IEEE’s model explicitly places priority regeneration under the specific bridge port.

## 7. The regenerated priority is used for traffic class selection

This is the central bridge consequence.

The bridge YANG states that the regenerated priority may be used to index the Traffic Class Table for each input port. The type model describes the Traffic Class Table as the table for a given port that maps a priority index to a traffic class, with up to eight traffic classes supported.

So priority regeneration is not cosmetic. It affects which internal queue or class gets the frame.

That is why a regenerated priority change can alter:

- latency

- queue choice

- bandwidth competition

- drop behavior under congestion

even if the frame contents otherwise remain unchanged.

## 8. First-principles example

Suppose a bridge ingress port receives traffic with these priorities:

- PCP 7 from a control endpoint

- PCP 5 from a camera

- PCP 1 from a diagnostic logger

If the Priority Regeneration Table is identity-mapped, the bridge internally keeps them as 7, 5, and 1, then uses the Traffic Class Table to map those values into traffic classes.

Now imagine the network designer does not trust the access port’s markings and configures that ingress port’s regeneration table like this:

- 7 → 5

- 5 → 4

- 1 → 0

Then the bridge is effectively saying:

- “incoming emergency/control marking is downgraded to local high”

- “incoming medium marking is compressed further”

- “incoming low marking becomes best effort”

That is priority regeneration in the simplest possible sense: incoming meaning is rewritten into local meaning before queue selection.

## 9. Priority regeneration versus PCP decoding

Another subtle distinction matters here.

The PCP decoding table associates a received PCP value with an internal priority and drop-eligible value, based on the configured PCP selection type. IEEE’s public type model says exactly that.

So a useful mental sequence is:

- the tag’s PCP is decoded into priority and drop eligibility

- then the ingress port’s regeneration table can map the incoming priority to a regenerated priority

- then the regenerated priority feeds the Traffic Class Table

In other words:

PCP decoding answers “what priority does this tag represent here?”
priority regeneration answers “what priority should this port actually use internally?”

That is why the standard keeps them as separate managed objects.

## 10. PCP selection modes

IEEE 802.1Q also defines PCP selection modes through pcp-selection-type. The public type model lists these values:

- 8P0D = 8 priorities, 0 drop eligible

- 7P1D = 7 priorities, 1 drop eligible

- 6P2D = 6 priorities, 2 drop eligible

- 5P3D = 5 priorities, 3 drop eligible.

This matters because the decoding and encoding tables depend on which PCP interpretation mode is selected. The bridge-port model says the pcp-selection object identifies the rows in the PCP encoding and decoding tables used to remark frames on that port if remarking is enabled, and its default is 8P0D.

So the full priority story is not just “0–7 in, 0–7 out.” The bridge can also interpret PCP/DEI combinations differently depending on the selected PCP mode.

## 11. Drop Eligible Indicator and drop behavior

IEEE 802.1Q lets a bridge carry drop eligibility alongside priority. The bridge-port model says that if use-dei is true, the drop_eligible parameter is encoded in the DEI bit of transmitted frames, and on reception drop_eligible becomes true either when the DEI bit is set or when the PCP decoding table says so for that received PCP value. If use-dei is false, DEI is transmitted as zero and ignored on receipt.

This means the bridge can distinguish between:

- “high priority, protect strongly”

- “same priority class, but more discardable under pressure”

That is related to congestion treatment, but it is not the same as priority regeneration. Priority regeneration changes the priority value. DEI changes the drop eligibility policy signal.

## 12. Egress encoding is a different table

After the frame has passed through the bridge, queued, and selected for transmission, the bridge may need to place a PCP and DEI into the outgoing VLAN tag. IEEE’s public type model says the PCP encoding table maps internal priority and drop-eligible parameters to the outgoing PCP field.

So the correct first-principles separation is:

- regenerated priority = what the bridge uses internally

- encoded PCP/DEI = what the bridge writes on transmission

A bridge may regenerate internally without preserving the original PCP on egress, or it may preserve it, depending on policy and configuration. That separation is built into the standard’s management objects.

## 13. Service access priority

IEEE 802.1Q also models Service Access Priority Selection and a Service Access Priority Table. The bridge-port model says this indicates whether service access priority selection is supported on a customer bridge port to request priority handling from a port-based service interface, and the type model says the Service Access Priority Table associates a received priority with a service access priority.

This is related, but it is not the same mechanism as ordinary VLAN-tag PCP regeneration. It belongs more to service-interface behavior in provider-bridge style contexts. For most ordinary VLAN-aware bridge discussions, the main concepts are still:

- default priority

- PCP decoding

- priority regeneration

- traffic class mapping

- PCP/DEI encoding.

## 14. Standards clause map you can teach from

A compact IEEE 802.1Q-oriented map is:

- 6.9.4 / 12.6.2 / 12.6.2.3
Priority regeneration and the per-port Priority Regeneration Table. The public YANG says it maps incoming priority values per port, defaulting to identity mapping.

- 6.9.3 / 12.6.2.7 / 12.6.2.9
PCP decoding and encoding. The public type model says decoding maps received PCP to priority and drop eligibility, while encoding maps priority and drop eligibility to outgoing PCP.

- 12.6.2.5.3
PCP selection types such as 8P0D, 7P1D, 6P2D, and 5P3D.

- 12.6.3 / 8.6.6
Traffic Class Table. The public type model says this maps a priority index to a traffic class for a given port.

- 12.6.2
Per-port default priority, use of DEI, drop encoding, and related bridge-port priority parameters.

## 15. Priority regeneration override table

There is also a more specialized standards provision: the Priority Regeneration Override Table. The public IEEE 802.1 FQTSS Management Information Base (Forwarding and Queuing for Time-Sensitive Streams Management Information Base) says this table contains priority-regeneration override values for the port, that rows exist for priorities associated with Stream Reservation (SR) classes, and that it maps a priority value to an override value at the SRP domain boundary. It cites 12.20.3 and 6.9.4.

That means ordinary per-port priority regeneration is the general mechanism, while the override table is a more specialized management hook for time-sensitive stream reservation boundary behavior.

## 16. The clean engineering takeaway

If you want one correct sentence to teach:

Priority regeneration is the ingress-port function by which an IEEE 802.1Q bridge maps received priority values to locally regenerated priorities before traffic-class selection. The regenerated priority is then used internally, while PCP/DEI encoding on egress is handled by separate encoding tables.

And if you want the most important practical warning:

Do not confuse “what priority the bridge believes internally” with “what PCP the transmitted frame carries.” IEEE 802.1Q models those as separate mechanisms.