First-principles explanation of bridge forwarding, filtering, and flooding in a VLAN-aware bridge.

- VLAN Bridge

A bridge is a Layer 2 device. Its job is not to look at Internet Protocol (IP) addresses. Its job is to look at Ethernet frames and decide:

- Does this frame belong to a VLAN? Which one?

- Do I know where the destination Media Access Control (MAC) address lives?

- On which ports am I allowed to send it?

That is the whole subject in one sentence.

A VLAN-aware bridge does not merely ask “what is the destination MAC address?” It asks “what is the destination MAC address within this VLAN context?” In public IEEE 802.1Q text, the filtering database effectively behaves like separate per-VLAN databases because filtering entries carry VLAN-related information, and modern bridge implementations commonly model forwarding entries as a {port, MAC, VLAN} tuple.

# 2. What forwarding, filtering, and flooding really mean

## Forwarding

The bridge sends the frame only to the specific egress port where it believes the destination station resides.

Example:

- “Destination MAC 00:11:22:33:44:55 in VLAN 10 is on port 5.”

- So the bridge sends the frame only to port 5.

## Flooding

The bridge does not know the exact destination port, or the frame is inherently one-to-many, so it sends copies out multiple ports in the same forwarding domain, usually all eligible ports in that VLAN except the ingress port. Vendors describe flooding as the mechanism used when the switching table does not contain the destination address, and they note that flood traffic is not sent back out the interface where it arrived.

## Filtering

The bridge deliberately does not relay the frame on a given port, or on any port at all.

Filtering can happen for several reasons:

- the frame belongs to a VLAN not allowed on the ingress port

- the destination is known to be on the same port it came from

- the candidate egress port is not a member of that VLAN

- the port is blocked by spanning tree

- static or policy-based rules say “do not forward”

So filtering is not only “security filtering.” It is the general act of suppressing relay when bridge rules say “not allowed.”

# 3. The minimum information a VLAN-aware bridge needs

For each received frame, the bridge cares about:

- ingress port

- source MAC address

- destination MAC address

- VLAN Identifier (VID), if tagged

- whether the frame is untagged, priority-tagged, or VLAN-tagged

A VLAN-aware bridge classifies each frame into a VLAN first. Public 802.1Q text states that the Port VLAN Identifier (PVID) provides the VLAN identifier for untagged and priority-tagged frames received on that port. The Linux bridge model says the same in operational terms: pvid means untagged ingress frames are assigned to that VLAN, and untagged means that VLAN is transmitted without a tag on egress.

In short:

No VLAN classification, no correct forwarding decision.

# 4. Think of the bridge as a pipeline

A useful mental model is this:

Ingress classification → ingress filtering → source learning → destination lookup → egress filtering → transmit

# 5. Step 1: ingress VLAN classification

When a frame enters a port, the bridge first decides which VLAN it belongs to.

## Case A. The frame is tagged with VLAN 10

Then the bridge says: “This belongs to VLAN 10.”

## Case B. The frame is untagged

Then the bridge uses the port’s PVID.

Example:

- Port 1 has PVID 10

- An untagged frame arrives on port 1

- The bridge internally treats it as belonging to VLAN 10

## Case C. The frame is priority-tagged with VLAN ID 0

That means the frame carries priority bits but not a real VLAN membership. Public 802.1Q text says such frames are classified using the port’s PVID. Linux bridge and switchdev documentation likewise says priority-tagged traffic is handled like untagged traffic for PVID purposes.

So the bridge first answers:

“What VLAN context am I operating in for this frame?”

# 6. Step 2: ingress filtering

Now that the bridge knows the VLAN, it asks:

“Is this ingress port allowed to receive frames for that VLAN?”

If not, the frame is dropped immediately.

The 802.1Q YANG model published with IEEE 802.1Q-2022 describes ingress filtering as discarding a frame received on a port that is not in the member set associated with that frame’s VLAN, when ingress filtering is enabled. Linux switchdev documentation gives the practical behavior: with VLAN filtering enabled, frames arriving with a VLAN ID not programmed into the bridge VLAN table must be dropped, and if no PVID exists on the ingress port, untagged and priority-tagged frames must be dropped.

This is a very important distinction:

- VLAN classification tells you which VLAN the frame claims to belong to

- ingress filtering tells you whether that claim is acceptable on this port

Example:

- Port 2 is an access port for VLAN 10 only

- A tagged VLAN 20 frame arrives on port 2

- The bridge drops it

That is filtering before forwarding even begins.

# 7. Step 3: source MAC learning

If the frame survives ingress checks, the bridge learns from the source MAC address.

Suppose a frame arrives:

- ingress port = 1

- source MAC = A

- VLAN = 10

The bridge records something like:

(MAC A, VLAN 10) → port 1

This is why a bridge becomes smarter over time. Juniper’s documentation explains that learning creates source MAC entries so later traffic can be forwarded directly rather than flooded. Public 802.1Q text also explains that MAC information is learned independently for each VLAN or set of VLANs.

This also explains why the same MAC can, in principle, appear in different VLAN contexts without confusing the bridge. The VLAN is part of the lookup context.

# 8. Step 4: destination lookup

Now the bridge examines the destination MAC address in that VLAN context.

There are four big cases.

## Case 1. Known unicast

The bridge has an entry for that destination in the forwarding database.

Example:

- lookup: (MAC B, VLAN 10) → port 3

- result: send only to port 3

That is forwarding in the strict sense.

## Case 2. Unknown unicast

The destination is an individual MAC address, but the bridge has no entry yet.

Then the bridge floods the frame to all eligible ports in that VLAN except the ingress port. Vendors describe this as the way a bridge discovers the location of a destination whose MAC address is not yet in the switching table.

## Case 3. Broadcast

Destination MAC is all ones.

This is inherently one-to-many traffic, so the bridge floods it within the VLAN.

## Case 4. Multicast

If the bridge has no multicast-specific knowledge, it often floods within the VLAN. If multicast snooping is enabled, it can forward only to ports that actually have interested receivers. Linux bridge documentation describes this using a multicast forwarding database rather than blind flooding.

# 9. Step 5: egress filtering

Even after destination lookup identifies candidate output ports, the bridge is still not finished.

For each candidate port, it asks:

“Am I allowed to emit this VLAN on this port?”

If a port is not a member of VLAN 10, then a VLAN 10 frame cannot go there.

So a flood is not “send everywhere.” It is:

send to every eligible egress port in the same VLAN forwarding domain, except the ingress port, subject to state and policy

This is why VLANs confine flood traffic. Traffic is not flooded across the whole switch. It is flooded only inside the relevant VLAN or bridge domain. Juniper’s bridging documentation explicitly notes that packets whose source and destination are in the same VLAN are forwarded only within that VLAN, and filtering limits traffic from leaking to other segments.

# 10. Step 6: tag or untag on egress

After deciding the egress port, the bridge decides how the frame should leave that port.

## Access-style egress

If that port is configured to send VLAN 10 untagged, the bridge strips the tag before transmission.

## Trunk-style egress

If that port carries VLAN 10 as tagged, the frame leaves with the VLAN tag.

Linux bridge documentation exposes exactly these ideas through pvid and untagged configuration semantics.

So a frame can be:

- received untagged

- internally classified as VLAN 10

- forwarded across the bridge as VLAN 10

- emitted tagged on one port and untagged on another

That is perfectly normal.

# 11. A concrete bridge example

Consider this switch:

- Port 1: Host A, access VLAN 10, PVID 10

- Port 2: Host B, access VLAN 10, PVID 10

- Port 3: Host C, access VLAN 20, PVID 20

- Port 4: uplink trunk carrying VLANs 10 and 20 tagged

Now let us trace frames.

## Example 1. First frame from A to B

Host A sends an untagged frame to Host B.

### Ingress

- frame enters port 1 untagged

- PVID on port 1 = 10

- bridge classifies frame into VLAN 10

### Ingress filtering

- port 1 is allowed for VLAN 10

- frame accepted

### Learning

- learn (MAC A, VLAN 10) → port 1

### Destination lookup

- (MAC B, VLAN 10) not yet known

### Result

- unknown unicast flood in VLAN 10

- send to port 2 and port 4

- do not send to port 3 because port 3 is VLAN 20

- do not send back to port 1

### Egress

- out port 2 as untagged

- out port 4 as tagged VLAN 10

That is flooding, but only inside VLAN 10.

## Example 2. B replies to A

Now Host B replies.

### Learning

- bridge learns (MAC B, VLAN 10) → port 2

### Destination lookup

- bridge already knows (MAC A, VLAN 10) → port 1

### Result

- forward only to port 1

Now the bridge is doing forwarding, not flooding.

## Example 3. A sends to C

Host A in VLAN 10 sends to Host C in VLAN 20 by destination MAC.

Even if the bridge somehow knows MAC C on port 3, the VLAN context is different.

Lookup is done in VLAN 10 context, not in VLAN 20 context.

So:

- if no entry exists for (MAC C, VLAN 10), the frame may flood within VLAN 10 only

- it still will not go to port 3 because port 3 is not an egress member of VLAN 10

This is the heart of VLAN isolation.

# 12. What “filtering” looks like in real life

Filtering is easiest to understand with examples.

## Example A. Same-port filtering

Suppose the bridge has learned:

- (MAC B, VLAN 10) → port 1

Then a frame arrives on port 1 destined for MAC B in VLAN 10.

The bridge does not send it back out port 1. It filters that relay attempt.

## Example B. Wrong VLAN on ingress

Port 2 is access VLAN 10 only. A tagged VLAN 20 frame arrives.

Ingress filtering drops it.

## Example C. Port blocked by spanning tree

Even if the forwarding database says the destination is on that port, the bridge does not relay through a blocked path.

## Example D. Static filtering entry

A management rule may say that certain destination MAC addresses or groups must not be forwarded on specific ports. Public 802.1Q text describes static filtering entries as specifying, per outbound port, whether frames matching a MAC and VLAN specification are forwarded or filtered.

So filtering is not one special case. It is a family of “relay suppressed” cases.

# 13. Why flooding is necessary at all

A beginner often asks:

“Why not always forward directly?”

Because the bridge is not born knowing where every MAC address lives. It must learn by observing source addresses. Until it learns, it has only one safe option for unknown destinations:

flood inside the relevant VLAN

That is why learning and flooding always come together. Learning reduces future flooding. Vendors explicitly describe learning as the mechanism that avoids continued flooding once source locations are known.

# 14. The shortest possible rule set

A VLAN-aware bridge functional behaviour can be reduced to these rules:

- Classify the frame into a VLAN

  - explicit tag, or

  - ingress port PVID for untagged / priority-tagged traffic

- Apply ingress filtering

  - if the ingress port is not allowed for that VLAN, drop

- Learn the source

  - record source MAC against ingress port in that VLAN context

- Look up the destination

  - known unicast → forward to one port

  - unknown unicast → flood within VLAN

  - broadcast → flood within VLAN

  - multicast → flood or selectively forward depending on multicast knowledge

- Apply egress filtering

  - only ports that belong to that VLAN and are operationally eligible

- Tag or untag on egress

  - according to the egress port’s VLAN presentation rules

# 15. Summary

- Forwarding = targeted Layer 2 delivery based on a destination MAC lookup in the correct VLAN context.

- Flooding = replication to all eligible ports in the same VLAN when exact destination location is unknown or the traffic is inherently one-to-many.

- Filtering = deliberate non-relay because ingress rules, VLAN membership, topology state, or forwarding rules prohibit transmission.

A VLAN-aware bridge answers three questions in order:

- Which VLAN is this frame in?

- Where does that destination live in this VLAN?

- Which ports are permitted to carry it?