Bridge forwarding/filtering/flooding;

FDB learning and aging;

Congestion management;

Priority regeneration;

VLAN relay,

Trunk vs access ports; S-VLAN/C- VLAN

## 1. The real job of a bridge

An Institute of Electrical and Electronics Engineers (IEEE) 802.1Q bridge is not just a box that “sends packets onward.”

In the standard’s own summary, IEEE 802.1Q-2022 specifies how the Media Access Control (MAC) Service is supported by bridged networks, the principles of operation of those networks, and the operation of MAC Bridges and Virtual Local Area Network (VLAN) Bridges, including management, protocols, and algorithms.

A bridge filters and forwards frames between segments based on the destination MAC address.

That sounds simple until you ask what a switch really has to decide for every incoming frame. It must determine at least four things: what VLAN context the frame belongs to, whether the source address should be learned, whether the destination is known or unknown, and whether the frame should be forwarded to one port, many ports, or no ports at all. In a VLAN-aware bridge, forwarding decisions depend on both the destination MAC address and the VLAN tag, not just the MAC address alone.

So the right mental model is not “the switch looks up a MAC address.” The right model is:

classify → learn → look up → filter → forward or flood.

## 2. What forwarding, filtering, and flooding actually mean

Forwarding implies

The bridge has enough information to select one or more specific egress ports for the frame.

In normal unicast forwarding, that means the destination MAC address is known in the relevant VLAN context, so the frame is sent only where it should go.

The bridge exists to filter and forward frames based on destination MAC address, and with VLAN filtering enabled, thus bridge behaviour is explicitly based on destination MAC address plus VLAN tag.

Filtering implies

The frame is intentionally not sent to a given port, or not admitted at all.

Filtering can happen for multiple reasons:

the frame arrived on a port that is not allowed to carry that VLAN,

the destination is not reachable through that port,

the port is in a state where data forwarding is disabled, or

policy says the frame type is not acceptable there.

IEEE 802.1Q bridge-management models expose exactly these control points through acceptable frame types and ingress filtering.

A port can be configured to admit

only VLAN-tagged frames,

only untagged and priority-tagged frames, or

all frames;

Ingress filtering can then discard traffic whose VLAN is not valid on that port.

Flooding is

The fallback behavior when the bridge does not know a single correct egress destination or when the traffic is defined to reach many receivers.

Unicast traffic for which there is no Forwarding Database (FDB) entry can be flooded, and that multicast traffic with no matching multicast database entry can also be flooded.

In provider-style Q-in-Q operation, at the service-VLAN level: broadcast, unknown unicast, and multicast traffic is forwarded to all members in the service VLAN.

So the summary is:

- forward when the bridge knows the right place

- filter when the bridge knows a place is wrong or forbidden

- flood when the bridge does not yet know one correct destination, or the traffic is inherently one-to-many.

## 3. Why flooding is necessary even in a “smart” switch

Flooding as not a sign that switching has failed.

A bridge learns where destinations live only after it has first seen them as sources.

A port can learn source MAC addresses from received traffic, and it also says that dynamic FDB entries are refreshed by incoming traffic.

That means the first time device A sends to device B, the bridge may know where A is but not yet where B is.

Unknown unicast flooding is therefore the bridge’s safe discovery behavior.

This leads to a very important principle:

bridges learn from source addresses, not from destination addresses.

That is why flooding and learning are paired behaviours.

Flooding is what the bridge does before it has enough knowledge.

Learning is what reduces future flooding.

Once the bridge later sees B transmit a frame, it can learn B’s location and stop flooding future traffic destined for B.

## 4. The Forwarding Database: what it really contains

The FDB is the bridge’s memory of where it believes destinations are located.

IEEE 802.1Q management models describe the filtering database as containing the information used by the forwarding process to decide through which ports frames should be forwarded.

The same model exposes an aging time, the maximum size of the FDB, and counts of static and dynamic entries.

It also shows that filtering entries carry both a MAC address and a set of VLAN identifiers, which is the standards-facing way of saying that forwarding knowledge is VLAN-aware.

That last point is essential. In a VLAN-aware bridge, the FDB is not just “MAC address → port.” It is closer to:

destination MAC address + VLAN context → forwarding/filtering decision.

From an implementation point of view: an offloaded Layer 2 forwarding entry is a tuple of port, MAC, VLAN.

## 5. Dynamic learning, static entries, and aging

The bridge can populate the FDB in more than one way.

A dynamic entry is learned from live traffic. IEEE 802.1Q bridge models call these dynamic or learned filtering entries. Learning is controlled per port, and that incoming traffic can refresh dynamic entries.

A static entry is installed deliberately and does not depend on passive learning from traffic. IEEE bridge models expose the count of static entries separately from dynamic ones, which reflects this exact distinction.

Aging is what stops dynamic learning from becoming stale. IEEE 802.1Q bridge models set the default timeout for aging out dynamically learned forwarding information to 300 seconds. Without aging, a bridge could keep sending traffic toward a port long after the device had moved, been unplugged, or been reattached elsewhere. Aging is therefore not cosmetic clean up; it is what keeps learned topology information believable over time.

So the logic is:

- dynamic learning makes the bridge adaptive

- static entries make it intentional

- aging stops yesterday’s truth from becoming today’s bug.

## 6. VLAN relay: what the bridge is actually relaying

Now bring VLANs into the picture.

A VLAN-aware bridge is not relaying “raw Ethernet” in one flat universe. It is relaying frames within a VLAN context. Once VLAN filtering is enabled, forwarding decisions depend on destination MAC address and VLAN tag together. IEEE 802.1Q YANG models expose the Port VLAN Identifier (PVID) as the primary default VLAN identifier assigned to a bridge port, which is the standard way to express how an untagged incoming frame can be classified into a VLAN context.

That leads to the correct mental model for VLAN relay:

- the ingress port classifies the frame into a VLAN context

- the bridge learns the source address in that VLAN context

- the bridge looks up the destination in that VLAN context

- the bridge forwards or floods only within the rules of that VLAN context

- the egress port may transmit the frame tagged or untagged depending on port configuration and role.

So “VLAN relay” is not just tag carriage. It is the whole bridge behavior of classifying, learning, forwarding, filtering, and flooding inside a chosen VLAN scope.

## 7. Trunk ports and access ports: useful terms, but not the deepest model

The terms access port and trunk port are very widely used, but they are better understood as operational shorthand than as the deepest 802.1Q architectural language.

An access port can be defined as a logical interface configured to accept untagged packets and forward them within a specified bridge domain. A trunk port can be defined as a logical interface configured to accept packets tagged with any VLAN identifier in a configured list, and optionally untagged packets as well.

The standards-facing way to think about the same behaviour is through:

- the Port VLAN Identifier (PVID) for default VLAN assignment on ingress

- acceptable frame types

- ingress filtering

- egress tagging or untagging behavior.

So the practical mapping is:

- an access-port style configuration usually means “untagged ingress is mapped to one default VLAN, and egress for that VLAN is typically sent untagged”

- a trunk-port style configuration usually means “multiple VLANs are carried on one link, usually tagged, under explicit per-port admission rules.”

Access versus trunk should be understood in the following way:

access and trunk are deployment concepts; PVID, acceptable frame types, ingress filtering, and egress tagging are the underlying bridge mechanisms.

## 8. Customer VLAN and Service VLAN: what the two tag types mean

A Customer VLAN (C-VLAN) is the ordinary customer-facing 802.1Q VLAN tag type using EtherType 81-00. A Service VLAN (S-VLAN) is the service-provider-facing tag type using EtherType 88-A8, originally introduced in IEEE 802.1ad and later incorporated into IEEE 802.1Q. IEEE 802.1Q YANG type definitions spell this out directly.

That means the two tag types do not just differ by number. They also differ by role.

- a C-VLAN usually expresses customer-side VLAN meaning

- an S-VLAN usually expresses provider-side transport or service context.

IEEE provider-bridge modelling says that a Provider Bridge is associated with a single S-VLAN component and zero or more C-VLAN components. That is a very compact but very powerful architectural statement: the provider bridge is built around a service-VLAN transport view that can carry multiple customer-VLAN meanings.

## 9. What Q-in-Q is really doing

Q-in-Q, usually means carrying one 802.1Q tag inside another. The operational reason very clearly is: service providers may need to carry traffic for multiple customers that use overlapping VLAN IDs, or they may want to bundle many customer VLANs into a smaller set of service VLANs. They do this by adding another outer 802.1Q tag so the original customer tag remains intact while the provider forwards on the basis of the service tag.

So the deep idea is not “two tags because two is better than one.” The deep idea is:

the inner tag preserves customer meaning; the outer tag carries provider transport meaning.

In this Q-in-Q model, traffic is learned from S-VLANs, not C-VLANs, and that broadcast, unknown unicast, and multicast traffic is forwarded to all members in the S-VLAN. That is exactly the kind of fact that helps us  understand what the outer service context really controls: it is not just decoration, it becomes the actual relay scope of the provider bridge.

## 10. Congestion management: what it means inside a bridge

Now shift from “where does the frame go?” to “what happens when too many eligible frames want the same egress resource?”

That is congestion management in a bridge. In practice, this means some combination of classification, metering, queue selection, scheduling, and selective dropping. The 802.1Q family explicitly includes this kind of machinery. The IEEE 802.1Qav amendment says bridges can provide guarantees for time-sensitive traffic by using per-priority ingress metering, priority regeneration, and timing-aware queue draining algorithms. It also says VLAN-tag-encoded priorities are allocated to segregate frames among controlled and non-controlled queues.

The current 802.1Q data models expose the same internal logic in management form. A bridge port has:

- a priority regeneration table

- Priority Code Point (PCP) decoding and encoding tables

- a traffic class table

- a transmission selection algorithm table.

So the congestion-management pipeline is best understood like this:

- decode incoming priority from the VLAN tag or port defaults

- possibly regenerate or remap that priority

- map priority into an internal traffic class

- apply a transmission-selection algorithm to decide service order and shaping

- if needed, use Drop Eligible Indicator (DEI) information to decide what may be discarded under pressure.

That is a much better explanation than saying “the switch has queues.” Queues are only one stage of a bigger chain.

## 11. Priority regeneration: why the bridge might rewrite priority internally

Priority regeneration is one of those phrases that sounds mysterious until you say what it really means.

IEEE 802.1Q YANG definitions state that the priority regeneration table provides the ability to map incoming priority values on a per-port basis, under management control. The bridge-port model then says the regenerated priority value may be used to index the traffic-class table for each input port, and that the default regenerated priorities are the same as the received user priorities.

So priority regeneration means:

the priority that arrives on the wire does not have to be the priority the bridge uses internally.

That is extremely useful because different ports may sit at the boundary between different priority conventions or administrative domains. The bridge may decide that a received priority 6 on one ingress port should remain 6, while on another ingress port it should be demoted or promoted before traffic-class lookup. IEEE 802.1Qav explicitly includes priority regeneration as part of the bridge’s ability to support bounded-latency time-sensitive streams.

In other words, priority regeneration is how the bridge says:

“I understand what priority you claimed, but here is the priority I will actually use inside this bridge.”

## 12. Priority Code Point decoding and encoding

The Priority Code Point (PCP) is the three-bit priority field in an 802.1Q VLAN tag. IEEE 802.1Q YANG types define it as a three-bit field referring to the class of service associated with a tagged frame, with values from 0 to 7. The same models define both a PCP decoding table and a PCP encoding table. The decoding table maps an incoming PCP to internal priority and drop-eligible state; the encoding table maps internal priority and DEI back into an outgoing PCP field.

This is a very important bridge concept because it shows that PCP is not simply carried through untouched. A bridge can decode the external tag into an internal representation, apply regeneration and policy, and then re-encode the outgoing tag according to egress rules. That is how priority semantics can be preserved, translated, or normalized across a network.

To the question, “Does the switch just preserve the incoming PCP bit-for-bit?” the correct answer is:

not necessarily; it may decode, regenerate, and re-encode according to bridge policy.

## 13. Traffic classes and transmission selection

Once the bridge has an internal priority, it still has to decide how that priority is served on egress.

IEEE 802.1Q models a traffic class table that maps priority into internal traffic classes, and a transmission selection algorithm table that assigns a scheduling or shaping algorithm to each traffic class. The YANG type definitions explicitly identify algorithms such as strict priority, credit-based shaper, enhanced transmission selection, and asynchronous traffic shaping.

This is where congestion management becomes real. Under light load, every frame eventually gets out and the details seem unimportant. Under heavy load, the switch’s behavior is determined by these mappings:

- which traffic class the frame lands in

- which algorithm serves that class

- whether lower-priority or drop-eligible frames may be delayed or discarded.

Hence congestion management should be seen as a policy chain, not as “there are eight priorities.” The priorities are only inputs. The actual behavior comes from the class mapping and the egress selection algorithm.

## 14. Drop eligibility and what gets sacrificed first

Congestion management is not only about who goes first. It is also about who may be sacrificed when buffers are under stress.

IEEE 802.1Q bridge-port models describe use-dei, meaning whether drop-eligible state is encoded in the DEI bit of transmitted frames and decoded from received frames, and drop-encoding, which can require that frames marked drop-eligible be encodable on that port or else be discarded rather than transmitted. The PCP decoding and encoding tables also model the association between priority, PCP, and drop-eligible state.

That means DEI is not just another bit in the tag. It is the bridge’s way of carrying the idea:

“This frame belongs to this VLAN and this priority level, but it is also eligible to be dropped if congestion becomes severe.”

That is the bridge between VLAN priority marking and real congestion behavior.

## 15. Putting access ports, trunk ports, and VLAN relay together

On an access-port style interface, an untagged frame usually arrives, the ingress port assigns it a default VLAN through the PVID, the bridge learns the source in that VLAN, and future forwarding remains inside that VLAN’s scope. Juniper describes an access interface exactly as one that accepts untagged packets and forwards them within a specified bridge domain.

On a trunk-port style interface, the incoming frame usually arrives tagged, possibly with one of several allowed VLAN identifiers. Juniper describes a trunk interface as one configured to accept packets tagged with any VLAN ID in a configured list. IEEE 802.1Q bridge management expresses the standards-side equivalent using acceptable frame types, ingress filtering, and port membership.

In both cases the bridge does the same core job: relay the frame within the correct VLAN context. The difference is mainly how the VLAN context is supplied:

- by the port’s default classification rules

- or by the tag that arrived on the wire.

## 16. Putting Service VLAN and Customer VLAN together

With Customer VLANs, the bridge is usually thinking in customer-local VLAN meanings. With Service VLANs, the provider network can carry those customer VLANs across its own transport domain. IEEE’s provider-bridge model exposes direct customer VLAN identifier (cvid) and service VLAN identifier (svid) mapping elements, while Juniper’s documentation explains that Q-in-Q tunneling prepends the service VLAN tag and carries the original customer tag transparently.

The outer service tag changes the provider’s relay scope.
It does not erase the customer’s original VLAN meaning.

So in a provider bridge, forwarding, filtering, and flooding often happen in the S-VLAN domain, while the customer-facing semantics of the inner C-VLAN are preserved for the edge of the provider network.

## 17. Common misunderstandings

“Flooding is always bad.”
No. Flooding is the bridge’s normal fallback for unknown unicast and some multicast situations before sufficient destination knowledge exists.

“The Forwarding Database is just a flat MAC table.”
No. In a VLAN-aware bridge, forwarding entries are tied to VLAN context as well as MAC address.

“Access and trunk are the deepest standard concepts.”
Not really. They are useful operational terms, but the standards-facing machinery is PVID, acceptable frame types, ingress filtering, and egress tagging behavior.

“Priority Code Point just passes through unchanged.”
Not necessarily. A bridge can decode incoming PCP, regenerate priority, map to traffic classes, and then encode PCP again on egress.

“Congestion management just means bigger buffers.”
No. In IEEE 802.1Q it fundamentally involves metering, priority handling, traffic-class mapping, scheduling or shaping, and drop-eligibility policy.

“Service VLAN and Customer VLAN are just two names for the same kind of tag.”
No. A C-VLAN uses EtherType 81-00 and carries customer-side VLAN meaning; an S-VLAN uses EtherType 88-A8 and carries provider-side service context.

## 18. Final take-away

A frame arrives, the port classifies it into a VLAN context, the bridge learns the source, looks up the destination in the VLAN-aware Forwarding Database, filters forbidden paths, and either forwards to known egress ports or floods within the permitted scope if the destination is still unknown. Congestion management then decides how urgently the frame is served, using regenerated priorities, traffic classes, transmission-selection algorithms, and drop-eligibility policy. Access ports and trunk ports are operational shorthands for different ways of supplying VLAN context, while Customer VLANs and Service VLANs let a provider bridge carry customer traffic without destroying the customer’s own tag meaning.