The principles of bridging, flooding, priority handling, and VLANs come from general Ethernet. But an automotive switch is not being used in an open office network. The Institute of Electrical and Electronics Engineers (IEEE) 802.1DG automotive profile is explicitly scoped for secure, highly reliable, deterministic-latency, automotive in-vehicle bridged IEEE 802.3 Ethernet networks, and IEEE’s profile work also states that a profile selects the subset of base-standard features needed for an application and can make optional features effectively mandatory for conformance. That is the key mindset shift: in a vehicle, the question is not “which switching features exist,” but “which of them must be implemented, constrained, or hardened so the network behaves predictably every time the car boots.”

the automotive perspective is about how much freedom the switch should allow, which behaviors should be deterministic rather than opportunistic, and which policies should be enforced in hardware rather than left to software hope. Real automotive switch silicon reflects exactly that. For example, the NXP SJA1105P/Q/R/S family is marketed not as a generic bridge but as a safe and secure automotive Ethernet switch with ISO 26262 Automotive Safety Integrity Level A compliance, while the newer NXP SJA1110 family adds Time-Sensitive Networking (TSN) support, packet inspection, denial-of-service prevention, functional-safety features, and Wake-over-Ethernet support for automotive Electronic Control Units.

## 1. Bridge forwarding, filtering, and flooding in a vehicle

In a general Ethernet explanation, forwarding means “send known unicast traffic to the learned port,” filtering means “do not send it where it should not go,” and flooding means “send unknown or broadcast-like traffic more widely.” In a vehicle, those same principles still apply, but the tolerance for uncontrolled behavior is much lower because flooding and weak filtering directly increase latency variability, diagnostic noise, and failure-propagation scope. IEEE 802.1DG’s automotive scope is explicitly about bounded latency, reliability, security, and maintainability, so the bridge behavior should be tuned for those goals rather than for broad plug-and-play flexibility.

This means filtering becomes more important than flooding. On paper, unknown unicast flooding is a normal bridge fallback. In a vehicle, it is something you generally want to constrain. The IEEE 802.1Q-2022 bridge model includes acceptable frame types and ingress filtering precisely so a port can admit only the classes of frames it is meant to carry, and discard frames whose VLAN Identifier (VID) is not valid for that port. The YANG model for 802.1Q-2022 says a port can be set to admit only VLAN-tagged frames, only untagged and priority-tagged frames, or all frames, and that ingress filtering can discard a frame if the port is not in the member set associated with that frame’s VID. In automotive terms, that is the standards-backed mechanism for turning a switch from a permissive relay into a bounded, topology-aware policy device.

Real automotive switch silicon shows this clearly. In the NXP SJA1105P/Q/R/S, the VLAN Lookup (VLAN_LU) block determines forwarding limitations and tagging or untagging options, the Layer 2 Address Lookup (L2ADDR_LU) block uses destination MAC address plus VLAN ID as the lookup key, and the same lookup hardware can also be configured as a filter that determines the subsequent action for frame processing. The Layer 2 Forwarding (L2_FORW) block then maintains destination-port vectors for unicast, broadcast, and unknown multicast traffic. That is exactly the automotive version of bridge behavior: forwarding is not one flat MAC-table decision, but a chain of VLAN-aware admission, lookup, filtering, and controlled replication.

From an automotive design perspective, I would therefore recommend this interpretation: for critical control and sensing VLANs, forwarding should be as explicit as possible and flooding should be minimized; for diagnostics, plant, engineering, and service traffic, some broader fallback behavior is acceptable; and for security-sensitive ports, ingress filtering and frame-class restrictions should be aggressive. That recommendation is an engineering inference, but it follows directly from the automotive profile’s emphasis on deterministic latency and from the hardware support automotive switches already expose for filtering, per-port forwarding limits, and policy-driven frame handling.

## 2. Forwarding Database learning and aging in a vehicle

In ordinary switch theory, the Forwarding Database (FDB) learns from source addresses, stores dynamic entries, and ages them out later. That still works in a vehicle, but the automotive context changes how much trust you place in dynamic learning. The NXP SJA1105P/Q/R/S explicitly supports both dynamically learned and statically configured entries, and it says dynamically learned entries can be configured to time out. The same device also supports either shared or independent address learning. IEEE 802.1Q’s management model similarly exposes separate counts for static and dynamic registration entries in the FDB.

That matters because a vehicle network is usually a closed, engineered topology, not a campus where arbitrary new hosts appear. So while dynamic learning is convenient, it should not be the only mechanism you rely on for critical paths. For example, traffic between a zonal controller and a central compute node, or between an Advanced Driver Assistance Systems sensor path and its consumer, is often better treated as engineered connectivity rather than as something that must be rediscovered opportunistically after every aging event. This is an inference, but it is well aligned with 802.1DG’s “profile” philosophy and with the fact that automotive switches such as the SJA1105 expose static entries and programmable timeout behavior rather than only generic dynamic learning.

In automotive practice, that leads to a more nuanced FDB strategy. Dynamic learning is still very useful for noncritical traffic, service laptops, maintenance flows, and ordinary edge behavior. But for fixed, safety-relevant, or tightly bounded traffic paths, you often want one of three things: static forwarding entries, long and carefully controlled aging, or filtered ports where address learning is restricted. The SJA1105 feature list even calls out hardware support related to disabling address learning as part of its security feature set. That is a strong hint that in automotive, learning is not always a universally good thing; sometimes it must be constrained.

Aging also has a different meaning in a vehicle. In an office, a 300-second default aging timer is often fine. In a car, the question becomes: should a quiet but valid Electronic Control Unit disappear from the switch’s learned view just because it has not transmitted for a while? On some ports, probably not. On others, especially for maintenance or transient tools, dynamic aging is perfectly reasonable. So the automotive lesson is that one global aging policy is usually too blunt. The switch should let the system architect choose where dynamic aging is beneficial and where it creates unnecessary uncertainty. IEEE 802.1Q’s FDB model and NXP’s timeout-capable learned entries both support that kind of selective policy thinking.

## 3. Congestion management in a vehicle

In generic networking, congestion management is often described as queues, drops, and maybe shaping. In an automotive network, that is too shallow. The real question is: when multiple traffic classes contend for the same egress resource, which traffic is protected, which traffic is delayed, and which traffic may be discarded without violating system intent? IEEE 802.1DG explicitly frames automotive Ethernet around bounded latency and reliability, and the 802.1DG project material specifically calls out “latency and congestion loss” and “traffic separation” as core topics for the automotive profile.

That is why an automotive switch needs more than “eight queues.” It needs ingress policing, priority remapping, traffic-class mapping, and egress scheduling that correspond to the vehicle’s traffic contract. The NXP SJA1105P/Q/R/S is a very good real-world example. Its Layer 2 Policing (L2_POLICE) block can limit transmission rate for any of the eight priority levels and for broadcast traffic at each port, dropping noncompliant traffic and recording flags and counters. Its Layer 2 Forwarding (L2_FORW) block performs priority remapping and egress-queue priority mapping. Its Transmit MAC (TXM) supports eight priority queues and strict-priority scheduling. Its Audio Video Bridging (AVB) block implements credit-based traffic shaping according to IEEE 802.1Qav and provides high-resolution timestamping for IEEE 802.1AS and IEEE 1588 version 2 operation.

That feature mix is much closer to what automotive congestion management should look like. You do not just want to buffer harder; you want to stop nonconformant traffic at ingress, preserve the intended priority semantics through the switch, and shape or schedule the protected classes so they see bounded interference. On higher-end or time-sensitive variants, the same SJA1105 family goes further with virtual-link policing and virtual-link forwarding for time-triggered or rate-constrained traffic, where noncompliant frames are dropped and correctly timed traffic can be held until its transmit trigger fires. That is a very automotive-style distinction: not all traffic is just “best effort but with a bigger priority number.” Some of it is contract traffic that must be validated against time or rate rules.

So the automotive interpretation of congestion management is this: implement hardware-enforced admission and shaping first, then use buffers and queues as the consequence of those policies, not as the whole strategy. If you leave congestion behavior mostly to software or to generic best-effort queueing, the network may still function, but it stops being an engineered automotive fabric and becomes just an overloaded Ethernet box. IEEE 802.1DG’s profile concept and NXP’s actual silicon block structure both point in the direction of policy-driven, not merely buffer-driven, congestion control.

## 4. Priority regeneration in a vehicle

In ordinary Ethernet, the Priority Code Point (PCP) in the VLAN tag is often treated as if it were the “priority truth.” In a vehicle, that is usually the wrong assumption. Different edge devices, different software stacks, supplier modules, diagnostics tools, and development hosts may all mark priorities differently. The switch therefore needs the ability to normalize or regenerate the effective priority on ingress according to the vehicle architect’s intent, not just the sender’s claim. IEEE 802.1Q-2022’s bridge model is explicit here: the priority regeneration table is a list of regenerated user priorities for each received priority on each port, and the regenerated priority may then index the traffic-class table for that port.

That is an extremely automotive-relevant feature. It means the switch can say, in effect, “I see that you arrived marked as priority 6, but on this port, in this vehicle, that class is going to be treated internally as something else.” The same 802.1Q-2022 model also includes PCP selection, PCP decoding, PCP encoding, DEI use, and the traffic-class and transmission-selection algorithm tables. In other words, the bridge can decode the incoming PCP, map it into internal priority and drop-eligibility meaning, regenerate it if needed, map it to a traffic class, and then later re-encode PCP for egress.

Automotive silicon mirrors this. The NXP SJA1105P/Q/R/S advertises per-port priority remapping and eight configurable egress queues per port, and its forwarding block explicitly handles priority remapping and egress-queue priority mapping. That is exactly the hardware manifestation of priority regeneration.

In automotive terms, this feature is important because the switch should enforce an original equipment manufacturer defined traffic contract, not merely reflect the markings that happen to appear at the edges. I would treat priority regeneration as essential wherever the network mixes safety-relevant traffic, infotainment traffic, diagnostics, engineering access, and service flows. In that setting, trusting incoming PCP blindly is too permissive. The switch should be the normalization point. That recommendation is an inference, but it follows directly from 802.1Q’s regeneration machinery and from the automotive need for bounded, reproducible behavior.

## 5. VLAN relay in a vehicle

From a general networking perspective, VLAN relay just means the bridge forwards frames inside their VLAN context. In a vehicle, VLAN relay becomes the main tool for making one physical Ethernet fabric behave like several different logical networks with different safety, timing, and exposure properties. IEEE 802.1DG’s automotive profile material explicitly discusses traffic separation as a core content area, and VLANs are the most basic Layer 2 mechanism for that separation.

That means a VLAN-aware automotive switch should use relay behavior to separate, for example, control, sensor, diagnostic, engineering, logging, and infotainment traffic onto distinct forwarding scopes even when they share the same hardware fabric. The SJA1105P/Q/R/S directly supports this style of design: its VLAN_LU block determines forwarding limitations and tagging or untagging, and its feature list includes 4096 VLANs, egress tagging/untagging on a per-VLAN basis per port, and optional double-tagging support.

So in an automotive perspective, VLAN relay should not be described as “office-style segmentation transplanted into the car.” It should be described as traffic-scope control inside a zonal or centralized in-vehicle fabric. That is a much more useful way to explain why VLANs matter in automotive switch silicon.

## 6. Trunk versus access ports in a vehicle

The terms trunk port and access port come from operational networking practice, not from the deepest standards language, but they are still useful if we reinterpret them correctly for vehicles.

An access-port style link is a leaf-facing link where the endpoint is treated as belonging to one default VLAN context. Juniper’s documentation describes an access interface as one that accepts untagged packets and forwards them inside a specified bridge domain. IEEE 802.1Q-2022’s bridge model expresses the same idea more formally through the Port VLAN Identifier (PVID), which it defines as the primary default VID assigned to a specific bridge port, along with acceptable-frame and ingress-filtering controls.

A trunk-port style link is a multi-VLAN carry link. Juniper describes a trunk interface as one configured to accept packets tagged with VLAN IDs from a configured list; its tagged-VLAN documentation also says trunk ports multiplex traffic among a number of VLANs and use the tag to determine origin and forwarding behavior.

In a vehicle, that usually maps like this. Links from a switch to a simple edge Electronic Control Unit (ECU), actuator controller, or sensor aggregator are often access-like: the switch classifies untagged ingress into a default VLAN and can send traffic back untagged if the endpoint is not VLAN-aware. Links between switches, between a zonal controller and a central computer, or between a gateway and a high-function host are often trunk-like: multiple VLANs ride the same physical link because the endpoint on the far side understands tagged traffic or needs several isolated domains at once. This is an engineering inference, but it follows directly from the standards mechanisms and from how trunk and access behavior are defined operationally.

The key automotive caution is that you should not copy enterprise habits blindly. In an office, it is common to tolerate mixed tagged and untagged behavior on trunks for convenience. In a vehicle, I would generally prefer explicit tagging on backbone links and simple default-VLAN behavior on leaf links, because ambiguity is expensive during debugging and validation. Standards support mixed cases, but deterministic design usually favors clearer boundaries. That is an engineering recommendation rather than a quoted rule.

## 7. Service VLAN and Customer VLAN in a vehicle

The Customer VLAN (C-VLAN) and Service VLAN (S-VLAN) concepts come from provider bridging, but they can still be useful in automotive if you use them for the right reason. IEEE 802.1Q type definitions identify the C-VLAN tag type with EtherType 81-00 and the S-VLAN tag type with EtherType 88-A8. Juniper’s Q-in-Q documentation explains the operational model: providers can segregate customers that use overlapping VLAN IDs or bundle multiple customer VLANs into a single service VLAN by adding an outer tag while preserving the inner customer tag.

Inside a car, this is usually not the first feature you need. Many vehicles will be well served by single-tag VLANs. But S-VLAN/C-VLAN becomes useful when you want the backbone to carry a zone-local or supplier-local VLAN namespace without collapsing it into the central network’s own VLAN numbering. In other words, the inner tag can preserve the local meaning of a subnetwork, while the outer tag identifies how that traffic is transported across the vehicle backbone. That is an inference from provider-bridge behavior, but it is exactly the kind of reuse pattern a complex zonal vehicle architecture can benefit from.

Automotive silicon support matters here. The SJA1105P/Q/R/S feature list includes optional double-tagging support, and its diagnostics and forwarding feature list includes filtering rules for forwarding, retagging, tunneling, and double-tagging, plus frame mirroring and retagging support. That means the silicon is already designed for the kind of tag manipulation that makes S-VLAN/C-VLAN useful.

So my automotive recommendation would be: treat C-VLAN/S-VLAN as an advanced backbone isolation tool, not as the default answer everywhere. Use it when you want to preserve local VLAN meaning across a transport domain, or when one part of the vehicle should carry another part’s preexisting VLAN structure transparently. Do not use it merely because the switch happens to support double-tagging. That is an engineering judgment, but it is the disciplined way to apply provider-style concepts inside an engineered in-vehicle network.

## 8. What this means as an automotive implementation checklist

If I reduce all of the above to “what the switch should really implement for automotive,” it becomes this.

For bridge forwarding/filtering/flooding, the switch should have strong VLAN-aware forwarding limits, ingress filtering, and preferably policy filtering beyond simple destination lookup, because deterministic automotive fabrics benefit from constraining flood scope and rejecting wrong traffic early. The SJA1105 and SJA1110 families are good examples: the former exposes explicit VLAN lookup, address lookup, forwarding vectors, frame mirroring, retagging, and filtering rules, while the latter adds Ternary Content-Addressable Memory (TCAM) based frame inspection, denial-of-service prevention, safety features, and TSN support.

For FDB learning and aging, the switch should support both dynamic learning and static provisioning, with configurable timeout and the option to constrain or disable learning on protected ports. That is much better suited to an engineered vehicle network than relying entirely on open-ended dynamic discovery.

For congestion management, the switch should provide ingress policing, priority remapping, multiple egress queues, shaping, counters, and where needed, time-aware or flow-aware treatment. In automotive, congestion management must enforce the traffic contract, not just absorb bursts until buffers fill.

For priority regeneration, the switch should normalize ingress priority markings to an original equipment manufacturer defined internal scheme instead of trusting every incoming PCP blindly. IEEE 802.1Q-2022 gives the machinery for that directly, and automotive switches such as the SJA1105 expose the corresponding remapping behavior in silicon.

For VLAN relay and trunk/access behavior, the switch should make leaf links simple and backbone links explicit. Use access-like behavior for simple edge ECUs, use trunk-like behavior where multiple isolated domains must traverse the same link, and use strong admission rules rather than enterprise-style convenience defaults.

For S-VLAN/C-VLAN, the switch should support them if the architecture needs nested VLAN semantics across a backbone, but this should be an intentional architectural tool rather than a default configuration style.

## 9. Summary

In a general data network, these features are mostly about efficient Layer 2 behavior.

In a vehicle, the same features become about:

- constraining uncertainty

- preserving timing

- limiting fault spread

- making behavior reproducible

- supporting safety, security, and diagnosis in hardware.

That is the real automotive reinterpretation of bridge forwarding, learning, congestion handling, priority regeneration, and VLAN structure.