Traffic policing (token bucket, dual-rate);

IEEE 802.1Qci PSFP;

IEEE 802.1Qcr ATS;

Storm control;

Traffic mirroring (RSPAN);

IGMP snooping

## 1. Why these topics belong together

These features all answer one practical question: what should a switch do when traffic is too fast, too bursty, too broad, or simply not meant to go everywhere?

Traffic policing and IEEE 802.1Qci Per-Stream Filtering and Policing (PSFP) are about enforcing traffic contracts and blocking bad behavior.

IEEE 802.1Qcr Asynchronous Traffic Shaping (ATS) is about smoothing traffic per hop so latency stays bounded without needing a network-wide transmission schedule.

Storm control is a coarse mechanism to suppress runaway broadcast, multicast, or unknown-unicast traffic.

Traffic mirroring, including Switched Port Analyzer (SPAN) and Remote Switched Port Analyzer (RSPAN), is about observability. Internet Group Management Protocol (IGMP) snooping is about learning multicast receiver interest so multicast data is forwarded only where it is needed.

## 2. Start with the basic distinction: policing versus shaping

Traffic policing and traffic shaping often use the same measurement idea, but they react differently.

Both mechanisms use traffic descriptors and token-bucket style measurement to detect contract violations, but a policer typically drops or re-marks excess traffic, while a shaper typically buffers and delays excess traffic to smooth the flow.

That is the cleanest foundational distinction:

policing is enforcement, while shaping is smoothing.

A useful intuition is this.

If traffic reaches a regulator faster than allowed, a policer acts like a bouncer at the door: it refuses or downgrades excess traffic.

A shaper acts like a metered ramp: it lets the traffic through later, at a more controlled rate.

The two can work together, but they are not the same thing.

Shaping and policing can be deployed together where policers enforce an average rate limit while still allowing a bounded burst.

## 3. Token bucket from first principles

A token bucket is the most common mental model for both policing and shaping.

It is a bucket that receives tokens at a fixed rate up to a maximum depth;

Each token represents permission to send or receive some amount of data, and when enough tokens are present the traffic is conforming.

The bucket has a burst size, a mean rate, and a time interval, and that packets consume a number of tokens proportional to packet size.

If not enough tokens exist, the packet is either delayed by a shaper or dropped/remarked by a policer.

The deepest point is this: a token bucket controls average rate while allowing bursts.

That is why it is more useful than a simple fixed-rate gate.

The token bucket allows a specified burst before discard starts, and the largest burst a source can send is roughly proportional to the bucket size.

So if you want to allow short bursts but still enforce a long-term traffic contract, token buckets are the natural tool.

## 4. Single-rate token-bucket policing

The Request for Comments (RFC) 2697 Single Rate Three Color Marker (srTCM) is a classic single-rate policing model.

It meters traffic against one Committed Information Rate (CIR) and two burst parameters: Committed Burst Size (CBS) and Excess Burst Size (EBS).

RFC 2697 says packets are marked green if they fit within the committed bucket, yellow if they exceed the committed burst but fit within excess allowance, and red otherwise.

The same RFC also describes the meter in terms of two token buckets, C and E, that share the common committed rate.

This is an important fundamental point because “single-rate policer” does not mean “only one bucket.”

In the srTCM model, there is one replenishment rate but two token buckets representing two levels of burst allowance.

That is how the policer distinguishes conforming, exceeding, and violating traffic. In practice, a device can use those colors to decide whether to pass, deprioritize, or drop traffic.

## 5. Dual-rate token-bucket policing

The classic dual-rate model is RFC 2698 Two Rate Three Color Marker (trTCM). It meters traffic with two different rates: Peak Information Rate (PIR) and Committed Information Rate (CIR), along with their associated burst sizes Peak Burst Size (PBS) and Committed Burst Size (CBS). RFC 2698 says the meter uses two token buckets, P and C, with the peak bucket enforcing the peak limit and the committed bucket distinguishing committed versus excess traffic. Packets that exceed peak become red; packets within peak but above committed become yellow; packets within committed become green.

This makes dual-rate policing fundamentally different from single-rate policing. A single-rate policer says, roughly, “here is your long-term contract plus some excess allowance.” A dual-rate policer says, “here is your committed contract, but also here is an absolute peak you may never exceed.” RFC 2698 even gives the service interpretation: red traffic can be discarded, yellow can be best-effort, and green can get the best assurance.

## 6. What policing is really buying

In pure data communications, policing exists to stop one source or stream from overdriving shared resources. It protects links, queues, and downstream traffic classes from bursts or sustained rates that exceed what was agreed. Policing and shaping are both contract-enforcement mechanisms, and RFC 3290’s token-bucket discussion says token buckets determine whether traffic still conforms to one or more specified rates. That is why token-bucket policers are so common at ingress boundaries.

So the short data-com answer is: a policer is a guardrail for admissible traffic behavior. It is not about “making traffic nicer” in a vague way. It is about deciding whether arriving traffic still fits the contract it claimed to have.

## 7. IEEE 802.1Qci Per-Stream Filtering and Policing from first principles

IEEE 802.1Qci Per-Stream Filtering and Policing (PSFP) takes the general policing idea and applies it at the level of individual traffic streams inside a bridge. The official IEEE 802.1Qci page says the amendment lets a bridge perform frame counting, filtering, policing, and service-class selection for a frame based on the particular stream to which it belongs, and that these functions help detect and mitigate disruptive transmissions by other systems. The standard itself says it adds enhancements to the forwarding process that support per-stream filtering and policing.

The important idea here is that ordinary policing is often too coarse. It may police per port or per class. PSFP is more selective: it can identify one specific stream and apply filtering or metering decisions specifically to that stream rather than punishing everything on the same port. That is why the feature is called per-stream, not just “per-port policing.”

## 8. The three core building blocks of PSFP

PSFP can be divided into three conceptual building blocks: stream filter, stream gate, and meter. PSFP protects against bandwidth violation, malfunctioning, and malicious attacks; the stream filter provides filters and counters, the stream gate can be open or closed and can be time-scheduled, and the meter is based on the bandwidth profile of Metro Ethernet Forum (MEF) 10.3 with marking behavior. The standard’s management model also explicitly includes a Stream Filter Instance Table, stream-gate control state machines, and a Flow Meter Instance Table.

That gives you the cleanest mental model of PSFP:

- the stream filter decides which stream a frame belongs to and which stream-specific rule set applies

- the stream gate decides whether frames of that stream are currently allowed through

- the flow meter decides whether the stream is still conforming to its rate and burst contract.

## 9. What the stream gate really adds

The stream gate is what makes PSFP more powerful than a plain policer. A gate can force all frames to be dropped when the operational gate state is closed, and it also allows operational and administrative stream-gate control lists. The gate can be open or closed and can even be time-scheduled. That means PSFP is not only about rate conformance; it can also say “this stream is only valid during certain times or in certain phases of operation.”

That is a major conceptual step. A token bucket says “are you too fast?” A stream gate can also say “even if you are within rate, you are not allowed right now.” This makes PSFP much more expressive than plain ingress rate limiting.

## 10. What the flow meter really adds

The flow meter is where token-bucket style contract enforcement shows up inside PSFP. IEEE 802.1Qci’s management objects include values such as FlowMeterCBS, FlowMeterEIR, FlowMeterEBS, colorBlind/colorAware, and DropOnYellow. In other words, PSFP can incorporate familiar burst and excess-rate logic directly into a per-stream enforcement block.

So PSFP is best understood as a stream-aware ingress policy chain: classify the stream, optionally gate it, meter it, and then decide whether to forward, mark, or drop. That is much richer than “drop packets above X megabits per second on this port.”

## 11. IEEE 802.1Qcr Asynchronous Traffic Shaping from first principles

IEEE 802.1Qcr Asynchronous Traffic Shaping (ATS) addresses a different problem. The official IEEE 802.1Qcr page says the amendment specifies procedures and managed objects for bridges and end stations to perform asynchronous traffic shaping over full-duplex links with constant data rates. The standard itself says ATS can be modeled as a layer of shaped First In, First Out (FIFO) queues merged into per-traffic-class FIFO queues in transmission ports.

The key word is asynchronous. ATS is not built around one network-wide synchronized transmission calendar. Instead, it reshapes traffic hop by hop using local scheduler state. A good summary would be that ATS provides zero congestion loss without time synchronization and smooths traffic patterns by reshaping per hop while prioritizing urgent traffic over relaxed traffic.

## 12. How ATS works conceptually

The deepest concept in ATS is the eligibility time. The standard says ATS schedulers maintain a group eligibility time and assign eligibility times to frames, and that these eligibility times are updated based on parameters such as Committed Burst Size, Committed Information Rate, Maximum Residence Time, frame arrival times, and frame lengths. It also says that the organization of ATS schedulers into groups results in a non-decreasing ordering of eligibility times, which permits FIFO queueing inside a group.

So the clean mental model is this: when a frame arrives, the ATS scheduler does not ask only “which priority queue are you in?” It also computes when you become eligible to leave. Frames are then released according to those eligibility times, not just according to raw arrival order or static priority alone. That is why ATS is a shaping mechanism, not merely a queue selector.

## 13. ATS versus simple shaping and scheduling

ATS sits conceptually between plain weighted scheduling and fully time-aware scheduled traffic. It is more disciplined than “serve queues by weight,” because it assigns per-frame eligibility based on rate contracts. But it is less dependent on global time than a network-wide scheduled gate plan. The official standard describes ATS as asynchronous shaping over full-duplex constant-rate links, and the IEEE tutorial explains its attraction as per-hop reshaping without time synchronization.

ATS gives each frame a controlled earliest-send time based on its stream contract, then merges those shaped streams into the ordinary port queues.

## 14. Storm control from first principles

Storm control is not an IEEE bridging standard feature in the same way that 802.1Qci and 802.1Qcr are. It is a widely implemented vendor switch feature. It can be  defined as a way to prevent disruptions on Ethernet interfaces caused by a broadcast, multicast, or unknown-unicast traffic storm, and says it works by monitoring the level of incoming traffic of those classes over a measurement interval and suppressing traffic when thresholds are exceeded.

The reason storm control exists is that some traffic types naturally spread more widely than ordinary known unicasts. Broadcast traffic is meant to reach many receivers. Unknown unicast and unresolved multicast can also be widely replicated. If one faulty host, loop, or attack source causes these classes to explode, the switch can suffer queue pressure and the rest of the network can become unusable very quickly. Storm control is the coarse brake pedal for that scenario.

The important caution is that storm control is coarse. It usually works on classes like broadcast, multicast, and unknown unicast at the ingress port, not on application-aware flow semantics. It is therefore complementary to fine-grained tools like PSFP, not a replacement for them. It is supported for those broad traffic classes and on ingress.

## 15. Traffic mirroring and why RSPAN is different

Traffic mirroring means making a copy of traffic for observation without changing the forwarding of the original traffic. SPAN sessions, local or remote, allow you to monitor traffic on one or more ports or VLANs and send the monitored traffic to one or more destination ports. That is the basic mirroring concept.

RSPAN, or Remote Switched Port Analyzer, is not an IEEE standard feature; it is a vendor term. RSPAN consists of a source session, an RSPAN VLAN, and a destination session on another device. Source traffic is mirrored into the RSPAN VLAN, carried over normal trunk links, and then collected on the destination switch and sent out of a destination port for analysis. The original VLAN ID is overwritten by the RSPAN VLAN ID in the RSPAN transport path.

So the clean distinction is:

- local SPAN mirrors traffic to an analyzer port on the same device

- RSPAN transports mirrored traffic across the network using a dedicated mirror VLAN to reach a remote analyzer point.

## 16. IGMP and IGMP snooping from first principles

IGMP, the Internet Group Management Protocol, is the IPv4 host-to-router protocol used to report multicast group membership. RFC 3376, which specifies IGMP version 3, says IGMP is used by IPv4 systems to report their IP multicast group memberships to neighboring multicast routers, and that version 3 adds source filtering.

That immediately creates a switch-side opportunity. If a Layer 2 switch can observe IGMP membership reports and router queries, it can learn which ports actually have interested multicast receivers instead of flooding multicast traffic everywhere. That is IGMP snooping. RFC 4541 describes best-current-practice recommendations for IGMP and Multicast Listener Discovery (MLD) snooping switches and says the snooping functionality is separated into a control section for IGMP forwarding and a data section for multicast data forwarding.

## 17. What a snooping switch actually listens for

RFC 4541 says a snooping switch should forward IGMP Membership Reports only to ports where multicast routers are attached, not to ports where only hosts are attached. It also says the switch can build its knowledge of multicast-router ports by listening to router advertisements and to the arrival ports of IGMP queries. RFC 4541 further says the switch must not rely only on Group Leave messages to age out forwarding state; it should implement a membership-timeout mechanism like router-side IGMP does.

That gives the right conceptual model:

- hosts tell routers what multicast groups they want

- routers ask membership questions with IGMP queries

- the snooping switch watches that exchange and builds Layer 2 multicast forwarding state from it

- that way, multicast data goes only to ports with interested receivers and router-facing paths.

## 18. Why IGMP snooping is useful and why it can fail

IGMP snooping is useful because it shrinks the multicast flood domain. If a switch did not snoop, unresolved or unmanaged multicast would often be treated much more like flooding. RFC 4541 warns that incorrect handling can either flood groups too broadly or prune traffic incorrectly, and it specifically discusses compatibility problems when a snooping switch does not understand the IGMP version in use. It even recommends disabling snooping in some mixed-version situations rather than doing the wrong thing.

That is an important lesson: IGMP snooping is not the multicast protocol itself. It is a switch optimization built by watching the protocol. When it understands the control protocol correctly, it is powerful. When it misunderstands it, it can become worse than plain flooding.

# Automotive perspective and context

## 19. PSFP in automotive networks

In automotive Ethernet, PSFP becomes much more than a neat standards feature. The IEEE automotive Ethernet tutorial says IEEE 802.1Qci PSFP provides protection against bandwidth violation, malfunctioning, and malicious attacks, and it explicitly breaks the mechanism into stream filter, stream gate, and meter. The same tutorial shows the automotive use directly: if an Engine Electronic Control Unit (ECU) exceeds a predefined rate, ingress policing on switches can block it in a fail-silence style, provide local or distributed notification, and support fallback behavior.

This is exactly why PSFP matters in a vehicle. Ordinary queueing protects traffic classes from each other only indirectly. PSFP lets the switch say, “this exact stream has violated its contract, so I will stop or downgrade that stream without punishing unrelated traffic.” In a zonal or central-compute vehicle architecture, that is much more useful than relying only on coarse per-port rate limits. NXP’s SJA1110 automotive switch family reflects that design priority by advertising up to 1024 streams, each with IEEE 802.1Qci-based policing capabilities, plus up to 4096 policers for Layer 2 based streams.

## 20. ATS in automotive networks

The same IEEE automotive tutorial presents Asynchronous Traffic Shaping as one of the tools for deterministic low latency in a shared network. It says ATS provides zero congestion loss without time synchronization, smooths traffic patterns by reshaping per hop, and prioritizes urgent traffic over relaxed traffic. That is a very automotive way to think about ATS: it is attractive when you want better latency control than plain weighted queueing, but you do not want to depend on a strict network-wide time schedule.

That matters because vehicles often carry mixed traffic: hard real-time or near-real-time flows, moderate-latency control flows, background diagnostics, and infotainment-type data. ATS is useful in that environment because it can reshape bursty streams at each bridge hop and bound residence time without requiring the full planning overhead of a time-aware gate schedule everywhere. The standard’s own model of shaped FIFOs, scheduler groups, and eligibility times is exactly the sort of mechanism that fits a mixed-criticality full-duplex switched fabric.

## 21. Storm control in automotive networks

Storm control is not a Time-Sensitive Networking-grade deterministic tool, and it is not a substitute for PSFP. In automotive terms, it is better thought of as a coarse safety and survivability backstop for broad Layer 2 traffic classes such as broadcast, multicast, and unknown unicast. Since the IEEE automotive tutorial explicitly treats PSFP as protection against malfunctioning and malicious behaviour, it is reasonable to see storm control as a second, less selective layer of defence for traffic explosions that do not map cleanly onto one preconfigured stream. That conclusion is an inference from the generic purpose of storm control and the automotive purpose of PSFP.

In a vehicle, that makes storm control most useful on ports where discovery, diagnostics, maintenance tooling, or other non-stream-engineered traffic could otherwise create broad replication pressure. It is far less elegant than per-stream policing, but it is also much easier to use as a blunt containment tool when something goes badly wrong.

## 22. Traffic mirroring in automotive networks

Traffic mirroring becomes extremely important during vehicle bring-up, validation, fault reproduction, and field debugging. Automotive switch silicon reflects that. The NXP SJA1105P/Q/R/S datasheet explicitly lists frame mirroring and retagging for enhanced diagnostics, and the NXP SJA1110 documentation likewise lists mirroring of traffic for enhanced diagnostics along with rich counters and status flags.

From an automotive perspective, local mirroring is usually the essential requirement. Engineers need a way to copy traffic to a capture point without perturbing the operational forwarding path. The concept of RSPAN can still be useful, especially on development benches or larger in-vehicle backbones, because it lets a mirrored copy travel to a remote analyzer across normal trunk links. But RSPAN itself is vendor terminology rather than an IEEE automotive standard, so it should be treated as an implementation option, not as a mandatory automotive Ethernet feature.

## 23. IGMP snooping in automotive networks

IGMP snooping is only relevant when the vehicle architecture actually uses IPv4 multicast distribution. If it does, then snooping can be valuable because it prevents multicast traffic from being flooded through the whole switched fabric. If it does not, then IGMP snooping is irrelevant no matter how good the switch implementation is. That is the first automotive lesson: snooping is workload-dependent, not universally required. The generic RFC model still applies: the switch watches IGMP membership reports and queries, learns where multicast listeners and multicast-router-facing ports are, and constrains multicast data forwarding accordingly.

In an automotive design that uses IP multicast for one-to-many data distribution, snooping can be quite valuable because unnecessary multicast flooding increases bandwidth pressure and timing variability. But in vehicles, many high-value flows are not simple “ordinary IPv4 multicast” in the office-network sense, so the architect should not assume that IGMP snooping is the multicast answer for every in-vehicle stream. That part is an engineering conclusion based on the role of IGMP itself.

## 24. What would one prioritize in an automotive switch

In pure data communications, all of these features are useful in the right places. In the automotive context they  can be ranked as follows.

For deterministic protection against misbehavior, prioritize PSFP and hardware policers. The IEEE automotive tutorial and NXP silicon both point directly in that direction.

For bounded latency without fully scheduled networking everywhere, ATS is highly valuable on full-duplex switched backbones carrying mixed-criticality traffic.

For gross Layer 2 fault containment, storm control is worth having, but it should be treated as a coarse safeguard rather than the main traffic-governance tool.

For observability, traffic mirroring is essential. Automotive switch vendors advertise it for a reason.

For multicast optimization, IGMP snooping is useful only if the vehicle actually uses IP multicast on those links. Otherwise, it is just another box to configure and validate.

## 25. Short recap

In the data-communications view, token-bucket policing enforces traffic contracts, dual-rate policers add a peak-rate ceiling, PSFP adds stream-aware filtering, gating, and metering, ATS reshapes traffic per hop using eligibility times, storm control suppresses broad Layer 2 traffic storms, RSPAN carries mirrored copies across a dedicated mirror VLAN, and IGMP snooping listens to multicast membership control traffic so multicast data is not flooded unnecessarily.

In the automotive view, PSFP is the precise defense against bandwidth violation and malfunctioning streams, ATS is a practical way to smooth mixed-criticality traffic without global time schedules, storm control is a coarse containment fallback, mirroring is central to diagnostics, and IGMP snooping matters only if the in-vehicle workload actually uses IP multicast.