Firewall/ACL filtering;

802.1Q counter taxonomy;

MDIO/SPI/YANG management access;

Firmware update strategy;

IEEE 802.1Qcw YANG introduction

## 1. Start with the real layering problem

Firewall / Access Control List filtering is about deciding which frames or packets are allowed through the device at all.

IEEE 802.1Q counters are about observing what the bridge actually did with traffic. Management Data Input/Output (MDIO), Serial Peripheral Interface (SPI), and YANG are three very different kinds of management access: MDIO is a low-level register-access bus for Ethernet physical layers, SPI is a chip-level host interface often used to configure or bootstrap switches, and YANG is a data modeling language for describing management data, not a pin-level interface. Firmware update strategy is a lifecycle topic: how the device changes safely over time. IEEE 802.1Qcw matters because it expands the model-driven management side of 802.1Q into more advanced bridge capabilities such as scheduled traffic, frame pre-emption, and Per-Stream Filtering and Policing (PSFP).

A good way to keep them straight is this:

- filtering decides what traffic may pass

- counters tell you what the bridge actually saw and did

- MDIO and SPI let software touch the device’s register-level control plane

- YANG, usually carried over Network Configuration Protocol (NETCONF) or RESTCONF Protocol (RESTCONF), lets software manage the device through a model-driven management plane

- firmware update strategy controls how the device itself changes safely.

## 2. Access Control Lists first, then firewalls

An Access Control List (ACL) is the simpler concept. ACL processing as an ordered list of rules: incoming traffic is compared against the entries in sequence, matching stops at the first hit, and if no rule matches, there is an implicit deny at the end. The same document shows that ACL rules can match not only source and destination addresses, but also ports and protocol-specific fields.

That means the right mental model for an ACL is:

an ACL is an ordered permit/deny rule list that usually makes a decision from packet or frame header fields, one packet at a time.

A firewall is broader. Firewalls operate according to predefined rules, but also notes that stateful inspection is a more advanced technology that monitors the state of network connections and makes decisions with session context, not just isolated packet headers. It also mentions deep packet inspection, which looks beyond headers into packet contents.

So the clean distinction is:

- an ACL is usually a stateless header-based filter

- a firewall can add state awareness and sometimes deeper payload awareness.

That is why switches often expose ACL-style filtering in hardware, while “firewall” usually implies a richer, more stateful policy function.

## 3. What ACL-style filtering means inside a switch

Inside a switch or bridge, ACL filtering is not the same thing as normal destination lookup. Ordinary forwarding asks, “where should this frame go?” ACL filtering asks, “should this frame be allowed at all, and if so, should it be permitted, dropped, mirrored, remarked, or classified differently?” Rules are evaluated in order and can match addresses, ports, and protocol types. That ordered, first-match model is the key to how hardware packet filters are usually reasoned about.

So in a data-communications context, ACL filtering is best understood as a policy decision plane layered on top of the ordinary forwarding plane. Destination lookup may say a frame could be forwarded; the ACL may still say it must be dropped, mirrored, or handled specially. That is the conceptual bridge between “switching” and “security filtering.”

## 4. IEEE 802.1Q counter taxonomy: what to count, and why

IEEE 802.1Q does not hand you a pretty marketing taxonomy, but its YANG data models expose a very practical one. In the bridge-port statistics grouping, the model includes counters such as frame-rx, octets-rx, frame-tx, octets-tx, discard-inbound, forward-outbound, discard-lack-of-buffers, discard-transit-delay-exceeded, delay-exceeded-discards, mtu-exceeded-discards, and discard-on-error. Those names are already a useful taxonomy because they separate simple traffic volume from filtering outcomes, resource exhaustion, and pathological forwarding failures.

A very useful operational taxonomy is therefore:

- volume counters: how many frames and octets were received or transmitted

- forwarding/filtering outcome counters: how many valid frames were forwarded or filtered

- resource and pathology counters: how many frames were dropped for lack of buffers, excessive transit delay, excessive size, or forwarding/transmission errors.

That is a much better mental model than “the switch has some counters.” It tells you which questions the counters answer. Volume counters tell you load. Filtering and forward counters tell you policy effect. Buffer, delay, and error counters tell you where the forwarding pipeline is under stress or misbehaving.

## 5. Why the 802.1Q counters matter more than generic interface counters

Generic network interfaces often expose only broad receive and transmit counters plus a few error totals. The 802.1Q bridge counters are more useful because they are tied to bridge behavior. For example, discard-inbound is explicitly defined as the count of received valid frames that were discarded, meaning filtered, by the forwarding process. forward-outbound counts frames forwarded to the associated MAC Entity. discard-lack-of-buffers and discard-transit-delay-exceeded tell you whether the problem is resource pressure or delay violation, not just “something failed.”

So if you are diagnosing a bridged network, this taxonomy gives you a path:

- “Is traffic arriving?” → frame-rx, octets-rx

- “Is it leaving?” → frame-tx, octets-tx

- “Was it dropped by policy?” → discard-inbound

- “Was it dropped because the switch ran out of room?” → discard-lack-of-buffers

- “Was it dropped because it sat too long?” → discard-transit-delay-exceeded / delay-exceeded-discards

- “Was it malformed or impossible to transmit?” → mtu-exceeded-discards or discard-on-error.

That is why I would call this a counter taxonomy rather than a random list.

## 6. MDIO: what it is, and what it is not

MDIO stands for Management Data Input/Output. It is the standardized management interface used to access Ethernet Physical Layer (PHY) registers. The Clause 45 MDIO text says it is an extension of the two-signal MDIO interface from Clause 22, and that its purpose is to access more device registers while preserving logical compatibility with Clause 22. It also says Clause 45 uses MDIO Manageable Devices (MMDs) so one management entity can reach up to 32 PHYs, each with up to 32 MMDs and up to 65,536 registers per MMD. Texas Instruments’ MDIO application note likewise says the serial management interface is compatible with IEEE 802.3 Clause 22 and Clause 45 and is used to interrogate and control PHYs over a shared two-wire bus.

So MDIO is best understood as low-level PHY management access. It is where software configures link speed, duplex, auto-negotiation, and other PHY state, or reads status and diagnostics back out of the physical layer. It is not how you push a full switch policy model, and it is not how you normally manage a whole bridge feature set. It is the PHY-side register doorway.

## 7. Clause 22 versus Clause 45, in one clean picture

Clause 22 is the simpler, older register access model. Clause 45 extends it because modern PHYs need a much larger and more structured register space. The Clause 45 text says the extension exists specifically to access more registers, that Clause 22 uses an ST code of 01, and that Clause 45 adds extended access with an ST code of 00. Texas Instruments’ note gives the practical engineer’s view: the Clause 22/45-compatible management interface provides access to the standard register set and the extended register space for status and configuration.

So the clean rule is:

- Clause 22: smaller, direct PHY management space

- Clause 45: larger, structured PHY management space with MMD addressing.

## 8. SPI: why switches often use it instead of MDIO for switch control

SPI, or Serial Peripheral Interface, is not an IEEE Ethernet management standard. It is a general chip-to-chip host interface that many switch devices use for configuration and bootstrap. NXP’s SJA1105 automotive Ethernet switch datasheet says directly that switch configuration is performed via an SPI interface. The later SJA1105P/Q/R/S revision also says the family exposes an SPI-slave interface. The SJA1110 fact sheet shows both QSPI and SPI in the block diagram, alongside the switch’s integrated Arm Cortex-M7 core.

This leads to a very important conceptual point:

MDIO and SPI are not competitors in the same layer.
MDIO is the standard low-level access path for PHY management. SPI is a common host interface for switch configuration, switch tables, bootstrapping, or firmware/image loading. A real switch system can easily use both at once: SPI for the switch itself, MDIO for attached PHYs.

The Clause 45 MDIO text even helps explain why this is reasonable: it says that where no physical embodiment of MDIO exists, an equivalent mechanism to access registers is recommended. That reinforces the architectural point that what matters is controlled register access, not blind loyalty to one physical management bus everywhere.

## 9. YANG, NETCONF, and RESTCONF: what each one actually is

YANG is a data modeling language. RFC 7950 says YANG is used to model configuration data, state data, Remote Procedure Calls (RPCs), and notifications for network management protocols. It also says YANG was originally designed for NETCONF and is now also used with other protocols such as RESTCONF.

NETCONF is the protocol side. RFC 6241 says NETCONF provides mechanisms to install, manipulate, and delete the configuration of network devices, uses Extensible Markup Language (XML) encoding, and realizes its operations as RPCs.

RESTCONF is another protocol side. RFC 8040 says RESTCONF is an HTTP-based protocol that provides a programmatic interface for accessing data defined in YANG, using the datastore concepts defined in NETCONF.

So the right mental stack is:

- YANG = the model of what can be configured or observed

- NETCONF = an RPC-style protocol for operating on that model

- RESTCONF = an HTTP-based protocol for operating on that model.

That is why “MDIO versus SPI versus YANG” is the wrong comparison. MDIO and SPI are chip/control buses. YANG is a management data model.

## 10. IEEE 802.1Qcw: where it fits in the YANG story

IEEE 802.1Qcw-2023 is not the first YANG-related amendment in the 802.1Q family. The IEEE 802.1Qcw page says 802.1Qcp-2018 introduced a YANG data model for a subset of bridging capabilities, 802.1Qcx-2020 further developed YANG for Connectivity Fault Management (CFM), and 802.1Qcw-2023 specifies YANG data models for bridges and bridge components with the capabilities of scheduled traffic, frame preemption, and Per-Stream Filtering and Policing (PSFP). The published ieee802-dot1q-bridge.yang module also says its 2023 revision was published as part of IEEE 802.1Qcw-2023.

So the clean introduction is:

- 802.1Qcp gave 802.1Q its first major bridge YANG foundation

- 802.1Qcx extended that into Connectivity Fault Management

- 802.1Qcw extended the YANG story into more advanced Time-Sensitive Networking-related bridge features.

That matters because once you manage bridges in a model-driven way, advanced features such as scheduled traffic and frame preemption need the same kind of model-driven visibility and configuration as classic VLANs and bridge ports.

## 11. Firmware update strategy: the data-communications fundamentals

A robust firmware update strategy for a managed switch or bridge has one overriding goal: never trade a successful update path for an unrecoverable brick. The principles are well established across embedded networking systems: keep a trusted boot path separate from the updatable application image, authenticate firmware before execution, stage new software safely, verify it before activation, and preserve rollback if activation fails. NXP’s SJA1110 fact sheet explicitly lists advanced secure boot capabilities, and AUTOSAR’s Firmware Over-The-Air (FOTA) explanation says the authenticity and integrity of the new software image must be checked during installation before activation.

A second principle is inactive-image staging. NXP’s update presentation on automotive edge nodes describes an A/B swap approach where the current application keeps running while a new firmware image is written to the inactive bank, and it lists the main advantages: the update can occur while the current application is still running, the original firmware remains available for rollback, and vehicle downtime is avoided, at the cost of roughly doubling application storage. Even though that document is automotive-focused, the pattern is a general embedded-network-device best practice.

A third principle is activation only under controlled conditions. AUTOSAR’s FOTA requirements say software installation can proceed in the background, but activation of the new software should occur only in a safe state, and rollback must be possible if activation or verification fails. Those are automotive requirements in the source, but again they express a general rule for safety-critical or always-available network devices: separate download from activation, and separate activation from irreversible commitment.

## 12. What a good update architecture looks like in practice

A good practical update architecture therefore has four layers.

First, a small trusted boot or recovery path that is hard to corrupt. NXP’s A/B update presentation explicitly assumes reset vectors to a bootloader that is never erased.

Second, a signed image acceptance policy, so the device authenticates firmware before execution. NXP’s SJA1110 fact sheet calls out secure boot, and AUTOSAR’s FOTA material says integrity and authenticity of the new image must be ensured before activation.

Third, an inactive-image staging area, whether in a second bank or external non-volatile memory, so the running image is not overwritten blindly. NXP’s A/B material is explicit about keeping the original firmware for rollback while writing the new one separately.

Fourth, a rollback path and version introspection, so management software can know what is running and recover if activation fails. AUTOSAR’s requirements say the target ECU shall support rollback in case of processing, verification, or activation errors and shall provide a way to read back the current software version.

That is the general data-com answer, even though the specific examples here come from embedded automotive networking sources.

# Automotive perspective and context

## 13. Firewall and ACL filtering in automotive switches

In an automotive switch, ACL-style filtering matters much more than it does in ordinary “best-effort” switching because the switch is often a containment boundary between traffic classes, zonal domains, or external-service paths. The automotive IEEE 802.1DG scope is explicit: it targets secure, highly reliable, deterministic-latency in-vehicle Ethernet. NXP’s SJA1110 fact sheet likewise highlights best-in-class packet inspection and Denial of Service (DoS) prevention capabilities, along with safety and security features.

That means the automotive interpretation of ACL/filtering is not merely “drop packets from source A to destination B.” It is often:

- constrain diagnostics or engineering traffic to the ports and VLANs where it belongs

- prevent unintended lateral movement between vehicle domains

- rate-limit or block malformed or malicious traffic early

- keep noncritical traffic from reaching safety-relevant consumers.

A fully stateful firewall may live in a gateway or compute node rather than in every switch ASIC, but ACL-style hardware filtering inside the switch is still a first-class automotive feature because it is cheap, deterministic, and immediate.

## 14. 802.1Q counter taxonomy in automotive switches

The 802.1Q counter taxonomy becomes more valuable in a vehicle because the question is not only “is there traffic?” but also “is the switch violating a timing or resource assumption?” The bridge-port statistics grouping’s separate counters for discard-lack-of-buffers, discard-transit-delay-exceeded, discard-on-error, and discard-inbound are therefore far more informative in an automotive network than a single lumped receive-error total would be.

In automotive bring-up and safety validation, those counters help separate different classes of failure:

- a filtering or policy problem

- a queue/resource exhaustion problem

- a delay-budget violation problem

- a frame-format or transmit-path problem.

That is one reason the move toward model-driven 802.1Q management is important in vehicles: the counters are only useful if software can collect and interpret them consistently across devices.

## 15. MDIO, SPI, and YANG in automotive systems

Automotive systems usually need all three, not one instead of the others.

MDIO is still the natural low-level path for controlling external PHYs, reading link status, running PHY diagnostics, and setting modes. Texas Instruments’ note makes clear that MDIO is the serial management interface used to configure PHY speed, duplex, auto-negotiation, and related status.

SPI is extremely common for switch control in automotive silicon. NXP’s SJA1105 is configured over SPI, and the SJA1110 exposes SPI and Quad SPI (QSPI) interfaces in its block diagram. On the SJA1110 evaluation board, NXP documents both NVM boot mode, where firmware boots from external flash, and SDL boot mode, where a companion device loads firmware via the SPI_HAP interface.

YANG/NETCONF/RESTCONF sits above that. In an automotive architecture, it may not always be exposed directly by every edge switch, but it becomes increasingly attractive for central controllers, manufacturing tools, validation rigs, and software-defined vehicle management stacks because it gives a common model for bridge state instead of a pile of ad hoc register pokes. That is exactly what the 802.1Qcp / 802.1Qcx / 802.1Qcw lineage is enabling.

So the automotive rule is:

- use MDIO for PHY-centric low-level control

- use SPI/QSPI for switch/bootstrap/device-local control

- use YANG over NETCONF or RESTCONF where you need repeatable, software-defined, model-driven configuration and observability across the network.

## 16. Firmware update strategy in automotive switches

Automotive makes firmware update strategy much stricter than ordinary networking. AUTOSAR’s FOTA explanation says updates should be installable in the background while the current software remains functionally available, and AUTOSAR’s requirements add that activation should occur only in the vehicle safe state, dependencies between software and configuration must be handled, rollback must be possible, and the target must support update-related diagnostics in normal operating mode.

That means an automotive switch or switch-centric Electronic Control Unit should ideally support:

- secure boot

- background image download

- inactive-bank or external-flash staging

- verification before activation

- rollback on failure

- version reporting and progress reporting

- safe-state activation rather than uncontrolled mid-drive reboot.

NXP’s SJA1110 fact sheet already shows the secure-boot direction, and NXP’s A/B update material shows the practical dual-image pattern. AUTOSAR then gives the system-level rules for when and how such updates should be orchestrated in a vehicle.

## 17. Why 802.1Qcw matters specifically in automotive

802.1Qcw matters in automotive because the features it models — scheduled traffic, frame preemption, and Per-Stream Filtering and Policing — are exactly the kinds of advanced bridge behaviors that become important in deterministic in-vehicle Ethernet. The official 802.1Qcw summary says that is precisely what the amendment adds YANG support for. The automotive IEEE 802.1DG profile, in turn, is about selecting and constraining IEEE 802.1 Time-Sensitive Networking and security features for secure, highly reliable, deterministic-latency in-vehicle Ethernet.

So the deeper automotive significance of 802.1Qcw is not “yet another YANG amendment.” It is that the advanced features automotive networks actually care about are moving into the same model-driven management ecosystem as ordinary VLANs and bridge ports. That matters for validation, plant tooling, in-field diagnostics, and long-term software-defined vehicle operation.

## 18. Summary

In pure data communications:

- ACL filtering is ordered stateless permit/deny policy

- firewall filtering adds broader, often stateful policy

- the 802.1Q counter taxonomy separates traffic volume from filtering outcomes, resource drops, delay drops, and transmit errors

- MDIO manages PHYs, SPI often manages switch silicon, and YANG models management data that NETCONF or RESTCONF can carry

- a robust firmware strategy keeps a trusted boot path, authenticates images, stages updates safely, and supports rollback

- 802.1Qcw extends 802.1Q YANG into scheduled traffic, frame preemption, and PSFP.

In automotive:

- filtering becomes a containment and safety/security boundary

- counter taxonomy becomes a validation and diagnostics tool

- MDIO, SPI, and YANG become complementary layers of control, not alternatives

- firmware update strategy must add safe-state activation, background installation, and rollback discipline

- 802.1Qcw matters because the advanced bridge features needed for deterministic in-vehicle Ethernet now need first-class model-driven management as well.