## 1. Data-communications context first

### 1.1 What AVTP is really for

AVTP is a Layer 2 transport protocol designed for time-sensitive applications in bridged local area networks.

The standard specifies the protocol, data encapsulations, and synchronization procedures used to enable interoperability between time-sensitive audio, video, and control applications using the quality-of-service capabilities provided by IEEE 802 Time-Sensitive Networking standards.

The protocol is not limited to audio and video; it also covers transport of data such as Controller Area Network (CAN) and Local Interconnect Network (LIN) over Layer 2 Ethernet.

That is the first deep idea to get right: AVTP is not “Ethernet with a media payload stuck inside.” It is a transport model that assumes the network can provide synchronized time, bounded latency, and reserved resources, and then uses those assumptions to move time-sensitive data in a predictable way.

AVTP streams requires Stream Reservation Protocol (SRP) reservations, which reinforces that AVTP lives in a deterministic-networking context rather than ordinary best-effort Ethernet.

### 1.2 Why AVTP exists when Ethernet already exists

Ordinary Ethernet can move packets, but by itself it does not tell a listener when a media sample or control event should be consumed.

AVTP adds common timing semantics so that multiple listeners can consume the same data coherently even when they are at different places in the network.

An AVTP talker inserts timestamps with a common time reference into outgoing IEEE 1722 streams, and that the AVTP presentation time tells a listener when to start processing the stream’s data.

Presentation time is the mechanism by which multiple listeners can render the media payload at the same time.

So AVTP’s core contribution is not only format encapsulation.

It is timed transport. It gives the payload a timing meaning relative to a shared network time base, usually generalized Precision Time Protocol (gPTP) in Audio Video Bridging or Time-Sensitive Networking systems. TI’s media-clock note explicitly ties AVTP timestamps to gPTP time.

### 1.3 The major AVTP data formats in IEEE 1722-2016

Public references to IEEE 1722-2016 show the major format clauses clearly: AVTP Audio Format (AAF), Compressed Video Format (CVF), AVTP Control Format (ACF), and Clock Reference Format (CRF) are all explicit parts of the 2016 standard.

The IEEE 1722 catalog table of contents lists AAF in Clause 7, CVF in Clause 8, ACF in Clause 9, and CRF in Clause 10. AVnu and NXP public material use the same names.

That means AVTP is not one monolithic payload format. It is a family of transport subtypes sharing common timing and stream concepts while specializing the payload structure for different data types.

## 2. AVTP Audio Format and Compressed Video Format

### 2.1 AVTP Audio Format

AAF stands for AVTP Audio Format. If an automotive Ethernet AVB audio endpoint supports AAF, it shall support AVTP Audio Format as defined in IEEE 1722-2016 Clause 7.

AAF is a native audio format that supports raw audio, integer and floating-point encodings, and common audio sample rates.

The important concept is that AAF is a format built specifically for carrying audio samples as timed AVTP payloads, without inheriting unnecessary baggage from older media encapsulations.

One of the goals of the newer AVTP audio formats was to “lose the baggage” from older IEC 61883 formatting.

So AAF is best understood as: a direct, timed audio payload format for AVTP streams. It is designed so the listener can reconstruct and play the audio at the intended shared presentation time, not merely “receive bytes.”

### 2.2 Compressed Video Format

CVF stands for Compressed Video Format. Public references to IEEE 1722-2016 place CVF in Clause 8, and NXP’s AVB material explicitly lists compressed video support such as H.264, Motion JPEG (MJPEG), and JPEG 2000 under the compressed video format heading. Later IEEE 1722b planning material also treats new compressed video codecs such as H.265 as extensions of the CVF area rather than a separate architecture.

The concept behind CVF is parallel to AAF: AVTP carries the compressed media payload, but the stream still has AVTP’s timing model around it. The video payload is therefore not just a file-like blob carried over Ethernet. It is part of a timed stream with sequence numbers, timing semantics, and listener buffering behavior. AVnu’s media-on-AVB presentation discusses professional and compressed video formats in exactly this stream-oriented way.

So CVF is best understood as: a timed compressed-video carriage format inside AVTP.

## 3. AVTP Control Format and CAN-over-Ethernet tunneling

### 3.1 What ACF is

ACF stands for AVTP Control Format. Public references to IEEE 1722-2016 list ACF as a major format clause, and AUTOSAR’s IEEE 1722 transport specifications state that the AUTOSAR IEEE1722 transport module supports handling of ACF streams and transportation of encapsulated bus frames such as CAN and LIN.

AVTP control streams as support non-audio/video data and specifically calls out automotive protocols such as FlexRay, CAN, and LIN.

That means ACF is the part of AVTP that lets the protocol carry control-oriented or tunneled non-media payloads rather than only audio/video streams.

In the automotive context, that becomes especially interesting because it lets legacy or edge-bus traffic be carried over an Ethernet backbone using a time-sensitive Layer 2 transport.

### 3.2 ACF and CAN-over-Ethernet tunneling

AUTOSAR’s IEEE 1722 transport module specification says the module supports transportation of encapsulated bus frames, specifically CAN and LIN, and handling of ACF streams.

The same document says this includes inspection of ACF payload, unpacking ACF messages, queueing and forwarding Link-layer Service Data Units (L-SDUs) according to the given AVTP presentation time, packing bus frames with bus type CAN and LIN, adding them to an ACF stream, and triggering transmission.

The cleanest way to explain CAN-over-Ethernet tunneling with ACF:

- the original CAN frame is not converted into a native IP service model

- it is encapsulated as an ACF message

- that ACF message is carried inside an AVTP stream across Ethernet

- the receiving side unpacks it and forwards it toward the legacy bus side.

So ACF tunneling is a transport bridge for legacy bus traffic, not a reinvention of CAN semantics at the application layer.

## 4. TSCF versus NTSCF

### 4.1 The expansions

TSCF stands for Time-Synchronous Control Format. NTSCF stands for Non-Time-Synchronous Control Format. An IEEE automotive “short Ethernet frames” presentation based on IEEE 1722-2016 shows exactly those two control-header types and gives their header sizes: 24 octets for TSCF and 12 octets for NTSCF. AUTOSAR’s IEEE1722 transport specification also defines an ACF stream as an IEEE 1722 stream of subtype TSCF or NTSCF that transports encapsulated bus frames as ACF messages.

### 4.2 The real difference

The key difference is timing semantics. AUTOSAR’s specification says IEEE 1722 specifies an AVTP presentation time for each defined AVTP stream-data subtype except NTSCF. It also says that when handling ACF streams, the common stream header is used to forward the presentation time of received ACF messages transported via TSCF to the receiving application.

So the clean conceptual difference is:

- TSCF carries control information that is meant to be consumed in a time-synchronous way, using AVTP presentation time semantics.

- NTSCF carries control information without that AVTP presentation-time mechanism.

That is why TSCF is the natural choice when a control message must be applied at a coordinated global time, while NTSCF is suitable when such synchronized application time is not required. AUTOSAR’s explicit statement that presentation time exists for all stream-data subtypes except NTSCF is the key standards-facing clue.

## 5. Media clock recovery

### 5.1 What “media clock” means here

AUTOSAR’s IEEE 1722 transport specification defines media clock as an entity that generates a rate, for example a precise hardware clock with a constant rate such as 48 kHz. It defines a media clock provider as an end node that hosts a media clock and transmits an IEEE 1722 stream of subtype Clock Reference Format (CRF) containing several presentation timestamps that correlate to the media clock rate. It defines a media clock consumer as an end node that receives a CRF stream and performs recovery of its media clock, for example with a Phase-Locked Loop (PLL), based on the received encapsulated data from the provider.

That already gives the core idea: media clock recovery is not just “guess the sample rate from packet arrival.” It is an explicit mechanism for reconstructing the source media timing from timing information carried in the network stream, especially CRF streams.

### 5.2 How presentation time participates in media clock recovery

Texas Instruments’ media-clock note says the AVTP presentation time represents the gPTP time at which a designated media sample or event is transferred to the time-sensitive application within each listener. It also says presentation time tells a listener when to start processing the stream’s data and is also used to recover the stream’s media clock. AVnu’s media-on-AVB presentation similarly says clock reference streams convey media clocks across the network and leverage the presentation-time concept of IEEE 1722.

So there are really two linked timing notions:

- stream alignment time, which lets all listeners present the same media content coherently

- media-rate recovery, which lets a listener regenerate the source clock rate for continuous playback or rendering.

That is why AVTP timing is more than “a playback deadline.” It is also part of how the listener rebuilds the source timing base.

## 6. Presentation time versus launch time

### 6.1 Presentation time

AUTOSAR’s Time Sensitive Network feature explanation says AVTP presentation time represents the globally synchronized time at which designated data within an AVTPDU payload is transferred to a time-sensitive application of a stream-data consumer. It also states the computation explicitly: TavtpPresentationTime = TcurrentGlobalTime + TmaxTransitTime. Texas Instruments says the AVTP talker adds a fixed offset, typically the maximum delay between the talker and all listeners, and generates presentation timestamps; the listener then uses presentation time to know when to start processing the data.

That means presentation time is a consumer-side timing target. It tells the listener when the data is meant to be rendered, consumed, or applied. It is therefore fundamentally about consumption timing, not about when the packet physically left the talker.

### 6.2 Launch time

“Launch time” is not the same thing as the AVTP presentation timestamp field, and it is better treated as an implementation concept than as the primary AVTP semantic. Early IEEE 1722 working material described a “nominal launch time” idea, but the 1722 timing model that survived centers on presentation time. In current practical implementations, “launch time” usually means the actual or scheduled moment when the talker injects the AVTP frame onto the network so that, after worst-case transit and buffering, it still arrives before the presentation deadline. AUTOSAR’s presentation-time formula and TI’s explanation of adding a fixed offset make this relationship explicit, even though they do not define launch time as a standard header field.

So the clean conceptual difference is:

- presentation time = when the listener should consume/render the data

- launch time = when the talker must put the packet onto the network to meet that presentation target. This second statement is an implementation inference from the AVTP timing model, not a separate AVTP payload field.

That distinction matters because students often think the timestamp in the packet is “the time the packet was sent.” In AVTP, it is usually more useful to think of it as “the time the payload is meant to matter at the listener.”

## 7. Automotive perspective and context

### 7.1 Why IEEE 1722 matters in automotive at all

The COVESA Open1722 project states that IEEE 1722 is gaining traction in the automotive domain because, compared with higher-layer protocols such as Scalable service-Oriented MiddlewarE over IP (SOME/IP) or Data Distribution Service (DDS), it is well suited for transporting data to and from very resource-constrained devices, and for tunneling fieldbuses such as CAN and LIN across zones or domains. AUTOSAR’s IEEE1722 requirements and transport specifications also show clear automotive intent: support for encapsulated bus frames, ACF streams, and interaction with synchronized time and media clock handling.

So in automotive, IEEE 1722 is not just an infotainment/media protocol. It is also a deterministic Layer 2 transport tool for bridging legacy edge buses and time-sensitive application data into a switched Ethernet backbone.

### 7.2 AAF and CVF in vehicle systems

In vehicles, AAF and CVF map naturally to audio and video distribution use cases. The Avnu automotive interoperability specification devotes sections to AAF and compressed video formats, while NXP’s AVB material highlights AAF for raw audio and CVF for compressed video such as H.264, MJPEG, and JPEG 2000. The same Avnu automotive profile ties these streams to stream-reservation classes and timing uncertainty budgets.

That means in an automotive context:

- AAF is attractive for deterministic in-vehicle audio transport

- CVF is attractive for time-sensitive compressed video transport

- both rely on the broader Audio Video Bridging / Time-Sensitive Networking toolchain, including gPTP time, reserved bandwidth, and listener buffering against presentation time.

### 7.3 ACF and zonal architectures

ACF is especially important in modern zonal architectures because zonal controllers often sit between legacy buses and an Ethernet backbone. IEEE and NXP automotive presentations explicitly illustrate zonal ECUs with local CAN and LIN connections tunneling that traffic over the Ethernet backbone via IEEE 1722-2016. AUTOSAR’s 2024 “Deterministic Communication with TSN” overview likewise says completion of IEEE1722 transfer for encapsulated CAN and LIN bus frames enables Classic Platform ECUs to communicate with conventional bus systems across several architectural zones through an Ethernet network.

That makes ACF one of the most automotive-relevant parts of IEEE 1722. It gives the architecture a way to transport fieldbus traffic without forcing every edge device to become a full IP or SOME/IP node.

### 7.4 TSCF versus NTSCF in automotive control

In a vehicle, the TSCF versus NTSCF distinction becomes very practical.

If a tunnelled control or bus-frame payload must be consumed relative to synchronized global time, TSCF is the right semantic because AVTP presentation time is available and can be forwarded to the receiving application. If the payload does not require such synchronized consumption semantics, NTSCF can avoid the extra timing mechanism. AUTOSAR is explicit that presentation time is defined for AVTP stream-data subtypes except NTSCF, and that the TSCF case forwards presentation time metadata to the receiving side.

That means a vehicle designer can choose between:

- time-synchronous control tunneling, for payloads whose execution time matters globally

- non-time-synchronous control tunneling, for payloads whose value matters but not at a synchronized presentation instant.

### 7.5 Media clock recovery in automotive infotainment and perception chains

For audio/video in a vehicle, media clock recovery is not optional. You do not just need packets to arrive; you need speakers, displays, and processing nodes to run at the right rate and consume data coherently. Some automotive profiles have explicit requirements around Clock Reference Format (CRF) support, frequency accuracy, and the relation between generated stream presentation timestamps and associated CRF timing. presentation time is used to recover the stream’s media clock, and AUTOSAR’s IEEE1722 transport module defines the provider/consumer model around CRF and clock recovery.

So in automotive systems, media clock recovery should be explained as part of synchronous rendering and actuation discipline, not just as a media-player trick. It is how a distributed network keeps multiple endpoints “playing the same timed reality.”

### 7.6 Presentation time versus launch time in automotive engineering

Automotive engineering discussions often need both ideas, even though only one is directly standardized in the AVTP headers.

The presentation time is the synchronized consumer deadline. AUTOSAR gives the exact formula using current synchronized global time plus a configured maximum transit time.

The launch time is the transmitter-side scheduling consequence: the talker, traffic shaper, or egress scheduler must release the packet early enough that it reaches all listeners before presentation time.

The talker adds a fixed offset, typically the maximum delay to the listeners, to create presentation timestamps.

That distinction becomes especially important in automotive TSN networks where queues, reservation classes, and switch shaping all influence when a talker can actually inject the frame.

Some automotive profile even defines Maximum Timing Uncertainty and maximum transit time concepts to make listener buffering and presentation-time validation meaningful.

## 8. Summary

In pure data communications, IEEE 1722-2016 AVTP is a timed Layer 2 transport family for time-sensitive applications. AAF carries timed audio, CVF carries timed compressed video, ACF carries timed or untimed control-oriented payloads, and CRF carries timing information used for media clock recovery. TSCF means Time-Synchronous Control Format and supports AVTP presentation time; NTSCF means Non-Time-Synchronous Control Format and is the main control subtype without AVTP presentation time. Media clock recovery uses CRF and presentation-timestamp semantics so listeners can reconstruct source timing, and presentation time is the synchronized consumer deadline rather than simply the sender’s transmit timestamp.

In automotive, IEEE 1722 becomes more than an audio/video protocol. It is also a practical tunneling mechanism for CAN and LIN across Ethernet backbones, especially in zonal architectures. ACF is therefore one of the most automotive-relevant 1722 formats, while AAF, CVF, CRF, and presentation-time semantics remain central for synchronized media transport. The vehicle designer chooses between TSCF and NTSCF depending on whether synchronized control consumption is required, and the overall system must treat launch time as the engineering scheduling problem required to meet the AVTP presentation-time contract.