.

In Asynchronous Traffic Shaping (ATS), frames from multiple streams can be interleaved in a common ATS-controlled traffic-class queue and selected in ascending eligibility-time order. However, it is not always one global merge of all streams into one single queue. The standard model is more like this:

Stream 1 ── eligibility-time calculation ─┐

Stream 2 ── eligibility-time calculation ─┼──> ATS traffic-class queue

Stream 3 ── eligibility-time calculation ─┘       ordered/selected by eligibility time

|

v

egress scheduler / port transmission selection

But if different streams are mapped to different traffic classes, then they may go to different egress queues:

Stream A ── ATS shaper ──> ATS Queue for Traffic Class 5 ─┐

Stream B ── ATS shaper ──> ATS Queue for Traffic Class 5 ─┤

├──> port scheduler

Stream C ── ATS shaper ──> ATS Queue for Traffic Class 4 ─┤

Best effort traffic ─────> BE Queue                       ┘

So for streams that are mapped into the same ATS traffic-class queue, it is correct to say that they are merged into a single FIFO. They are not necessarily all merged into one universal ATS queue across the whole port.

## What “eligible frames then compete under local scheduler rules” really means

There are two stages.

### Stage 1: ATS eligibility-time control

For each ATS-controlled frame, the ATS shaper computes an eligibility time. The frame is not allowed to be selected before that time.

Frame arrives at bridge

|

v

ATS computes eligibility time

|

v

Frame waits until current time >= eligibility time

The public IEEE 802.1Qcr draft describes this directly: ATS shapers assign eligibility times, and the ATS transmission selection algorithm then uses those assigned eligibility times to regulate traffic transmission.

A frame becomes eligible when:

assigned eligibility time <= current transmission-selection time

The draft wording says that, for a queue supporting ATS transmission selection, a frame is eligible when its assigned eligibility time is less than or equal to the current time.

### Stage 2: Local egress scheduling

Once a frame becomes eligible, ATS has only removed the shaping restriction. It has not guaranteed immediate transmission.

Now the frame becomes available to the normal egress-port transmission-selection machinery.

That means it can still be delayed by:

- another higher-priority queue

- a time-aware gate if Time-Aware Shaping is also used

- a frame already being transmitted

- credit-based shaper rules on another queue

- strict-priority arbitration

- implementation-specific queue arbitration

- frame preemption or non-preemption behavior

So “eligible frames compete under local scheduler rules” means:

ATS says: “This frame is now allowed to be considered.”

The egress scheduler says: “Among all queues that currently have allowed frames,

which queue/frame actually gets the port next?”

## The key distinction

Do not think of ATS like this:

Wrong mental model:

All streams in the bridge

|

v

One global sorted queue by eligibility time

|

v

Transmit strictly in global eligibility-time order

A better mental model is:

Correct mental model:

Each ATS stream or stream group gets an eligibility time calculation

|

v

Frames are placed into traffic-class queues

|

v

Within an ATS queue, eligibility time controls which frames become selectable

|

v

Across queues, the local egress scheduler arbitrates

The IEEE draft also says that the internal priority value determines the traffic class of a frame, hence selecting the outbound queue. So queue selection is still traffic-class based. ATS does not erase the normal bridge queue model.

## What happens inside one ATS queue?

Inside a queue that supports ATS transmission selection:

Frame A: eligibility time = 100 µs

Frame B: eligibility time = 120 µs

Frame C: eligibility time = 105 µs

The ATS queue should make them available in eligibility-time order:

100 µs -> Frame A

105 µs -> Frame C

120 µs -> Frame B

The IEEE draft says that frames that reach their selectability time are selected for transmission in ascending order of assigned eligibility times, while frames with identical eligibility times must preserve the required ordering.

So within an ATS queue, yes, it is reasonable to think:

multiple shaped streams

|

v

interleaved by eligibility time

|

v

eligible frame becomes candidate for transmission

This is why ATS is often described as an interleaved regulator.

## But across queues, eligibility time is not the only decision

Suppose we have:

Queue TC5, ATS:

Frame X eligibility time = 100 µs

Queue TC4, ATS:

Frame Y eligibility time = 90 µs

Queue TC0, best effort:

Frame Z waiting

At time 100 µs, both Frame X and Frame Y are eligible. But the port scheduler may still choose based on traffic class priority.

For example, if TC5 has higher priority than TC4:

Even though Frame Y became eligible earlier,

Frame X may transmit first because TC5 wins scheduler arbitration.

So ATS eligibility time is not necessarily a global transmit order across all queues.

It is more precise to say:

Eligibility time determines when a frame is allowed to join transmission selection.

The configured egress scheduler determines which available queue/frame actually transmits.

## Very simple analogy

Think of ATS as a security checkpoint and the egress scheduler as the boarding gate.

ATS eligibility:

“You are not allowed to enter the waiting area before your allowed time.”

Egress scheduler:

“Among all people already in the waiting area, who boards next?”

So an eligible ATS frame is not necessarily transmitted immediately. It is only now allowed to compete.

## Final answer to your question

Yes, for streams mapped to the same ATS traffic-class queue, ATS effectively interleaves frames from multiple streams using their eligibility times. The queue or transmission-selection logic then selects eligible frames in increasing eligibility-time order.

But no, ATS does not necessarily merge all streams from all traffic classes into one single global eligibility-time queue. Frames are still mapped to traffic classes and egress queues. ATS controls when a frame in an ATS-capable queue becomes selectable. After that, the local port scheduler arbitrates among all available queues according to the configured bridge scheduling rules.

ATS computes eligibility time using a per-stream or per-shaper token-bucket-like regulator.

The important inputs are:

tA  = arrival time of the frame

L   = frame length in bits, including configured media overhead

r   = Committed Information Rate, CIR, in bits/s

C   = Committed Burst Size, CBS, in bits

E   = BucketEmptyTime

τG  = GroupEligibilityTime

Δmax = Maximum Residence Time

The public IEEE 802.1Qcr draft says each arriving frame invokes ProcessFrame(frame), and that this procedure computes and assigns eligibility time. It also states that the ATS shaper state machine is based on a token bucket shaping algorithm, although the formal state machine is expressed using variables such as BucketEmptyTime, GroupEligibilityTime, CommittedBurstSize, and CommittedInformationRate.

## 1. First calculate how long this frame consumes the stream rate

For a frame of length L and configured committed rate r:

lengthRecoveryDuration = L / r

Meaning:

If this frame consumes L bits of credit,

how long does it take to recover those L bits at rate r?

Example:

L = 4000 bits

r = 10 Mbit/s

lengthRecoveryDuration = 4000 / 10,000,000

= 0.0004 s

= 400 µs

So after sending this frame, the stream has consumed 400 µs worth of its configured rate.

## 2. Then calculate when the frame would be legal according to this stream’s bucket state

ATS keeps a variable called BucketEmptyTime.

Think of it this way:

BucketEmptyTime = the virtual time at which the bucket would be empty.

AUTOSAR’s explanation of the IEEE 802.1Q ATS algorithm says that if BucketEmptyTime is in the past relative to the frame arrival time, positive virtual credit has accumulated. If BucketEmptyTime is in the future, virtual credit is negative. It also gives the virtual-credit interpretation as c[t] = (t − E) × r. (autosar.org)

The shaper computes:

shaperEligibilityTime = BucketEmptyTime + lengthRecoveryDuration

or:

τ0 = E + L/r

This is the first key formula.

Meaning:

If the virtual bucket state is represented by E,

then this frame can be released when enough credit exists

to pay for L bits at rate r.

## 3. Then ATS takes the maximum of three times

The eligibility time is:

eligibilityTime = max(arrivalTime,

GroupEligibilityTime,

shaperEligibilityTime)

or:

τ = max(tA, τG, E + L/r)

The draft pseudo-code gives exactly this form: eligibilityTime = max(arrivalTime(frame), GroupEligibilityTime, shaperEligibilityTime).

Each term has a different meaning.

arrivalTime:

A frame cannot become eligible before it has actually arrived.

shaperEligibilityTime:

The frame cannot become eligible before the stream has enough virtual credit.

GroupEligibilityTime:

Frames in the same ATS shaper group must not get decreasing eligibility times.

That last one is important. The draft says ATS shaper groups produce non-decreasing eligibility times for successive frames in a group, allowing frames of one group to be queued in first-in first-out order.

So the calculation is not simply:

eligibilityTime = E + L/r

It is:

eligibilityTime = max(actual arrival time,

previous group eligibility time,

per-stream bucket-based eligibility time)

## 4. Then ATS checks whether the frame would stay too long

After calculating eligibility time, ATS checks:

eligibilityTime <= arrivalTime + MaxResidenceTime

If this is true, the frame is accepted and assigned that eligibility time.

If false, the frame is discarded.

The draft pseudo-code performs exactly this check before assigning the frame and updating state. AUTOSAR also explains that the frame is discarded if its eligibility time lies too far in the future according to MaxResidenceTime. (autosar.org)

Meaning:

ATS does not allow a badly behaving or over-rate stream

to create an indefinitely delayed backlog.

## 5. Then ATS updates the bucket state

After accepting the frame, ATS updates:

GroupEligibilityTime = eligibilityTime

Then it updates BucketEmptyTime.

The draft pseudo-code is:

BucketEmptyTime =

if eligibilityTime < bucketFullTime:

shaperEligibilityTime

else:

shaperEligibilityTime + eligibilityTime - bucketFullTime

where:

bucketFullTime = BucketEmptyTime + CommittedBurstSize / CommittedInformationRate

The public draft defines CommittedBurstSize as the token bucket capacity and CommittedInformationRate as the token refill rate. It also defines bucketFullTime as the instant when the token bucket is full.

The intuition is:

If the bucket was not already full, normal frame consumption moves BucketEmptyTime forward.

If the bucket would have become full before this frame became eligible,

the extra idle time beyond full capacity is not allowed to accumulate unlimited credit.

That is how the burst size limit is enforced.

Without this correction, a stream that was idle for a long time could accumulate infinite credit and then release an unlimited burst. ATS does not allow that. The maximum useful accumulated credit is bounded by CommittedBurstSize.

# Numerical example

Assume:

Committed Information Rate, r = 10 Mbit/s

Committed Burst Size, C       = 12,000 bits

Frame length, L               = 4,000 bits

Maximum Residence Time        = large enough, ignore for now

GroupEligibilityTime          = in the past, ignore for first example

The time to recover one frame length is:

L/r = 4000 / 10,000,000 = 400 µs

The time to fill the whole bucket is:

C/r = 12000 / 10,000,000 = 1200 µs

Now suppose a frame arrives at:

tA = 10.000 ms

and the current BucketEmptyTime is:

E = 9.000 ms

Then:

shaperEligibilityTime = E + L/r

= 9.000 ms + 0.400 ms

= 9.400 ms

Now ATS computes:

eligibilityTime = max(10.000 ms, 9.400 ms)

= 10.000 ms

So the frame is eligible immediately at arrival.

Why?

Because the stream has accumulated enough credit. In virtual-credit terms:

credit at arrival = (tA - E) × r

= (10.000 ms - 9.000 ms) × 10 Mbit/s

= 10,000 bits

The frame only needs 4,000 bits of credit. Therefore, it can pass immediately.

# Burst example

Now suppose three 4,000-bit frames arrive together at 10.000 ms, and the bucket is full with C = 12,000 bits.

Each frame consumes 4,000 bits.

Initial available burst credit = 12,000 bits

Frame 1 consumes 4,000 bits

Remaining credit = 8,000 bits

Eligible immediately

Frame 2 consumes 4,000 bits

Remaining credit = 4,000 bits

Eligible immediately

Frame 3 consumes 4,000 bits

Remaining credit = 0 bits

Eligible immediately

A fourth frame arriving at the same instant would not be eligible immediately:

Frame 4 needs another 4,000 bits

Recovery time = 4,000 / 10 Mbit/s

= 400 µs

Frame 4 eligibility time = 10.400 ms

So ATS allows a burst up to the committed burst size, then spaces additional frames according to the committed rate.

That is the essence of ATS.

# What this means in plain language

ATS computes eligibility time like this:

1. Measure when the frame arrived.

2. Look at the stream’s virtual bucket state.

3. Calculate how long it takes to recover enough credit for this frame length.

4. Prevent eligibility from going backward within the shaper group.

5. Assign the frame the earliest time that satisfies all of those constraints.

6. Discard the frame if that time is too far in the future.

7. Update the shaper state so the next frame sees the effect of this frame.

A compact version is:

lengthRecoveryDuration = L / CIR

shaperEligibilityTime = BucketEmptyTime + lengthRecoveryDuration

eligibilityTime = max(arrivalTime,

GroupEligibilityTime,

shaperEligibilityTime)

if eligibilityTime > arrivalTime + MaxResidenceTime:

discard frame

else:

assign eligibilityTime

update GroupEligibilityTime

update BucketEmptyTime

# Key point

ATS is not saying:

Send one frame every fixed period.

It is saying:

For each stream, allow bursts up to Committed Burst Size,

but over time force the stream to respect Committed Information Rate.

The eligibility time is the mechanism that converts that rate-and-burst contract into a concrete timestamp attached to each frame. Once the frame’s eligibility time has arrived, the frame may participate in the normal egress transmission selection process.

BucketEmptyTime is best understood as a virtual credit-state timestamp for one ATS shaper instance.

It is not saying:

“The physical bucket becomes empty at this wall-clock time.”

It is saying:

“If I represent this stream’s token/credit state using time instead of bits,

BucketEmptyTime is the reference time from which available credit can be computed.”

IEEE material describes BucketEmptyTime as the state of the ATS scheduler and says it may be in the past, present, or future; the 802.1Qcr draft also defines it as an internal state variable updated using committed burst size, committed information rate, frame arrival times, and frame lengths. (IEEE Standards Association)

## 1. Why does ATS use BucketEmptyTime at all?

A normal token bucket can be imagined like this:

tokens refill at rate r

bucket capacity = C

frame consumes L tokens

ATS could store the number of available tokens directly:

available_credit_bits = 8000 bits

But instead, ATS stores the same information as a time value:

BucketEmptyTime = E

Then credit at current time t can be interpreted as:

available credit ≈ (t - E) × r

where:

t = current time

E = BucketEmptyTime

r = Committed Information Rate

So:

If E is far in the past:

the stream has positive accumulated credit.

If E is equal to now:

the stream has approximately zero credit.

If E is in the future:

the stream is in debt and must wait.

AUTOSAR explains this same interpretation: when BucketEmptyTime is in the past, positive virtual credit has accumulated; when it is in the future, the virtual credit is negative. (AUTOSAR)

## 2. Simple mental picture

Assume:

Committed Information Rate, r = 10 Mbit/s

BucketEmptyTime, E = 10.000 ms

Current time, t = 10.500 ms

The difference is:

t - E = 0.500 ms

At 10 Mbit/s, 0.500 ms corresponds to:

10,000,000 bits/s × 0.0005 s = 5000 bits

So the stream has virtually accumulated about:

5000 bits of credit

That means a 4000-bit frame can be made eligible immediately, because enough credit exists.

## 3. What if BucketEmptyTime is in the future?

Assume:

r = 10 Mbit/s

Current time, t = 10.000 ms

BucketEmptyTime, E = 10.300 ms

Frame length, L = 4000 bits

Here, E is 300 µs in the future.

That means the stream is already in negative credit, or debt.

virtual credit = (t - E) × r

= (10.000 ms - 10.300 ms) × 10 Mbit/s

= -3000 bits

Now a new 4000-bit frame arrives. The stream first has to recover the 3000-bit debt, then recover another 4000 bits for the new frame.

The frame’s shaper eligibility time is:

E + L/r

Since:

L/r = 4000 bits / 10,000,000 bits/s

= 400 µs

we get:

shaperEligibilityTime = 10.300 ms + 0.400 ms

= 10.700 ms

So the frame cannot become eligible until:

10.700 ms

This is why BucketEmptyTime can be in the future. It means:

The stream has already consumed more than its allowed immediate credit.

New frames must wait until the rate contract catches up.

## 4. Why is it called “empty time”?

Think of the token bucket in reverse.

If the bucket has some credit at time t, then there must have been an earlier virtual time when the bucket had zero credit. That time is BucketEmptyTime.

Past empty time                 Current time

E                              t

|------------------------------|

accumulated credit

The longer the gap between E and t, the more credit has accumulated.

credit = (t - E) × r

So if:

E = 9.000 ms

t = 10.000 ms

r = 10 Mbit/s

then:

credit = 1.000 ms × 10 Mbit/s

= 10,000 bits

That is the cleanest way to understand it:

BucketEmptyTime is the virtual time origin from which the stream’s current credit is measured.

## 5. But credit cannot grow forever

The bucket has a maximum burst capacity:

Committed Burst Size = C

So the credit is not allowed to grow without limit.

If:

C = 12,000 bits

r = 10 Mbit/s

then the time needed to fill the bucket is:

C/r = 12,000 / 10,000,000

= 1.2 ms

So if BucketEmptyTime is more than 1.2 ms in the past, the bucket is already full. Extra idle time does not create more credit.

BucketEmptyTime = 5.000 ms

Current time    = 10.000 ms

Raw credit would be:

5 ms × 10 Mbit/s = 50,000 bits

But if C = 12,000 bits,

usable credit is capped at 12,000 bits.

That is why ATS also computes a bucketFullTime:

bucketFullTime = BucketEmptyTime + CommittedBurstSize / CommittedInformationRate

Meaning:

bucketFullTime = the virtual time at which the bucket would become full.

The 802.1Qcr draft pseudo-code uses this bucketFullTime check to prevent unlimited credit accumulation. (IDAlab)

## 6. How BucketEmptyTime affects eligibility time

For each arriving frame:

L = frame length in bits

r = Committed Information Rate

E = BucketEmptyTime

The frame consumes:

L/r

seconds of rate budget.

So ATS computes:

shaperEligibilityTime = E + L/r

Then the assigned eligibility time is:

eligibilityTime = max(arrivalTime,

GroupEligibilityTime,

E + L/r)

So BucketEmptyTime directly answers this question:

Based on the stream’s existing credit or debt,

what is the earliest time this frame can legally become eligible?

## 7. What happens after a frame is accepted?

After ATS assigns the frame an eligibility time, it updates BucketEmptyTime so the next frame sees the effect of this frame.

In simplified conceptual form:

New BucketEmptyTime moves forward by L/r

because the accepted frame consumed L bits of credit.

Example:

Old BucketEmptyTime = 10.000 ms

Frame length        = 4000 bits

CIR                 = 10 Mbit/s

L/r = 400 µs

New BucketEmptyTime ≈ 10.400 ms

Meaning:

This frame has consumed 400 µs worth of the stream’s committed rate.

If the bucket was already full before this frame became eligible, the update is adjusted so that excess idle time is discarded. That is the role of the bucketFullTime logic.

## 8. Very concrete timeline example

Assume:

CIR = 10 Mbit/s

CBS = 12,000 bits

Frame length = 4000 bits

L/r = 400 µs

Suppose the stream has been idle long enough that the bucket is full at:

t = 10.000 ms

A burst of four 4000-bit frames arrives at exactly 10.000 ms.

The bucket can hold 12,000 bits, so the first three frames fit inside the burst allowance:

Frame 1: eligible at 10.000 ms

Frame 2: eligible at 10.000 ms

Frame 3: eligible at 10.000 ms

Together they consume:

3 × 4000 bits = 12,000 bits

Now the bucket is empty. The fourth frame must wait for another 4000 bits of credit:

Frame 4: eligible at 10.400 ms

So the resulting eligibility times are:

Frame 1  -> 10.000 ms

Frame 2  -> 10.000 ms

Frame 3  -> 10.000 ms

Frame 4  -> 10.400 ms

Frame 5  -> 10.800 ms

Frame 6  -> 11.200 ms

This is exactly the behavior you want:

Allow a bounded burst up to CBS.

After that, space frames according to CIR.

BucketEmptyTime is the variable that makes this possible.

## 9. One-line definition

The best definition is:

BucketEmptyTime is the virtual timestamp that represents the ATS shaper’s

current credit state; comparing current time with BucketEmptyTime tells the

shaper whether the stream has positive credit, zero credit, or rate debt.

In plain words:

BucketEmptyTime in the past   -> the stream has credit.

BucketEmptyTime equal to now  -> the stream has no credit.

BucketEmptyTime in the future -> the stream is in debt and must wait.

That is the core idea.