.Automotive Ethernet security and MACsec

## 1. Data-communications context first

### 1.1 Start with the real threat model

An Ethernet network is attractive because it is fast, flexible, and layered. Those same qualities also enlarge the attack surface. In general Ethernet Local Area Networks, common threats include passive eavesdropping, message injection, replay, in-transit modification, software compromise, and using one compromised machine as a foothold for later attacks. Marvell’s MACsec tutorial lists exactly those Ethernet-LAN threats, including eavesdropping, sending arbitrary messages, replaying recorded messages, and modifying messages in transit.

At the vehicle-system level, the same pattern appears in a more safety-critical form. NHTSA’s 2022 cybersecurity guidance says an attack surface is the set of interfaces where an unauthorized user can inject or extract data or modify system behavior, and it warns that wireless paths, open Internet Protocol ports, exposed services, and weak segmentation can become attack vectors into vehicle systems. It also explicitly gives examples of spoofed control-command and spoofed vehicle-speed messages causing dangerous misbehavior.

So the right first-principles framing is this: automotive Ethernet security is not only about secrecy; it is about confidentiality, integrity, authenticity, replay resistance, segmentation, and survivability when one part of the system is already compromised.

### 1.2 Where MACsec fits

MACsec stands for Media Access Control Security and is standardized in IEEE 802.1AE. The IEEE 802.1 security page says MACsec specifies cryptographic cipher suites that ensure authenticity, integrity, and optionally confidentiality of the data of the MAC service and internal sublayer service. The IEEE 802.1AE page says it provides connectionless user-data confidentiality, data integrity, data-origin authenticity, and privacy protection, and that the MACsec Key Agreement (MKA) protocol from IEEE 802.1X discovers mutually authenticated peers and elects a key server to distribute the Secure Association Keys (SAKs) used to protect frames.

That already tells you two important things.

First, MACsec is a Layer 2 hop-by-hop security mechanism, not an end-to-end application-security protocol. It is a point-to-point security protocol for traffic on Layer 1 or Layer 2 links.

Second, MACsec is not “just encryption.” It always aims at integrity and authenticity, and confidentiality is optional. That is why the standards language consistently says integrity and optionally confidentiality, rather than the other way around.

### 1.3 The core MACsec architecture: CA, SC, SA, and SecY

The architecture is easiest to understand in layers.

A secure Connectivity Association (CA) is the overall security relationship among the participants that are meant to communicate securely on a given LAN. IEEE 802.1AE defines a CA as a security relationship, established and maintained by key-agreement protocols, comprising a fully connected subset of service access points on a single Local Area Network that are to be supported by MACsec.

Inside a CA, traffic is carried on one or more Secure Channels (SCs). IEEE 802.1AE defines a Secure Channel as a security relationship used to provide security guarantees for frames transmitted from one member of a Connectivity Association to the others. It also says an SC is supported by a sequence of Secure Associations.

Inside each Secure Channel, there are Secure Associations (SAs). An SA is the actual cryptographic operating context that uses one Secure Association Key (SAK). IEEE 802.1AE defines the SAK as the secret key used by an SA, and it defines the Secure Association Identifier (SAI) as the Secure Channel Identifier concatenated with the Association Number (AN).

The protocol entity that implements MACsec on a port is the MAC Security Entity (SecY). IEEE 802.1AE identifies SecY as the MAC Security Entity and says a conformant SecY supports Controlled and Uncontrolled Ports plus a Common Port. The Controlled Port is the secure MAC service access point; the Uncontrolled Port is the insecure MAC service access point.

So the clean architecture picture is:

- CA = the overall secure relationship

- SC = one directional secure traffic relationship inside that CA

- SA = one keyed cryptographic context used by that Secure Channel

- SecY = the port-side MACsec engine that enforces all of this.

### 1.4 Why there are multiple SAs inside one SC

This is one of the most important non-obvious design points.

IEEE 802.1AE says an SC is supported by a sequence of Secure Associations, allowing the periodic use of fresh keys without terminating the relationship. The operational meaning is that each Secure Association has its own SAK and limited lifetime, and both sides establish a new SA and switch to it before the old one expires, which is essentially key rotation.

So an SC is long-lived, while SAs inside it roll over. That is how MACsec can rekey without tearing down the whole secure communication relationship.

### 1.5 MKA: what it actually does

MKA is not the data-protection algorithm. It is the peer-discovery and key-management protocol that makes MACsec operational.

The IEEE 802.1 security page says IEEE 802.1X specifies the MKA protocol used by IEEE 802.1AE. The IEEE 802.1AE page adds that MKA discovers mutually authenticated peers and elects one as a Key Server that distributes the symmetric SAKs used by MACsec to protect frames. AUTOSAR’s MACsec implementation guidance says the purpose of MKA is to discover MACsec peers and negotiate the security keys required to secure the link. It also says two key-generation mechanisms exist under IEEE 802.1X: Pre-Shared Keys (PSK) and keys derived from a successful Extensible Authentication Protocol (EAP) authentication.

So MKA answers three practical questions:

- who is my legitimate MACsec peer?

- which side is the key server?

- which SAK should the current Secure Association use?

### 1.6 The MACsec frame: SecTAG and ICV

A MACsec-protected frame is still an Ethernet frame, but with additional protection fields inserted.

The main added header is the SecTAG, the MAC Security Tag. IEEE 802.1AE shows that the received SecTAG contains bits of the Tag Control Information (TCI) and optionally the Secure Channel Identifier (SCI), and that the 32 least significant bits of the Packet Number (PN) are conveyed in the SecTAG. The standard also says the Integrity Check Value (ICV) is derived by algorithmic transformation on the protected data unit and is sent with the frame so the receiver can recalculate and compare it. It further says the ICV protects the destination and source MAC addresses as well as all the fields of the MACsec protocol data unit.

The TCI bits themselves matter because they carry protocol-control information about how to parse and validate the SecTAG. IEEE 802.1AE says the TCI supports protocol versioning, indicates whether the SCI is explicitly present, and includes flags such as the ES and SC bits.

So the concise frame-level picture is:

- SecTAG = control and sequencing metadata for MACsec

- PN = anti-replay and nonce-related packet numbering

- SCI = identifies the Secure Channel

- ICV = the authentication/integrity result attached to the frame.

### 1.7 Secure Channel Identifier, Association Number, and Packet Number

These three are easy to confuse.

The SCI, or Secure Channel Identifier, uniquely identifies a Secure Channel and is composed of a MAC address and a Port Identifier. The AN, or Association Number, identifies which SA inside the Secure Channel is currently in use. The PN, or Packet Number, increments per protected frame and is used for replay protection and nonce construction. Marvell’s tutorial states that replay protection is performed by checking the Packet Number in the SecTAG against the locally stored value, and that each MACsec packet has a unique sequential packet number within a given SA.

So the easiest way to remember them is:

- SCI tells you which secure channel

- AN tells you which association within that channel

- PN tells you which frame number within that association.

### 1.8 The cipher suites: GCM-AES-128 and GCM-AES-256

IEEE 802.1AE’s cipher-suite table says the standard supports both integrity without confidentiality and integrity with confidentiality, and it lists GCM-AES-128 as the mandatory default cipher suite and GCM-AES-256 as an optional cipher suite. The same text says the default cipher suite uses Galois/Counter Mode (GCM) with the Advanced Encryption Standard (AES) 128-bit block cipher, and that the 128-bit SAK is the key input. The IEEE 802.1AE page also notes that GCM-AES-256 was added later as an option.

That means the two core deployment options are:

- GCM-AES-128: mandatory baseline support

- GCM-AES-256: stronger key size, optional in the standard baseline.

### 1.9 Integrity-only versus confidentiality modes

This is a crucial distinction.

IEEE 802.1AE explicitly supports integrity without confidentiality and integrity with confidentiality. The standard even says that when the default cipher suite is used for integrity protection without confidentiality protection, protocol analyzers and MACsec-aware tools can still understand the transmitted user data, but they cannot modify it without the receiving SecY detecting the intrusion.

So integrity-only mode means:

- the payload remains visible to legitimate analyzers and tools

- the frame is still authenticated and tamper-evident

- confidentiality is not provided.

Confidentiality mode means the protected data is encrypted as well as authenticated. IEEE 802.1AE further explains confidentiality offset: with a nonzero offset, the initial part of the user data can remain cleartext while the remainder is encrypted. The standard notes this was originally meant to help early deployments that needed to inspect the first bytes of IPv4 or IPv6 frames before decryption. It also notes that XPN cipher suites do not support confidentiality offsets.

So the clean mode split is:

- integrity-only = authenticate without encrypting

- integrity + confidentiality = authenticate and encrypt

- confidentiality offset = optional partial-cleartext mode in some suites, mainly a deployment accommodation.

### 1.10 What MACsec does and does not secure

MACsec is hop-by-hop. It secures traffic between adjacent MACsec peers or through explicitly defined MACsec domains; it is not, by itself, an end-to-end application security mechanism across multiple untrusted hops. Marvell’s tutorial describes it as a point-to-point security protocol, and the IEEE 802.1AE page frames it as protection of MAC service communication between peers.

This matters because MACsec can stop passive tapping, injection, replay, tampering, and some man-in-the-middle behavior on a protected link, but it does not magically secure traffic after that traffic is decrypted at the next trusted hop. You still need sound system architecture, segmentation, and higher-layer security where appropriate. NHTSA’s guidance explicitly recommends segmentation, isolation, and strong gateway boundary controls between vehicle network segments.

## 2. Automotive perspective and context

### 2.1 The automotive Ethernet threat landscape

In vehicles, Ethernet links are attractive targets because they often carry high-value traffic between cameras, switches, zonal controllers, gateways, and central compute. NHTSA says wireless interfaces create new attack vectors, recommends treating all external networks as untrusted, and explicitly recommends network segmentation and strong gateway boundary controls, including strict whitelist-based filtering of message flows between different segments. It also warns that unnecessary IP services and open ports in production vehicles create exploitable attack vectors.

At the in-vehicle message level, NHTSA gives concrete safety-relevant examples: a spoofed traction-control command could apply brakes without legitimate intent, and a spoofed vehicle-speed message could cause distributed controllers to misunderstand whether the vehicle is stationary or moving.

Some of the Ethernet-specific threat scenarios are even more explicit: passive wiretapping, replay, message modification, arbitrary message injection, and man-in-the-middle style attacks are all plausible on exposed or insufficiently protected Ethernet links.

So the automotive threat landscape is not just “someone hacks the infotainment system.” It includes:

- external entry through wireless or service interfaces

- lateral movement through weakly segmented in-vehicle networks

- direct Layer 2 tapping or injection on exposed Ethernet links

- replay or manipulation of safety-relevant traffic

- supply-chain weakness, since NHTSA stresses that system security is measured by its weakest link and that suppliers must meet verified cybersecurity expectations.

### 2.2 Why MACsec is attractive in vehicles

MACsec is attractive in automotive because it directly secures the Ethernet links that increasingly replace or aggregate legacy vehicle networks. Ethernet is becoming essential inside the vehicle platform and that MACsec prevents Layer 2 threats such as passive wiretapping, intrusion, man-in-the-middle, and replay attacks by offering line-rate encryption and traffic protection over Layer 1 and Layer 2 links.

That makes MACsec a strong fit for:

- camera-to-switch links

- zonal-controller uplinks

- gateway-to-central-compute links

- other “mission-critical” in-vehicle Ethernet segments where confidentiality and integrity both matter.

It is especially useful where physical access to harnesses, connectors, or intermediate taps cannot be ruled out completely. Marvell explicitly notes that it is not practical to secure the whole network against physical access by determined attackers, which is exactly where link-level cryptographic protection becomes valuable.

### 2.3 Automotive choices for MACsec modes

Automotive deployments often do not want every optional MACsec mode. The public OPEN Alliance Automotive MACsec Specification v1.0 says an automotive MACsec implementation shall support:

- integrity without confidentiality

- confidentiality and integrity with confidentiality offset 0

and it adds that automotive MACsec shall work even when offset 30 and offset 50 are not supported. It also says automotive MACsec should support the SecTAG without encoding the SCI, specifically to save 8 bytes per Ethernet frame.

That gives a very important automotive interpretation:

- integrity-only mode matters because some traffic may need tamper detection more than secrecy

- full confidentiality with offset 0 is the preferred encrypted mode

- the older partial-cleartext confidentiality offsets are not considered necessary in the automotive profile

- omitting explicit SCI in the SecTAG can reduce overhead on high-volume in-vehicle links.

### 2.4 MKA in automotive: startup matters

Automotive systems care deeply about startup time. AUTOSAR’s MACsec implementation guidance says MKA discovers peers and negotiates keys, but it also says current automotive implementation guidance relies on pre-shared keys (PSK) because of much better startup times, meaning the Connectivity Association Key (CAK) and Connectivity Association Key Name (CKN) should be pre-installed on the communication participants.

That is a very automotive-specific design pressure. In a generic enterprise deployment, certificate-based or Extensible Authentication Protocol - Transport Layer Security (EAP-TLS)-style onboarding may be acceptable. In a vehicle, long key-agreement delay at power-up can be unacceptable for control paths or user experience. So automotive MACsec deployments often favor faster, more deterministic key-establishment strategies even if the broader 802.1X ecosystem supports richer choices.

### 2.5 gPTP and MACsec: inline protection versus bypass

Now to the part that usually gets oversimplified.

The IEEE 802.1AS-2020 model for a time-aware system explicitly includes MACsec in the port model. The standard’s figure for a PTP instance on full-duplex point-to-point links shows MACsec inline with the 802.3 MAC and PHY on both sides of the link. So gPTP does not inherently require MACsec bypass; the standards model allows timing to operate with MACsec in the path.

At the same time, timing accuracy creates practical constraints. A 2023 security-controls study notes that using MACsec to protect PTP creates obstacles in one-step mode because encryption/decryption after timestamping can introduce variable delay, and it concludes that for MACsec to protect PTP without degrading accuracy, latency variation must be constrained and PTP should use two-step mode, which does not require on-the-fly timestamp insertion into the protected Sync message.

So the standards-faithful answer is:

- gPTP can run with MACsec inline

- but one-step timing and MACsec are an awkward combination

- two-step timing is the safer fit when MACsec protects timing traffic.

### 2.6 Why bypass exists in practice

Even though inline protection is possible, real automotive implementations often allow selected traffic to bypass MACsec using the Uncontrolled Port behavior.

IEEE 802.1AE defines the Uncontrolled Port as the access point used to provide the insecure MAC service to a client of the SecY. The OPEN Alliance automotive MACsec profile says forwarding to the Uncontrolled Port shall be selectable using frame fields such as destination MAC address, EtherType, and matching IEEE 802.1Q VLAN ID, in both transmit and receive directions. Technica’s automotive MACsec presentation says current MACsec chips allow selected traffic to bypass MACsec, explicitly naming MKA, and recommends bypass for key installation, diagnostics, and update traffic, while warning that the unprotected communication then becomes critical to secure separately.

That gives the practical picture:

- MKA commonly bypasses MACsec because it must establish MACsec in the first place

- some implementations may also bypass diagnostics, update, or timing traffic for startup or latency reasons

- bypass is not free: it creates an exposed path that must be protected by architecture and policy rather than by MACsec encryption itself.

### 2.7 gPTP / MACsec bypass: the real engineering answer

So should gPTP bypass MACsec? The honest answer is: it depends on the system design, but bypass should not be treated as automatic or harmless.

If the hardware and timing architecture support two-step timing with bounded MACsec latency variation, protecting gPTP with MACsec is defensible and standards-aligned. If startup constraints, one-step assumptions, or implementation limitations make that impractical, some systems may choose to bypass gPTP on the Uncontrolled Port using EtherType- or VLAN-based rules. The OPEN Alliance automotive profile makes that kind of rule-based uncontrolled-port forwarding possible, but it is clear that any unprotected communication then becomes a critical security concern.

A good takeaway is:

bypass is a design trade-off, not a free optimization.

It can help startup or timing implementation, but it weakens link protection and therefore raises the importance of segmentation, boundary control, and attack detection elsewhere. NHTSA’s emphasis on segmentation, strong gateways, protected services, and event logging fits exactly this kind of trade-off management.

### 2.8 What a realistic automotive MACsec policy looks like

A realistic automotive MACsec deployment usually combines several principles:

- use MACsec on the most exposed or most critical Ethernet links

- choose integrity-only or confidentiality-plus-integrity per traffic need

- prefer confidentiality offset 0 rather than partial-cleartext offsets

- use PSK-style or otherwise fast-start MKA where startup time is critical

- keep MKA itself on the Uncontrolled Port

- treat any additional bypass traffic, such as diagnostics, update flows, or possibly gPTP, as a consciously accepted exception with extra compensating controls

- maintain counters and logs, since the OPEN Alliance automotive profile explicitly requires statistics support based on IEEE 802.1AE counter families.

## 3. Short recap

In pure data-communications terms, MACsec is a Layer 2 hop-by-hop security architecture built around Connectivity Associations (CA), Secure Channels (SC), Secure Associations (SA), and the MAC Security Entity (SecY). MKA, from IEEE 802.1X, discovers authenticated peers and distributes the Secure Association Keys (SAK) that the SAs use. Frames carry a SecTAG with control and sequencing information plus an Integrity Check Value (ICV), and the standard supports both integrity-only and integrity-plus-confidentiality modes, with GCM-AES-128 mandatory and GCM-AES-256 optional.

In the automotive context, the threat landscape includes eavesdropping, replay, message injection, spoofed control or state messages, exposed IP services, and lateral movement through weak segmentation. MACsec is attractive because it directly secures Ethernet links that carry safety-relevant or mission-critical traffic, but real automotive deployments also care about startup time, selected bypass paths, and timing interactions with gPTP. The standards model allows gPTP to operate with MACsec inline, but practical timing accuracy often pushes designers toward two-step timing if they want MACsec protection, and some systems still choose controlled bypass for MKA and other selected traffic.