In AVTP, the first byte of the AVTPDU common header is the subtype field. It tells the receiver what kind of AVTP payload/header format follows.

The commonly used AVTP subtype values are:

| Subtype value | Name | Meaning |
| --- | --- | --- |
| 0x00 | 61883_IIDC | IEC 61883 / IIDC format, historically used for IEEE 1394-style audio/video transport over AVTP |
| 0x01 | MMA Stream | MIDI Manufacturers Association stream format |
| 0x02 | AAF | AVTP Audio Format, used for PCM, AES3, and similar audio streams |
| 0x03 | CVF | Compressed Video Format, for compressed video such as MJPEG, H.264, JPEG 2000 variants |
| 0x04 | CRF | Clock Reference Format, used to carry media-clock timing references |
| 0x05 | TSCF | Time-Synchronous Control Format |
| 0x06 | SVF | SDI Video Format |
| 0x07 | RVF | Raw Video Format |
| 0x6E | AEF Continuous | AES Encrypted Format, continuous stream |
| 0x6F | VSF Stream | Vendor-Specific Format stream |
| 0x7F | EF Stream | Experimental Format stream |
| 0x82 | NTSCF | Non-Time-Synchronous Control Format |
| 0xEC | ESCF | ECC Signed Control Format |
| 0xED | EECF | ECC Encrypted Control Format |
| 0xEE | AEF Discrete | AES Encrypted Format, discrete mode |
| 0xFA | ADP | AVDECC Discovery Protocol |
| 0xFB | AECP | AVDECC Enumeration and Control Protocol |
| 0xFC | ACMP | AVDECC Connection Management Protocol |
| 0xFE | MAAP | MAC Address Acquisition Protocol |
| 0xFF | EF Control | Experimental Format control |

The most important ones in automotive/TSN training are usually:

0x02  AAF     Audio stream

0x03  CVF     Compressed video stream

0x04  CRF     Clock reference stream

0x05  TSCF    Time-synchronous control

0x82  NTSCF   Non-time-synchronous control

0xFA  ADP     AVDECC discovery

0xFB  AECP    AVDECC enumeration/control

0xFC  ACMP    AVDECC connection management

0xFE  MAAP    Multicast MAC address acquisition

AUTOSAR’s IEEE 1722 Transport Layer specification, for example, explicitly maps configured streams to subtype values such as 0x04 for CRF, 0x02 for AAF, 0x00 for 61883/IIDC, 0x07 for RVF, 0x82 for non-time-synchronous ACF, and 0x05 for time-synchronous ACF. An open AVTP implementation header also lists the broader subtype allocation, including ADP 0xFA, AECP 0xFB, ACMP 0xFC, MAAP 0xFE, and experimental/vendor-specific ranges.