A wildcard stream is usually not a special AVTP/IEEE 1722 packet type.

It means a stream-identification rule in the switch or software stack where one or more matching fields are treated as “don’t care”.

So the wildcard is in the classifier configuration, not inside the AVTPDU.

For example, a normal AVTP/ACF stream filter may say:

Match only:

EtherType  = 0x22F0

Subtype    = NTSCF

Stream ID  = 0xAABBCCDDEEFF0001

VLAN ID    = 20

PCP        = 4

A wildcard-style rule may say:

Match:

EtherType  = 0x22F0

Subtype    = NTSCF

Stream ID  = ANY

VLAN ID    = 20

PCP        = 4

That means: all NTSCF AVTP streams on VLAN 20 with PCP 4 match this rule, regardless of the exact IEEE 1722 stream ID.

AUTOSAR’s Ethernet Switch Driver uses this kind of idea in stream filtering. It says multiple configured filter primitives are treated as AND-linked, and if a particular filter primitive is not configured, that primitive is considered matching. AUTOSAR calls such not-configured primitives “wildcard filter rule primitives.” It also notes that an empty stream filter rule can match every received Ethernet frame whose internal route matches the configured stream-identification entry. (autosar.org)

A useful mental model:

Wildcard stream = broad stream match

Specific stream = exact stream match

Example in an automotive ACF case:

Specific ACF stream rule

Match Stream ID = CameraZoneA_CAN_Tunnel_01

Apply gate G1, meter M1, queue TC4

Wildcard ACF stream rule

Match any IEEE 1722 TSCF/NTSCF stream from ingress port 3

Apply default ACF policing and queue TC4

This is useful when the switch designer wants to apply one common action to a class of streams, for example:

all ACF streams

all AVTP streams from one ingress port

all streams with one VLAN ID

all streams with one PCP

all frames that already have any assigned stream handle

AUTOSAR also defines a stream-handle wildcard flag, where any assigned stream handle ID carried by an Ethernet frame can match the stream filter. (autosar.org)

So in switch-pipeline terms:

Ingress frame arrives

↓

Parser extracts MAC / VLAN / EtherType / AVTP fields

↓

Stream identification table is searched

↓

Some fields may be exact-match

Some fields may be wildcard / don't-care

↓

A stream handle is assigned

↓

PSFP / Qci / gate / meter / queue action is applied

Important distinction:

| Term | Meaning |
| --- | --- |
| IEEE 1722 stream ID | A real 64-bit identifier carried in AVTP stream data frames |
| Wildcard stream | A classifier rule that does not require an exact match on one or more fields |
| Wildcard stream ID | Usually means “match any stream ID,” not a special stream ID value |
| Wildcard filter primitive | A not-configured match field treated as automatically matching |

So the clean answer is:

A wildcard stream is a switch/software classification concept where the rule matches a range or class of streams instead of one exact stream. It is not a distinct IEEE 1722 subtype, and it is not a special AVTP PDU format.

Wildcard priority means:

Do not require a specific priority value when matching the frame or stream.

It is a classifier/filter rule concept, not a new Ethernet priority value.

In Ethernet/TSN context, “priority” usually means the PCP, or Priority Code Point, in the IEEE 802.1Q VLAN tag.

802.1Q VLAN tag

+-------------------------------+

| PCP | DEI | VLAN ID           |

| 3 b | 1 b | 12 b              |

+-------------------------------+

The PCP field is 3 bits, so valid priority values are:

0, 1, 2, 3, 4, 5, 6, 7

A normal exact-match rule may say:

Match:

EtherType = 0x22F0   AVTP

Subtype   = NTSCF

VLAN ID   = 20

PCP       = 4

This matches only AVTP/NTSCF frames on VLAN 20 with PCP 4.

A wildcard priority rule says:

Match:

EtherType = 0x22F0   AVTP

Subtype   = NTSCF

VLAN ID   = 20

PCP       = ANY

This means:

PCP 0 matches

PCP 1 matches

PCP 2 matches

...

PCP 7 matches

So the priority field is treated as don’t care.

## Important point

Wildcard priority does not mean “highest priority.”

It also does not mean:

PCP = 8

PCP = -1

PCP = special priority

It simply means:

Ignore priority while matching this rule.

## Example in an ACF / AVTP case

Suppose a switch has this rule:

Ingress port = 3

EtherType    = 0x22F0

AVTP subtype = NTSCF

Stream ID    = ANY

Priority     = wildcard

Action       = assign stream handle ACF_DEFAULT

Then all these frames can match the same rule:

NTSCF frame, PCP 2

NTSCF frame, PCP 3

NTSCF frame, PCP 4

NTSCF frame, PCP 5

because the rule is not checking PCP.

## Why use wildcard priority?

It is useful when the designer wants to classify traffic based on something more important than PCP, for example:

Ingress port

Destination MAC

EtherType

VLAN ID

AVTP subtype

Stream ID

CAN tunnel identity

For example:

All ACF traffic from zone gateway port 5

→ send to traffic class 4

→ apply common policer

→ apply common stream gate

Here, the system designer may not care whether the incoming PCP was 3, 4, or 5. The switch rule can wildcard the priority and then assign an internal priority or traffic class later.

## Untagged frame case

If a frame is untagged, there is no on-wire PCP field.

Destination MAC

Source MAC

EtherType = 0x22F0

AVTPDU

FCS

In that case, the switch may first assign a default internal priority based on ingress port configuration.

A wildcard priority rule can still be useful because it does not depend on a specific received PCP value.

## Difference between exact priority and wildcard priority

| Rule type | Meaning |
| --- | --- |
| Priority = 4 | Match only frames whose priority is 4 |
| Priority = 5 | Match only frames whose priority is 5 |
| Priority = wildcard | Match regardless of priority |
| Priority = not configured | Often treated as wildcard, depending on switch/config model |

## Switch pipeline view

Frame arrives

↓

Ingress parser extracts VLAN PCP if present

↓

Default priority may be assigned if frame is untagged

↓

Stream identification / classification table is searched

↓

Priority field may be exact-match or wildcard

↓

Matching rule assigns stream handle / traffic class / gate / meter

So the clean rule is:

Wildcard priority means “match this stream/filter rule regardless of the frame’s PCP or internal priority.” It is a matching shortcut, not a transmitted value and not a special priority level.